3GPP TSG-SA WG2 Meeting #149E (e-meeting)	S2-2200215r03

# Feburary 14-25, 2022, Elbonia

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0064 | rev | - | Current version: | 17.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolve ENs for Security Parameters Provisioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe |  |  |  |  |  | Date: |  |  | 2022-01-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In clause 5.1 of TS 23.304, there are several ENs related to the security parameters provisioning, such as: Editor's note: Whether the security parameters can be provided by the PCF and details of security parameters will be determined by SA3 WG. Regarding the security parameters provisioning, TS 33.503 defines that 5G DDNMF provides the security parameters for the ProSe Direct Discovery in clause 6.1.3. In TS 33.503, clause 6.2.2, there is the following requirement for PC5 Direct Communication: The PCF shall be able to provision the PC5 security policies to the UE per ProSe application, during service authorization and information provisioning procedure as defined in TS 23.304 [2]. For Groupcast mode Direct Communication, no security parameter needs to be provisioned. Thus the ENs in clause 5.1 of TS 23.304 can be resolved based on the agreement in TS 33.503 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Resolve the ENs and corrections in clause 5.1.2.1 and 5.1.3.1 according to SA3 agreement. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The ENs in clause 5.1.2.1 and 5.1.3.1 are not resolved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.2.1, 5.1.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * First Change * * * *

#### 5.1.2.1	Policy/Parameter provisioning for 5G ProSe Direct Discovery

The following sets of information for 5G ProSe Direct Discovery over PC5 reference point is provisioned to the UE:

1)	Authorization policy for 5G ProSe Direct Discovery:

-	When the UE is "served by NG-RAN":

-	For open 5G ProSe Direct Discovery:

a)	open 5G ProSe Direct Discovery Model A monitoring authorization policy:

-	PLMNs in which the UE is authorised to perform 5G ProSe Direct Discovery monitoring.

b)	open 5G ProSe Direct Discovery Model A announcing authorization policy:

-	PLMNs in which the UE is authorized to perform announcing;

-	Authorised discovery range for announcing per PLMN.

-	For restricted 5G ProSe Direct Discovery:

a)	restricted 5G ProSe Direct Discovery Model A monitoring authorization policy:

-	PLMNs in which the UE is authorised to perform restricted 5G ProSe Direct Discovery Model A monitoring.

b)	restricted 5G ProSe Direct Discovery Model A announcing authorization policy:

-	PLMNs in which the UE is authorized to perform restricted 5G ProSe Direct Discovery Model A announcing;

-	Authorised discovery range for announcing per PLMN.

c)	restricted 5G ProSe Direct Discovery Model B Discoverer operation authorization policy:

-	PLMNs in which the UE is authorized to perform Model B Discoverer operation;

-	Authorised discovery range for announcing per PLMN.

d)	restricted 5G ProSe Direct Discovery Model B Discoveree operation authorization policy:

-	PLMNs in which the UE is authorized to perform Model B Discoveree operation;

-	Authorised discovery range for announcing per PLMN.

NOTE 1:	In this specification, [When the UE is "served by NG-RAN"] and [When the UE is "not served by NG-RAN"] are relevant to 5G ProSe Direct discovery/communications over NR PC5 reference point.

-	When the UE is "not served by NG-RAN":

-	Indicates whether the UE is authorized to perform 5G ProSe Direct Discovery for Model A and Model B when "not served by NG-RAN".

NOTE 2:	If both Model A and Model B are authorized for 5G ProSe Direct Discovery, it is up to UE and application implementation to select a discovery model or perform both models simultaneously.

2)	Parameters used for 5G ProSe Direct Discovery:

-	The mapping of ProSe services (i.e. ProSe identifiers) to Destination Layer-2 ID(s) for sending/receiving initial signalling of discovery messages.

NOTE 3:	The same Destination Layer-2 ID for 5G ProSe Direct Discovery can be mapped to more than one ProSe services.

NOTE 4:	The values provisioned for the Destination Layer-2 ID(s) for 5G ProSe Direct Discovery, for Destination Layer-2 ID(s) for 5G ProSe Direct Communication, defined in clause 5.1.3.1 and for Destination Layer-2 ID(s) for 5G ProSe UE-to-Network Relay Discovery defined in clause 5.1.4.1, are different from each other.

-	Application identifiers to be used for 5G ProSe Direct Discovery over PC5 interface.

NOTE 5: 	The Security parameters for 5G ProSe Direct Discovery can be provisioned by 5G DDNMF as defined in TS 33.503 [29].

3)	Radio parameters when the UE is "not served by NG-RAN":

-	Includes the radio parameters per NR PC5 with Geographical Area(s) and an indication of whether they are "operator managed" or "non-operator managed". The UE uses the radio parameters to perform 5G ProSe Direct Discovery over PC5 reference point when "not served by NG-RAN" only if the UE can reliably locate itself in the corresponding Geographical Area. Otherwise, the UE is not authorized to transmit.

NOTE 6:	Whether a frequency band is "operator managed" or "non-operator managed" in a given Geographical Area is defined by local regulations.

4)	Restricted ProSe Discovery UE ID for Restricted Direct Discovery:

-	ProSe Discovery UE ID.

5)	Group Member Discovery parameters:

-	For each discovery group that the UE belongs to include the following parameters that enable the UE to perform Group Member Discovery when provided by PCF or provisioned in the ME or configured in the UICC:

-	Application Layer Group ID: Identifies an application layer group or a discovery group that the UE belongs to;

-	Layer-2 Group ID: layer-2 ID for Application Layer Group ID;

-	User Info ID: For Model A, this corresponds to the Announcer Info parameter when the UE is acting as an announcing UE. For Model B, this corresponds to the Discoverer Info in Solicitation messages and the Discoveree Info in Response messages, when the UE is acting as a discoverer or discoveree UE respectively.

NOTE 7:	User Info ID is expected to be assigned uniquely to a user within the discovery group.

6)	Validity timer indicating the expiration time of the Policy/Parameter for 5G ProSe Direct Discovery.

The above parameter sets bullet 2), bullet 3), bullet 5) and bullet 6) may be provided or updated to the UE by the ProSe Application Server, except for the Security parameters in bullet 2).

* * * * Second Change * * * *

#### 5.1.3.1	Policy/Parameter provisioning for 5G ProSe Direct Communication

The following sets of information for 5G ProSe Direct Communications over PC5 reference point is provisioned to the UE:

1)	Authorization policy:

-	When the UE is "served by NG-RAN":

-	PLMNs in which the UE is authorized to perform 5G ProSe Direct Communications over PC5 reference point when "served by NG-RAN".

-	When the UE is "not served by NG-RAN":

-	Indicates whether the UE is authorized to perform 5G ProSe Direct Communications over PC5 reference point when "not served by NG-RAN".

NOTE 1:	In this specification, [When the UE is "served by NG-RAN"] and [When the UE is "not served by NG-RAN"] are relevant to 5G ProSe Direct Communications over NR PC5 reference point.

2)	Groupcast mode 5G ProSe Direct Communication policy/parameters:

-	For each application layer group supported include the parameters that enable the UE to perform Groupcast mode 5G ProSe Direct Communication when provided by PCF or provisioned in the ME or configured in the UICC:

-	Application Layer Group ID: Identifies an application layer group that the UE belongs to.

-	ProSe Layer-2 Group ID: Destination Layer-2 ID applicable only when the Application Layer Group ID is provided by the application layer

-	ProSe Group IP multicast address

-	Indication whether the UE should use IPv4 or IPv6 for that group

-	For a specific Group configured to operate using IPv4, optionally an IPv4 address to be used by the UE as a source address. If none is provisioned, then the UE shall use Dynamic Configuration of IPv4 Link-Local Addresses RFC 3927 [18] to obtain a link local address for the Group.

3)	Radio parameters when the UE is "not served by NG-RAN":

-	Includes the radio parameters NR PC5 with Geographical Area(s) and an indication of whether they are "operator managed" or "non-operator managed". The UE uses the radio parameters to perform ProSe Direct Communications over PC5 reference point when "not served by NG-RAN" only if the UE can reliably locate itself in the corresponding Geographical Area. Otherwise, the UE is not authorized to transmit.

NOTE 2:	Whether a frequency band is "operator managed" or "non-operator managed" in a given Geographical Area is defined by local regulations.

4)	Policy/parameters related to privacy:

-	The list of ProSe services (i.e. ProSe identifiers) with Geographical Area(s) that require privacy support.

-	A privacy timer value indicating the duration after which the UE shall change each source Layer-2 ID self-assigned by the UE when privacy is required.

5)	Policy/parameters when NR PC5 is selected:

-	The mapping of ProSe services (i.e. ProSe identifiers) to radio frequencies with Geographical Area(s).

-	The mapping of ProSe services (i.e. ProSe identifiers) to Destination Layer-2 ID(s) for broadcast.

-	The mapping of ProSe services (i.e. ProSe identifiers) to Destination Layer-2 ID(s) for groupcast.

-	The mapping of ProSe services (i.e. ProSe identifiers) to default Destination Layer-2 ID(s) for initial signalling to establish unicast connection.

NOTE 3:	The same default Destination Layer-2 ID for unicast initial signalling can be mapped to more than one ProSe service. In the case where different ProSe services are mapped to distinct default Destination Layer-2 IDs, when the UE intends to establish a single unicast link that can be used for more than one ProSe service, the UE can select any of the default Destination Layer-2 IDs to use for the initial signalling.

NOTE 4: 	Security policies for Unicast mode 5G ProSe Direct Communication can be provisioned by PCF as defined in TS 33.503 [29].

-	The mapping of ProSe services (i.e. ProSe identifiers) to PC5 QoS parameters defined in clause 5.4.2 (i.e. PQI and conditionally other parameters such as MFBR/GFBR, etc.).

-	AS layer configurations (see TS 38.331 [16]), i.e. the mapping of PC5 QoS profile(s) to radio bearer(s), when the UE is "not served by NG-RAN".

-	The PC5 QoS profile contains PC5 QoS parameters described in clause 5.4.2 of TS 23.287 [2], and value for the QoS characteristics regarding Priority Level, Averaging Window, Maximum Data Burst Volume if default value is not used as defined in Table 5.4.4-1 of TS 23.287 [2] and in Table 5.6.1-1.

6)	Path selection policy:

-	The mapping of ProSe services (i.e. ProSe identifiers) to path preference (i.e. PC5 preferred, Uu preferred, or no preference) as defined in clause 5.11.

NOTE 5:	The path selection policy can be a one mapping for all ProSe services, i.e. same path preference for all ProSe services.

7)	Validity time indicating the expiration time of the Policy/Parameter for 5G ProSe Direct Communication.

The above parameter sets from bullet 2) to 7) may be provided or updated to the UE by the ProSe Application Server.

* * * * End of Changes * * * *