TSG SA WG2 Meeting #149E	S2-2200234r02

14 - 25 February, 2022, Electronic meeting

Source:	Lenovo, Motorola Mobility, Huawei, HiSilicon, Vivo? Ericsson? CATT?

Title:	Key issue on traffic characteristics of XR and media service

Document for:	Approval

Agenda Item:	9.19

Work Item / Release:	FS_XRM / Rel-18

Abstract of the contribution: Proposes a Key Issue and annex on traffic characteristics for XR and media service

# 1	Background

According to the SP-211610[1], the study on 5G System support for XR and media services targeted at R18 supports 5GS architectural and functional extensions so that XR and media service can be better supported. One of the objectives that this study attempts to address is:

WT#3.1:  Study the traffic characteristics of media service enabling improved network resources usage and QoE.

In this paper, traffic characteristics for XR and media service and the open issues are discussed.

# 2	Discussion

The following contents in this paper focus on real-time XR/media services. In general, XR and media service has the following traffic characteristics.

## 2.1 Frame and GOP

A video consists of a sequence of consecutive video frames. Each video frame is a picture encoded/compressed using different codec mechanisms (e.g. H.264/H.265/H.266, AV1 or AVS) for efficient storage and transmission. During encoding, the video spatial and temporal redundancy can be removed using, e.g. inter-frame/intra-frame prediction, motion estimation/compensation. Typically, three major frame/picture types can be used during encoding, i.e. I-frame, P-frame and B-frame:

- I-frame, as an intra-coded picture, is a complete picture and can be encoded and decoded independently, like a JPG image file.

- P-frame, as a predicted picture, is not a complete frame and only contains the image changes compared to the previous frame. If the reference frame is lost, the P-frame cannot be decoded and displayed.

- B-frame, as a bidirectional predicted picture, contains the changes between the previous and following reference frames. With more reference frames, the compression ratio can be higher. However, the B-frame can only be decoded when the previous and following reference frames are available.

A Group of Pictures (GOP) includes a collection of successive video frames. The first frame in a GOP is an I-frame. And the following frames can be P-frames or B-frame.

For example, as shown in figure 1, a GOP contains five frames and the first frame is I frame. The second frame is P frame and refers to the I frame. Similarly, the third frame is P frame, referring to the second frame. The forth frame is a B frame which refers to both the previous and following frames. When delivering the GOP from the application server to a client, the I frame can be directly decoded and displayed. The P frame can be decoded only when the previous frame arrived and is successfully decoded. The B frame can be decoded only when both the previous and following frames arrive and are successfully decoded.

Figure 1 Frame types and GOP

## 2.2 Other Traffic Characteristics

Most of the objectives of this study item are closely related with the XR traffic characterisitcs as follows.

1) Periodic and jitter

Data burst of video frame are always periodic with variable packet size. For example, audio frame ususally have a periodicity of 20ms, while video stream can have different frame rate, e.g., 60/90/120 fps. Besides, pose/control packets are always periodic (e.g., 4ms) with fixed packet size. Due to the unpredictable jitter, the packet arrival time might not be exacly periodic, which leads to transmission delay or packet loss. 5GS should take both periodicity and jitter into consideration for XR and media service.

2) Frame importance

For XR and media service, different types of frame coexist and have different importance. For example, in Group of Picture (GOP), the first I-frame is the most important frame, other P-frames are encoded based on the I-frame. That is, the decoding of P-frames depends on the decoding of I-frame in the same GOP. Therefore, frame importance should be provided to enable 5GS to perform efficient packet dropping, e.g., the non-transmitted part with low importance of GOP will be discarded at the transmitter if the packets with high importance of the GOP have been lost or exceed the delay budget.

3) Media unit granularity

XR applications impose requirements in terms of Media Units(e.g., video/audio frame/tile, Application Data Units etc.), rather than in terms of single packets/PDUs. Packets of one media unit need to be jointly processed for XR traffics. 5G system should be aware of the media unit in order to improve transmission performance. Basically, the media unit SN, the packet SN within the media unit, the total packet number of the media unit or the last packet indication should be provided to 5G system for each packet.

4) Traffic correlation

Some advanced XR or media services may include multiple traffic streams such as video, audio, haptic data or sensor data etc., which are called multi-modality communication. Different traffic streams with different QoS parameters are always mapped to different QoS flows. Packets within the same traffic stream with different packet charateristics (e.g., packet importance, frame type etc.) may also be classified and mapped into different QoS flows. These QoS flows belong to the same XR and media service have strong correlations, which should be coordinated for QoS requirement, packet processing and time synchronization etc.

In short, 5GS should be aware of all the paremeters related with traffic characterisitcis for XR and media service, which are defined as application-aware parameters. Generally, downlink traffic dominates XR and media service. In this paper, we mainly discuss how application-aware parameters are provided to 5GS, either over control plane by configuration, or over user plane together with each packets? E.g., parameters used to identify media unit should be provided for each packet over user plane, while the periodicity and traffic correlation can be provided over control plane by configuration.

# 3 Reference

[1] SP-211610, New SID on Study on architecture enhancement for XR and media services, China Mobile, Huawei, HiSilicon, Tencent, Xiaomi

[2] TR 26.926, 3rd Generation Partnership Project; Technical Specification Group SA; Traffic Models and Quality Evaluation Methods for Media and XR Services in 5G Systems; (Release 17)

# 4 Proposal

* * * * Start of Change * * * *

*** NEXT CHANGE ***

# Annex X(informative): Traffic characteristics of XR and media services

## X.1 Frame and GOP

A video consists of a sequence of consecutive video frames. Each video frame is a picture encoded/compressed using different codec mechanisms (e.g. H.264/H.265/H.266, AV1 or AVS) for efficient storage and transmission. During encoding, the video spatial and temporal redundancy can be removed using, e.g. inter-frame/intra-frame prediction, motion estimation/compensation. Typically, three major frame/picture types can be used during encoding, i.e. I-frame, P-frame and B-frame:

- I-frame, as an intra-coded picture, is a complete picture and can be encoded and decoded independently, like a JPG image file.

- P-frame, as a predicted picture, is not a complete frame and only contains the image changes compared to the previous frame. If the reference frame is lost, the P-frame cannot be decoded and displayed.

- B-frame, as a bidirectional predicted picture, contains the changes between the previous and following reference frames. With more reference frames, the compression ratio can be higher. However, the B-frame can only be decoded when the previous and following reference frames are available.

A Group of Pictures (GOP) includes a collection of successive video frames. The first frame in a GOP is an I-frame. And the following frames can be P-frames or B-frame.

For example, as shown in figure 1, a GOP contains five frames and the first frame is I frame. The second frame is P frame and refers to the I frame. Similarly, the third frame is P frame, referring to the second frame. The forth frame is a B frame which refers to both the previous and following frames. When delivering the GOP from the application server to a client, the I frame can be directly decoded and displayed. The P frame can be decoded only when the previous frame arrived and is successfully decoded. The B frame can be decoded only when both the previous and following frames arrive and are successfully decoded.

Figure 1 Frame types and GOP

## X.2 Other traffic characteristics

1) Periodic and jitter

Data burst of video frame are always periodic with variable packet size. For example, audio frame ususally have a periodicity of 20ms, while video stream can have different frame rate, e.g., 60/90/120 fps. Besides, pose/control packets are always periodic (e.g., 4ms) with fixed packet size. Due to the unpredictable jitter, the packet arrival time might not be exacly periodic, which leads to transmission delay or packet loss. 5GS should take both periodicity and jitter into consideration for XR and media service.

2) Frame importance

For XR and media service, different types of frame coexist and have different importance. For example, in Group of Picture (GOP), the first I-frame is the most important frame, other P-frames are encoded based on the I-frame. That is, the decoding of P-frames depends on the decoding of I-frame in the same GOP. Therefore, frame importance should be provided to enable 5GS to perform efficient packet dropping, e.g., the non-transmitted part with low importance of GOP will be discarded at the transmitter if the packets with high importance of the GOP have been lost or exceed the delay budget.

3) Media unit granularity

XR applications impose requirements in terms of Media Units(e.g., video/audio frame/tile, Application Data Units etc.), rather than in terms of single packets/PDUs. Packets of one media unit need to be jointly processed for XR traffics. 5G system should be aware of the media unit in order to improve transmission performance. Basically, the media unit SN, the packet SN within the media unit, the total packet number of the media unit or the last packet indication should be provided to 5G system for each packet.

4) Traffic correlation

Some advanced XR or media services may include multiple traffic streams such as video, audio, haptic data or sensor data etc., which are called multi-modality communication. Different traffic streams with different QoS parameters are always mapped to different QoS flows. Packets within the same traffic stream with different packet charateristics (e.g., packet importance, frame type etc.) may also be classified and mapped into different QoS flows. These QoS flows belong to the same XR and media service have strong correlations, which should be coordinated for QoS requirement, packet processing and time synchronization etc.

* * * * End of Change * * * *