3GPP TSG-WG SA2 Meeting #149E e-meeting  	S2-2200255r04

Elbonia, February 14 – 25, 2022

# Title:	[DRAFT] Reply LS on Clarification on the change of Target NSSAI

# Response to:	LS (S2-2200051/C3-216445) on Clarification on the change of Target NSSAI from CT3

# Release:	Rel-17

# Work Item:	eNS_Ph2

Source:	SA2

To:	CT3

Cc:

Contact Person:

Name:	Haiyang,Sun

Tel. Number:

E-mail Address:	sunhaiyang3 AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

SA2 would like to thank CT3 for their LS on Clarification on the change of Target NSSAI.

Specific to the CT3 questions in C3-216445/S2-2200051, SA2 would like to provide the following feedback:

Question: Can SA2 confirm that the target NSSAI may be generated more than once during the lifetime of the UE registration (i.e. the change of target NSSAI may happen)? For example, if the subscribed NSSAI or requested NSSAI changes, will the Target NSSAI be changed accordingly? Can SA2 indicate if there are other situations that require the trigger to be invoked.

SA2 Answer: SA2 has agreed that the Target NSSAI is a one-time piece of information generated by NSSF or AMF in response to a registration request (RR) from the UE.  The Target NSSAI is a quite volatile information and has one-time use. So, the AMF only interacts with the PCF to obtain a RFSP for a new Target NSSAI but does not store this target NSSAI nor this RFSP, nor update it autonomously without a RR from the UE. When the AMF does not select a new PCF the AMF uses the existing AM Policy association to obtain the RFSP for a Target NSSAI.

2. Actions:

To CT3 group.

ACTION: 	SA2 asks CT3 group to take the above feedback into account.

3. Date of Next TSG SA WG2 Meetings:

See https://portal.3gpp.org/Home.aspx?tbid=385&SubTB=385#/