3GPP TSG-WG SA2 Meeting #149E e-meeting 	S2-2200272r02

Elbonia, February 14th – 25th, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon, Vivo?, Samsung?, Nokia?, Qualcomm?

Title:	New KI on WT1 for URSP roaming

Document for:	Approval

Agenda Item:	9.22

Work Item / Release:	FS_eUEPO/ Rel-18

Abstract: To add new KI on WT1 for URSP roaming.

# 1. Introduction/Discussion

As described in WT#1 of FS_eUEPO, it is proposed to study URSP rule provisioning and updating procedures for home-routed and LBO roaming scenario, while keeping the existing framework of policy control based on HPLMN.

The Use Cases could be:

- The UE policy related information (including LADN DNN or AF located in VPLMN) is very static. New scenarios, e.g. flexible LADN deployment or MEC deployment in VPLMN requires more dynamic UE policy provisioning and updating mechanism.

- In roaming scenario, there could be multiple S-NSSAIs in VPLMN mapping to one S-NSSAI in HPLMN. It needs more dynamic UE policy provisioning and updating mechanism for UE to select more efficient S-NSSAI in VPLMN.

Considering this, it is proposed to study the following issues in particular,

- Whether and which information from VPLMN should be involved to generate URSP.

- Whether and how URSP related to VPLMN should be generated.

- How V-PCF could adjust the URSP rules from H-PCF?

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-25.

# * * * * First change * * * * All new

# 5	Key Issues

## 5.x	Key Issue #x: URSP in VPLMN

### 5.x.1	Description

This Key Issue will study URSP rule provisioning and updating procedures in roaming scenario, while keeping the existing framework of policy control based on HPLMN. In particular,

- Whether and whatVPLMN information should be taken into account to generate URSP rules.

- Whether and how to generate a URSP rule with VPLMN information and its relationship with the HPLMN provided rule.

- Which functional entities and triggers are responsible for providing the VPLMN information.

The related use cases are:

- The UE policy related information is very static. New scenarios, e.g. flexible LADN deployment or MEC deployment in VPLMN requires more dynamic UE policy provisioning and updating mechanism.

- In roaming scenario, there could be multiple S-NSSAIs in VPLMN mapping to one S-NSSAI in HPLMN. The UE needs more dynamic policy provisioning and updating mechanism to select an efficient S-NSSAI in the VPLMN.

# * * * * End of changes * * * *