3GPP TSG-WG SA2 Meeting #149E e-meeting 	S2-2200272r09

Elbonia, February 14th – 25th, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon, Vivo?, Samsung?, Nokia, Nokia Shanghai Bell, Qualcomm?

Title:	New KI on WT1 for URSP roaming

Document for:	Approval

Agenda Item:	9.22

Work Item / Release:	FS_eUEPO/ Rel-18

Abstract: To add new KI on WT1 for URSP roaming.

1. Introduction/Discussion

As described in WT#1 of FS_eUEPO, it is proposed to study URSP rule provisioning and updating procedures for home-routed and LBO roaming scenario, while keeping the existing framework of policy control based on HPLMN.

The Use Cases could be:

- The UE policy related information is very static. New scenarios, e.g. flexible LADN deployment or MEC deployment in VPLMN requires more dynamic UE policy provisioning and updating mechanism.

- In roaming scenario, there could be multiple S-NSSAIs in VPLMN mapping to one S-NSSAI in HPLMN. It needs more dynamic UE policy provisioning and updating mechanism for UE to select more efficient S-NSSAI in VPLMN.

Considering this, it is proposed to study the following issues in particular,

- Whether and which information from VPLMN should be involved to generate URSP.

- Whether and how URSP related to VPLMN should be generated.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-25.

# * * * * First change * * * * All new

5	Key Issues

5.x	Key Issue #x: URSP in VPLMN

5.x.1	Description

This Key Issue will study URSP rule provisioning and updating procedures in roaming scenario, while keeping backward compatibility with the existing framework of provisioning of URSP Rules from the HPLMN. In particular,

- Whether and which information from the VPLMN is needed to generate URSP Rules in roaming. If so, how to use it.

- How to provide URSP Rules in roaming to the UE. In particular, how the HPLMN and VPLMN are involved in such procedure.

- .

- Whether and how to avoid conflicts to generate/use URSP in roaming.

5.x.2	Scenarios

- The UE policy related information is either stored in the UDR or received from the AF or based on local policies or a combination. This is considered static given that the application deployments change more often and there could be frequent changes on e.g. DNN used to access the AF(AS). AF guidance is provided to the HPLMN, how to provide AF guidance in roaming scenarios needs to be studied.

- The URSP Rules contains S-NSSAI that always refer to the HPLMN S-NSSAIs, given that the H-PCF generates the URSP Rules. In roaming scenario, there could be multiple S-NSSAIs in VPLMN mapping to one S-NSSAI in HPLMN. It is proposed to study whether there is a need to take into the account the S-NSSAIs in the VPLMN mapping in the generation of the URSP Rules and how.

# * * * * End of changes * * * *