3GPP TSG-WG SA2 Meeting #149E e-meeting  	S2-2200273

Elbonia, February 14 – 25, 2022	(revision of S2-220xxxx)

# Title:	[DRAFT] LS on RAN feedback for low latency

# Release:	Release 18

# Work Item:	FS_5TRS_URLLC

Source:	[Huawei to be] SA2

To:	RAN2

Cc:	RAN1,RAN3

Contact Person:

Name:	Haiyang, Sun

Tel. Number:

E-mail Address:	sunhaiyang3 AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

SA2 has agreed an objective in FS_5TRS_URLLC to study if there is a need for applications to adapt downstream scheduling in order for 5GS to meet really low latency (e.g. to meet a PDB of 2 msecs) requirement and if there is a need to have feedback from RAN (e.g. for application to consider DL packet transmission time slots to avoid buffering in the RAN) for this purpose.

SA2 has also agreed that although the objective focus on downstream scheduling, any adaptation on upstream scheduling should not be precluded if similar enhancement as for downstream scheduling applies

SA2 has discussed following problems for meeting very low latency PDBs:

- One potential problem considering low latency applications is that the arrive time of the packets may not fit well with the TDD cycle used in the network. RAN just receives the traffic flow periodicity and burst arrival times but cannot influence them. For example, if a downlink packet arrives at an uplink slot, then it has to wait for the first downlink slot to be transferred and vice versa (please see Figure 1). This creates additional delay (e.g. more buffering time) to the traffic flows. This can be an issue for QoS Flows requiring PDB 5 ms or lower. On the other hand, if there is perfect match between TDD pattern and TSC burst arrival time, this delay is negligible.

Figure 1

- SA2 could not conclude whether a similar issue as Bullet 1) also exists for a radio interface using Frequency Division Duplexing (FDD). For instance, some companies were of the opinion that a similar problem might exist with FDD radio interface due to the use of configured grants or Semi Persistent Scheduling.

- Another potential case is that, the arrival times of different flows in the same RAN node are not coordinated currently and they can collide with each other. This will also create additional delay for those flows. In some cases, RAN may receive the traffic flow periodicity and burst arrival times but cannot influence them. In these cases RAN may need to reject admission of a QoS flow even though the flow could have been admitted with a slightly modified BAT. In other cases, RAN may only receive traffic flow periodicity but not burst arrival time information (e.g., because 5GS and AF are not time synchronized). As a result, RAN may admit a very low latency QoS flow based on QoS parameters and periodicity alone but in the end may need to reject the flow once it starts because the particular BAT of that flow cannot be supported. This may exist in both FDD and TDD scenario. On the other hand, if there is no conflict among the different QoS Flows, this delay is negligible.

Questions:

SA2 would like to ask RAN2 WGs

- Whether the above problems are valid for SA2 to solve?

- Are there other problems could be solved by this objective?

2. Actions:

To RAN2 group.

ACTION: 	SA2 asks RAN2 group to

Take the above information into account and provide feedback.

3. Date of Next TSG SA WG2 Meetings:

See https://portal.3gpp.org/Home.aspx?tbid=385&SubTB=385#/