SA WG2 Meeting #149e	S2-2200292

# February 14th – 25th, 2022; Elbonia               		(revision of S2-220)

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Key Issue for DetNet controller to 5GS interface

Document for: 	Approval

Agenda Item: 	9.6

Work Item / Release:	FS_DetNet / Rel-18

Abstract of the contribution: This contribution proposes a KI for WT#1: Study whether and how to enable 3GPP support for DetNet.

# 1	Discussion

This contribution proposes a KI for WT#1: Study whether and how to enable 3GPP support for DetNet such that a mapping is provided between the central DetNet controller entity (as defined in IETF) and the 5G system.

WT#1 states: Mapping involves translation of DetNet traffic profile and flow specification to 5GS QoS parameters and TSCAI. Study which information needs to be exposed from the 5G system to the DetNet controller.

Considering the scope and assumptions described in the SID [1], Rel-17 TSC framework and IETF standards maturity, it would be beneficial to consider the following working assumptions:

- 5GS acts as a DetNet Node supporting DetNet IP Data Plane and centralized Controller Plane.

This means that the 5GS DetNet Data Plane supports DetNet forwarding sub-layer functions with DetNet service awareness but does not support any DetNet Service sub-layer functions as specified in [2].

- DetNet IP data plane is supported based on 5GS R17 TSC QoS procedures available for IP PDU sessions with potential enhancements in scope of other R18 SIs, in particular 5TRS_URLLC [3].

This means that in this release there would be no definition how DS-TT and NW-TT would support specific queuing techniques such as Credit Based Shaper for IP PDU sessions.

- DetNet Controller – 5GS interface is based on 5GS R17 exposure APIs with potential extensions necessary for DetNet controller plane operations.

This means that in this release 5GS (TSCTSF or NEF) would not expose specific protocol interfaces such as NETCONF, RESTCONF or PCEP to the DetNet Controller.

[1] SP-211633, New SID: Extensions to the TSC Framework to support DetNet

[2] RFC 8939: Deterministic Networking (DetNet) data plane: IP

[3] SP-211634, New SID: Study on 5G Timing Resiliency and TSC & URLLC enhancements

# 2 Proposal

It is proposed to update TR 23.700-46 as follows.

# * * * * First change (all new text)* * * *

## 5.X	Key Issue #X: DetNet Controller to 5GS interface

5.X.1 	Description

The objective of this Key Issue is to define the interface between DetNet Controller and 5GS based IP data plane.

Working assumptions:

- DetNet IP data plane is supported based on 5GS R17 TSC QoS procedures available for IP PDU sessions with potential enhancements in scope of other R18 SIDs, in particular for 5TRS_URLLC.

- DetNet Controller – 5GS interface is based on 5GS R17 exposure APIs with potential extensions necessary for DetNet controller plane operations. These API(s) are exposed by the TSCTSF with possibly the NEF acting as an API gateway enforcing security, parameter mapping, etc. as already defined in R17 TS 23.501[2].

- The DetNet controller communicates with the 5G system through TSCTSF. DetNet Flow-Related information may be provided by the DetNet controller as defined in RFC 9016 [X3] but the way to carry this information is to be studied. The TSCTSF should map the parameters provided by the DetNet controller into 5G QoS parameters or other parameters e.g. TSCAI to configure flow transferring path for the DetNet traffic.

- The IETF RFC 8655 [x] clause 4.8 describes the Resources, Capabilities, and Adjacencies which may be reported by DetNet node to DetNet CPF (Controller Plane Function) but the way to carry this information is to be studied.

For this Key Issue the following areas will be studied:

-	What information the 5GS acting as an DetNet IP Data Plane Node needs to expose to the DetNet Controller so that the Controller can perform path and worst-case end-to-end delay calculations for DetNet IP traffic flows served by the 5GS,

This may imply to study

-	which information 5GS needs to report;Which attributes are exposed from the 5GS to a DetNet controller, and how that information can be provided. Whether any other information should be exposed to a DetNet controller.

-	How the 5GS can expose the status of the DetNet flows and configuration in 5GS to the DetNet controller. How the 5GS can indicate that a particular required resource (e.g. bandwidth or max latency) for DetNet flow(s) cannot be fulfilled.

-	how the 5GS collects the information to report.

- .

-	What information the 5GS acting as an IP Data Plane Node needs to receive from the Controller after the Controller decides to setup an DetNet IP data flow to be served by the 5GS with specific QoS requirements,

This may include work on

- Which parameters provided by the DetNet controller should be mapped into which 5G parameters: what mechanisms are used for this mapping.

- How the 5GS finds the PDU Sessions corresponding to the given DetNet configuration, and how it maps the detnet attributes to PDU Session specific attributes

- Whether and how the DetNet configuration is stored in 5GS, especially when no PDU Session can be found for some of the attributes in the DetNet configuration

- How the 5GS authorizes a request from DetNet controller

- What parts of the required information can already be supported by the existing R17 5G exposure APIs and what additional information needs to be defined for them

# * * * * End of changes * * * *