SA WG2 Meeting #S2-149E	S2-2200297r01

14 February – 25 February 2022, Electronic, Elbonia	(revision of S2-220xxxx)

Source:	Futurewei

Title:	Key Issue: Configuration of Vehicle Mounted Relay

Document for:	Approval

Agenda Item: 	9.9

Work Item / Release:	FS_VMR /Rel-18

Abstract of the contribution: This paper proposes a key issue on VMR system configuration to be studied in FS_VMR.

# 1	Discussion

The key issue on Mobile base station relay and related functionality for TR 23.700-05 is proposed based on the study item description in SP-211636.

# 2	Proposal

It is proposed to adopt the following changes into TS23.700-05.

* * * * 1st Change * * * *

# 5	Key Issues

## 5.x	Key Issue #X: Mobile base station Relay Configuration

### 5.x.1	Description

Mobile base station relays may attach or detach to a mobile network depending on its location and movement. When a mobile base station relay has just attached to the network, it lacks the configuration data to serve as a mobile relay. Mobile base station relays may therefore need to be dynamically configured. While a mobile base station relay may attach to 5GS using existing procedures, it does not have several policies to function effectively, including charging data to be collected.

This key issue should investigate the policies and data to be configured for the mobile base station relay to be operational and the data to be provided by the mobile base station relay for accounting for the packets relayed.

The following aspects shall be studied:

- how to configure, provision, and control a mobile base station relay.

- what is needed in 5GC for managing mobile base station relay’s operational lifecycle including activation and deactivation.

- Investigate if any changes in 5GS are needed to support the collection of charging information for the packets that are relayed.

NOTE: Charging support will be coordinated with SA5.

* * * * End of Changes * * * *