3GPP TSG-SA WG2 Meeting #149E (e-meeting)	S2-2200317r01

# Feburary 14-25, 2022, Elbonia

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0498 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on Analytics Transfer for Group of UEs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2022-01-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the current version of TS 23.288, it is defined that source NWDAF can select target NWDAF and trigger the transfer of Analytics context. However, it is not clear whether and how the source NWDAF should trigger the Analytics transfer procedure when the Target of Analytics Reporting is a group of UEs, especially when the UEs are scattered in different areas. There are two scenarios we need to consider when the Target of Analytics Reporting is a group of UEs: Scenario 1: The consumer is only interested in a certain area Scenario 2: The consumer does not specific the area of interest In scenario 1: the consumer may select an NWDAF or an aggregator NWDAF covers the AoI. If some UEs in the group moves out of the AoI, the NWDAF does not need to track those UEs, since their data will not be computed in the Analytics anymore. In scenario 2: the consumer should select an central NWDAF which can cover the whole PLMN, in such case the NWDAF can always serve the UEs unless the UE moves to other PLMNs which is not discussed in Rel-17. Note that the central NWDAF can also act as an aggregator NWDAF, that it can select the distributed NWDAFs who can serve the UEs. The aggregator NWDAF should track all of the UEs in the group and select or reselect appropriate NWDAFs as the distributed NWDAFs. For a distributed NWDAF, if some of the UEs in the group moves out of its serving area, it can continue generating the analytics using the data of the UEs who are still in its serving area. The aggregator NWDAF will take care of the UEs who is moving, e.g. the aggregator NWDAF may select an new NWDAF that can serve the UEs that are moving, The aggregator NWDAF can provide the NWDAF IDs to the new NWDAF, so that it may request some analytics context from the NWDAFs who served those UEs. Alternatively, the consumer may select an NWDAF that covers the area where the whole group of UEs are located. When some UEs moves out of the serving area of the NWDAF, the NWDAF will transfer the analytics subscription to an aggregator NWDAF who covers bigger area. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that Analytics Subscription transfer initiated by the source NWDAF in the case that the Target of Analytics Reporting is a group UE and the trigger of the Analytics Subscription transfer is UE mobility, the source NWDAF will either do nothing or transfer the analytics subscription to an aggregator NWDAF if current analytics subscription is not from an aggregator NWDAF |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | For UE mobility, It is not clear whether and how the source NWDAF should trigger the Analytics transfer procedure when the Target of Analytics Reporting is a group of UEs or any UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1B.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * First Change * * * *

## 6.1B	Transfer of analytics context and analytics subscription

### 6.1B.1	General

In a multiple NWDAFs deployment scenario, procedures for transfer of analytics context and analytics subscription can be used to support the target NWDAF to produce the needed analytics.

When the analytics consumer provides the target NWDAF with information on the subscription that could be transferred from the source NWDAF, the target NWDAF may initiate the transfer of analytics context. The analytics consumer provides the information via Nnwdaf_AnalyticsSubscription_Subscribe service operation. When the analytics consumer is an AMF, the old subscription information (if related to a UE) may be provided by the source AMF to the target AMF using a UE context transfer procedure as specified in TS 23.502 [3].

An analytics subscription transfer to the target NWDAF may be initiated by the source NWDAF, followed by an analytics context transfer initiated by the target NWDAF. An NWDAF may transfer one or more of its analytics subscriptions to another NWDAF instance due to internal (e.g. load balancing, graceful shutdown) or external triggers (e.g. UE mobility). For external triggers, the NWDAF may subscribe to NF(s) to be notified about the corresponding events. As for UE mobility, upon the UE location change event notified by the AMF subscribed by the NWDAF, the NWDAF determines whether it can continue to provide the analytics service. If the NWDAF cannot continue to serve the consumer, it should either select a target NWDAF and initiate analytics subscription transfer, or notify the analytics consumer that it cannot provide the service anymore, so that the analytics consumer can select a new NWDAF. If the Target of Analytics Reporting is a group of UE(s), the source NWDAF will not do any analytics subscription transfer if the UEs moves out of its serving area and the AoI is provided in the subscription; if an AoI is not provided in the subscription, the source NWDAF may transfert the analytics subscription to an NWDAF that can serve the whole group, if the current analytics subscription is not from such NWDAF.

Procedures for analytics subscription transfer allow one NWDAF instance to transfer its ongoing analytics subscriptions to another NWDAF instance. The transfer can be done for all subscriptions or just a selected subset of subscriptions related to specific area(s), specific Analytics ID(s), specific NF(s) and/or specific UE(s).

The procedure for prepared analytics subscription transfer can be used if the source NWDAF instance anticipates that it will soon not be able to continue its current analytics tasks.

* * * * End of Changes * * * *