SA WG2 Meeting #S2-149e(e-meeting)	S2-2200328

14 - 25 February 2022, Electronic Meeting	(revision of S2-22xxxxx)

Source:	Ericsson, [Vivo], [Huawei, HiSilicon]

Title:	RedCap ph2 study key issues

Document for:	Approval

Agenda Item:	9.8

Work Item / Release:	FS_RedCap_Ph2

Abstract of the contribution: Introduce key issue1 for RedCap Phase 2 study.

# 1. Introduction

This paper proposes the key issues for RedCap Phase 2 study based on agreed objective in SID SP-211635 below:

# 4	Objective

The aim of this study is to investigate potential enhancements of 5GS that would enable the support of RRC_INACTIVE state with extended DRX cycle longer than 10.24s.

NOTE 1: RRC_INACTIVE is defined as “CM-CONNECTED with RRC_INACTIVE” in current 3GPP specification (See TS 23.501 clause 5.3.3.2.5 and TS 38.300 clause 9.2.2.1). The study will not restrict proposals for other CM state based solutions, i.e., CM-IDLE.

The following aspects are in the scope of the study:

- Support for enabling of eDRX cycle longer than 10.24s in RRC_INACTIVE state.

  - Study the support the of eDRX cycle longer than 10.24s and up to 10485.76s for RRC_INACTIVE state for RAN paging (e.g. PTW and eDRX cycle values).

  - Study the MT signalling and data handling for UE with long eDRX cycle in RRC_INACTIVE state.

# 2. Proposal

This paper proposes the following updates to TR 23.700-68 clause 5.

* Start of change *

# 5	Key Issues

Editor's note:	This clause provides a short description of the key issue.

## 5.1	Key Issue #1: Enabling long eDRX cycle support in RRC_INACTIVE state

### 5.1.1	General description

To enable the support of long eDRX cycle (e.g. longer than 10.24s) in RRC_INACTIVE state with the power saving benefit studied in TR 38.875 [5], following aspects need to be studied:

-	How the long eDRX cycle values which is longer than 10.24s are supported in RRC_INACTIVE state.

-	How MT signalling and data are handled for UE with long eDRX cycle value in RRC_INACTIVE state, including enhancement to enable extended data buffering for UE in RRC_INACTIVE state.

NOTE 1: RRC_INACTIVE is defined as “CM-CONNECTED with RRC_INACTIVE” in current 3GPP specification (See TS 23.501 [2] clause 5.3.3.2.5 and TS 38.300 [4] clause 9.2.2.1). The solutions will not restrict proposals for CM-IDLE state.

NOTE 2: Coordination with RAN WGs is needed for the KI.

* End of changes *