3GPP TSG-WG SA2 Meeting #149E e-meeting 	S2-2200388r07

Elbonia, February 14th – 25th, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon, Nokia, Nokia Shanghai Bell, Ericsson

Title:	Key Issue: Support Third Party ID Access in IMS

Document for:	Approval

Agenda Item:	9.17

Work Item / Release:	FS_NG_RTC / Rel-18

Abstract: This paper proposes to add third party ID Key Issue in FS_NG_RTC TR.

1. Introduction

The NG_RTC SID includes the following objective:

WT#3 Study whether and how supporting the requirements from SA1 R18 eMMTEL WID impacts IMS architecture and functionalities:

-	WT#3.2: To support 3rd party (e.g. enterprise) to verify via operator network that the caller (e.g. employee) is authorized to use or reference specific identity information which can be securely demonstrate to the callee when placing a call.

2. Discussion

When a third party (refer to an enterprise) accesses the operator IMS network and initiates a call, the call accept successful rate could be improved if the identity of the third party (refer to the enterprise) or even its employee identity can be verified and passed to called party, along with the phone number. At the same time, operators are troubled by unwanted calls initiated by unidentified entities through trunk access. In traditional normal user subscription scenario, operator needs to provision third party (refer to the enterprise) employee data for employee identity presentation, which will be influenced by frequent changes of employment relationships. Therefore, it is necessary to make some changes in operator network to let an authorized third party (refer to an enterprise) access operators to initial calls.

- When a third party (refer to an enterprise) accesses the IMS network, the IMS network should authenticate the third party and let the verified caller identity can be secure demonstrated to the called party.

- The prerequisite for authentication is subscription. The third party (refer to an enterprise) should order service subscription with the operators before accessing the carrier's network to initiate a call. The third party (refer to an enterprise) should be able to let its user (e.g. its employees) to initiate calls on behalf of the enterprise through the subscription. For details, see section 5.6.2 in TS 22.873. At present, IMS only supports individual-level subscription, and maybe there are some architecture impacts in supporting of the subscription for a third party and relative signaling procedure.

- According to clause 6.39.3 in TS 22.261, a third party (refer to an enterprise) should authenticates its user (e.g. its employees) and assigns a specific ID to the user (e.g. employees). The IMS network needs to use secure methods to verify the access of the user of the third party.

- When the user using a third party ID setups a call in the IMS network, how to present its caller identity along with the phone number, how to describe the specific call process, and how to know that the presented information of the calling party has not been tampered with are also key issues to be addressed.

- The call needs to be routed to the third party (refer to an enterprise) in call back scenario.

3. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-87.

# * * * * First change * * * *

5.X	Key issue #X: third party specific user identities

5.X.1	Description

The Key Issue is to study the architecture impact of third party specific user identities accessing IMS network, including:

- The following bullets apply to the scenario that third party users connect to carrier’s IMS and initiate the call:

  - Study and if needed, define a mechanism how the originating IMS network can authorize a third party, engaged in the IMS session, whether it is allowed to use third-party specific identities for its authorized subscribers.

  - Study and if needed, define a mechanism how the terminating IMS network can support the called party to verify third-party specific identities engaged in the IMS session.

  - Study whether and how IMS procedures need to be enhanced to support authentication, authorization, as well as verification to use third-party specific identities, which should be performed by the IMS network and third party. This includes studying potential impacts to the call-back procedure. Impacts on STIR/SHAKEN to be covered as well.

- Study and if needed, identify required enhancements to an IMS subscription to support trusted third parties.

NOTE: Security related aspects should be studied by SA WG3.

NOTE: It is assumed that the relationship between third party user and third party is not statically stored in IMS network.

# * * * * End of changes * * * *