SA WG2 Meeting #149e	S2-2200410r02

# February 14th – 25th, 2022; Elbonia               		(revision of S2-220)

Source: 	China Telecom

Title: 	FS_5WWC_Ph2 Solution related with WT#2

FS_5WWC_Ph2 TR Solution related with WT#2

Document for: 	Approval

Agenda Item: 	9.13

Work Item / Release:	FS_5WWC_Ph2 / Rel-18

Abstract of the contribution:

1	Discussion

Void

2 Proposal

It is proposed to create TR 23.XXX on FS_5WWC_Ph2 with the attached skeletton

*** Start of the change (all new text) ***

6.X	Solution #X: New method for non-3GPP device connected behind a 5G-RG

6.X.1	Description

This solution addresses Key Issue“Providing differentiated service for UE and Non-3GPP devices connected behind a 5G RG.”

Figure 6.x.1 depicts the architecture for devices connected behind a 5G-RG. In this solution, the AF, which supports the functionality of assigning virtual identifier (e.g. virtual SUCI) can preconfigure a list of the virtual identifier to the UDM through NEF. After the 5G-RG registers to the 5GC, a virtual identifier would be assigned to it as the identifier of the device which has been connected to 5G-RG. Then the 5G-RG could use the virtual identifier to initial registration procedure and PDU session establishment procedure via W-AGF on behalf of the device. Different QoS profiles may be applied per device behind the 5G-RG by 5GC.

NOTE 1: AF in figure 6.x.1 is managed by operator.

NOTE 2: The interaction between the AF and 5G-RG may be out of 3GPP scope.

NOTE 3: The virtual identifier corresponds to 5G-RG’s subscription information in UDM.

Figure 6.x.1 architecture for non-3GPP devices connected behind a 5G-RG

6.X.2	Procedures

Figure 6.x.2-1: Procedure for 5G RG and Device Registration

0. The AF preconfiguresthe list of virtual identifier (e.g. virtual SUPI) to UDM through the NEF, so that the virtual identifier can be seen as a legal identifier when 5G-RG use it to perform authentication procedure on behalf of non-3GPP device.

1. The 5G-RG registration procedure, as descibed in clause 7.2.1.1 of in TS 23.316.

2. The UDM informs the AF that the 5G-RG has been successfully registered with 5GC. AF should save a list of SUCI of registered 5G-RGs.

3. The device connects to the 5G-RG via a layer-2 (L2) connection.

4. The 5G-RG applies to the AF for a virtual identifier by carrying the SUCI of the 5G RG and the MAC address of the device.

5. The AF checks if the 5G-RG is in the list of registered 5G-RGs. If it has registered successfully, the AF will inform the UDM of the mapping between the virtual identifier and the MAC address of the non-3GPP device.

6. If the 5G-RG is in the list of registered 5G-RGs, the AF responds to the 5G-RG by assigning a virtual identifier to the device. The AF saves the timestamp and the mapping information between the virtual identifier and the MAC address of the device.

NOTE:	In step 4 and 6, the AF and 5G-RG communicate directly without going through the 5GC. Step 4 and 6 may be out of the 3GPP scope.

7. The 5G-RG uses the virtual identifier to initial registration procedure on behalf of the device, as described in clause 7.2.1.1 of TS 23.316.

Editor’s Note: modification of registration procedure in clause 7.2.1.1 of TS 23.316 is FFS

8. 5G-RG initates PDU Session Establishment procedures on behalf of device. And differentiated QoS can be provided to non-3GPP device according to the virtual identifier.

6.X.3	Impacts on Existing Nodes and Functionality

UDM:

- Inform AF that a 5G-RG has been successfully registered.

- Store the mapping between the virtual identifier and the MAC Address of the non-3GPP device

5G-RG:

- Apply to the AF for a virtual identifier

- Store the mapping information between the virtual identifier and the MAC address of the non-3GPP device

- Support performing registration and PDU Session related procedure on behalf of non-3GPP device behind.

### *** End of the change ***