3GPP TSG-WG SA2 Meeting #149E e-meeting 	S2-2200471r02

Elbonia, February 14 – 25, 2022	(revision of)

Source:	Ericsson

Title:	New Key Issue: Efficient Resource Utilization for 5MBS Broadcast in MOCN Network Sharing

Document for:	Approval

Agenda Item:	9.18

Work Item / Release:	FS_5MBS_Ph2 / Rel-18

Abstract: This paper proposes a key issue on efficient resource utilization for 5MBS Broadcast in MOCN network sharing.

# 1. Discussion

The 5MBS phase 2 SID (SP-211645 approved in SA#94E) contains the following working task:

WT#1.2 Study feasible and efficient resource utilization for the same broadcast content to be provided to 5G MOCN network sharing scenarios (i.e., multiple CNs are connected to the same NG-RAN); NOTE 2: The NG-RAN is assumed not to be aware of the same content via the application layer information detection. NOTE 3: Collaboration with RAN WGs is needed.

Per TS 23.501 (clause 5.18), in a 5G Multi-Operator Core Network (5G MOCN), multiple CNs are 
connected to the same NG-RAN, see below:

Figure 1 5G Multi-Operator Core Network (5G MOCN)

The enablers for 5MBS MOCN network sharing are mainly from the Mobile Operator Networks, as it can bring resource efficiency and reduce CAPEX. In addition, such feature has been discussed and well-accepted by the RAN WGs. As per the approved WID NR_MBS_enh for Rel-18:

Network sharing is a common practice to reduce network CAPEX. With RAN sharing deployment, if the same Multicast/Broadcast service is provided by two (or more) operators separately, this service would be recognized as separate TMGIs resulting in duplicated PTM radio resources consumption in the same cell for transmission of the same content. This justifies resource efficiency improvement in the RAN sharing scenario.

Study and if necessary, specify enhancements to improve the resource efficiency for MBS reception in RAN sharing scenarios [RAN3]

When the same broadcast content is to be delivered to multiple CNs, the AF will set up multiple broadcast MBS sessions towards those CNs, each CN delivers the same content towards the same shared NG-RAN node. Therefore, the same broadcast content will be sent over radio interfaces N times, where N is the number of CNs involved.

To investigate the feasibility of optimizing radio resource utilization so that the same broadcast content is sent only once over radio interface, the following aspects need to be considered:

-	Whether and how to assist NG-RAN node to determine the same content is delivered by broadcast MBS Sessions from different 5G CNs?

-	Whether and how to assist NG-RAN node to determine which PLMN is used to broadcast the MBS session data?

-  Which entity (e.g., AF or other NFs) could provide the assistance parameters to the shared NG-RAN if needed?

-	Whether and how to enable the UE to receive the broadcast content from the broadcast PLMN when the UE camps on cells of other PLMNs?

R02: merge S2-2200607 and S2-2200903 in.

# 2. Proposal

It is proposed to capture the following changes in TR 23.700-47.

# * * * * First change (all text are new) * * * *

# 5	Key Issues

## 5.X	Key Issue #X: 5MBS MOCN Network Sharing

### 5.X.1	Description

Per TS 23.501 (clause 5.18), in a 5G Multi-Operator Core Network (5G MOCN), multiple CNs are connected to the same NG-RAN.

When the same broadcast content is to be delivered to multiple CNs, the AF will set up multiple broadcast MBS sessions towards those CNs, each CN delivering the same content towards the same shared NG-RAN node. Therefore, for a broadcast MBS Session, the consumed radio resource will be (N-1) times more than needed, where N is the number of CNs involved.

To investigate the feasibility of avoiding allocating more radio resource than needed, the following aspects need to be considered:

-	Whether and how to assist shared NG-RAN node to determine the same content is delivered by broadcast MBS Sessions from different 5G CNs?

-	Whether and how to assist shared NG-RAN node to determine which PLMN is used to broadcast the MBS session data?

-  Which entity (e.g., AF or other NFs) could provide the assistance parameters to the shared NG-RAN if needed?

-	Whether and how to enable the UE to receive the broadcast content from the broadcast PLMN when the UE camps on cells of other PLMNs?

-	How to ensure that all UEs from the multiple MOCN PLMNs will be able to have the security material required in order to be able to “consume” the content?

-	Related to above how to ensure that each of the multiple MOCN PLMNs will be able to independently perform “access control” to the content based on their subscription of their UE?

NOTE 1: The feasibility of radio resource utilization optimization will be determined by RAN WGs.

NOTE 2: Collaboration with SA3 is required

# * * * * End of changes * * * *