3GPP TSG-SA WG2 Meeting #149E	S2-2200480

# E-meeting, Feb 14- Feb 25 2022	(revision of S2-22xxxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3511 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CH architecture update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN |  |  |  |  |  | Date: |  |  | 2022-01-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In LS C1-215137, there is a question from CT1 regarding the NSSAA requirement on SNPN. In addition, CT1 had assumed that NSSAA can be performed in an SNPN throughout Rel-16. However, since Figure 5.30.2.9.3-1 of 3GPP TS 23.501 does not show NSSAAF, it was questioned whether NSSAA can be performed in an SNPN. Thus, CT1 would like to ask SA1 and SA2 to clarify whether there is a requirement to enable NSSAA in an SNPN. Also, there is LS reply from SA1 S1-214234 adding requirement on performing authentication/authorization for UE accessing network slices of non-public network. The 5G system shall support a mechanism for the non-public network to authenticate and authorize UEs for access to network slices of that non-public network. To be able to address SA1 requirement, it is proposed to re-use the functionality in PLMN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add clarification in credentials holder architecture figure , so that NSSAA is supported per SA1 requirement. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NSSAA for UE from HPLMN cannot be performed when UE is registered in SNPN using CH feature, or completely new functionality is needed to address SA1 requirement. NSSAA for UE from one SNPN cannot be performed when UE is registered in another SNPN using CH feature, or completely new functionality is needed to address SA1 requirement. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.30.2.9.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

##### 5.30.2.9.3	Credentials Holder using AUSF and UDM for primary authentication and authorization

An SNPN may support primary authentication and authorization, and optionally support Network Slice-Specific Authentication and Authorization, of UEs that use credentials from a Credentials Holder using AUSF and UDM. The Credentials Holder may be an SNPN or a PLMN.

Figure 5.30.2.9.3-1 depicts the 5G System architecture for SNPN with Credentials Holder using AUSF and UDM for primary authentication and authorization.

NOTE:	The architecture for SNPN and Credentials Holder using AUSF and UDM is depicted as a non-roaming reference architecture as the UE is not considered to be roaming, even though some of the roaming architecture reference points are also used.

Figure 5.30.2.9.3-1: 5G System architecture with access to SNPN using credentials from Credentials Holder using AUSF and UDM

END CHANGE