3GPP TSG-SA WG2 Meeting #149E (e-meeting)	S2-2200561r03

# February 14 – 25, 2022, Elbonia

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.256 | CR | 0052 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on UAV Re-authorization procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ID_UAS |  |  |  |  |  | Date: |  |  | 2022-01-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Notification Endpoint is used for AMF/SMF to implicitly subscribe to UAS NF/NEF, and for UAS NF/NEF to implicitly subscribe to USS. This parameter also should be used during C2 authorization procedure. Clause 5.2.4.3 and 5.2.4.4 describes UAV Re-authorization procedure that includes not only UUAA re-authorization but also C2 re-authorization. In case of C2 re-authorization, regardless of UUAA-MM or UUAA-SM was performed, UAS NF/NEF should send Nnef_Authentication_Notification to the SMF serving C2 communication at step 3b. Because UAS NF/NEF received Notification Endpoint from SMF during the C2 authorization procedure. However, currently it is considered and described only if UUAA-MM or UUAA-SM was performed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 5.2.2.2: - Correct the typo. UAS NF/NEF USS Clause 5.2.4.3 and 5.2.4.4: when UAS NF/NEF receives Naf_Authentication_Notification message from USS, based on the received Notification Correlation Information, the UAS NF/NEF determines the corresponding Notification Correlation Information for Nnef_Authentication_Notification request - UAS NF/NEF sends UUAA re-authorization message to corresponding AMF/SMF based on the Notification Correlation Information - UAS NF/NEF sends C2 re-authorization message to corresponding SMF based on the Notification Correlation Information - Correct service operation name Cause 5.2.5.2.3 Notification Endpoint is also used during UE initiated PDU Session Establishment for C2 Communication procedure |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UAV Re-authorization procedure is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.2, 5.2.4.3, 5.2.4.4, 5.2.5.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

5.2.2.2	UUAA-MM Procedure

Figure 5.2.2.2-1: UUAA-MM procedure

1.	For a UE that requires UUAA or when triggered by re-authentication by USS, the AMF triggers a UUAA-MM procedure. If the UE does not have an Aerial subscription in the UE subscription data retrieved from the UDM, the AMF shall not trigger a UUAA-MM procedure.

2.	AMF to UAS NF/NEF: The AMF invokes Nnef_Authentication_AuthenticateAuthorize Request message. For initial authentication, this shall include the GPSI and the CAA-Level UAV ID and may include USS address (e.g. FQDN), UUAA Aviation Payload if it was provided by the UE. For re-authentication triggered by AMF, this may not include the CAA-Level UAV ID. UAS NF resolves the USS address based on CAA-Level UAV ID or uses the provided USS address, as described in clause 4.4.2. In addition, the AMF may also include the User Location Information (e.g. Cell ID). The UAS NF should store the serving AMF ID.

The AMF identifies the UAS NF/NEF based on local configuration or by NF discovery procedure using DNN/S-NSSAI and/or UE provided identity e.g. USS address.

The AMF also provides a Notification Endpoint to the UAS NF/NEF, so that UAS NF/NEF can include this Notification Endpoint together with UUAA updated parameters, as shown in clause 5.2.4. By providing the Notification Endpoint, the AMF is implicitly subscribed to be notified of re-authentication, update authorization data or revocation of UAV from UAS NF/NEF, if the UUAA result is successful in step 5.

NOTE 1:	Security details will be determined by SA WG3.

3.	UAS NF/NEF to USS: Naf_Authentication_AuthenticateAuthorize Request message, shall include the GPSI and CAA-Level UAV ID and optionally UAV location obtained from AMF in step 2 e.g. to support geo-caging functionality. UAS NF/NEF may translate the Cell ID received as UAV location from AMF in step 2 into a corresponding geographic area and/or may further obtain the UE location information using Location Service Procedures as defined in TS 23.273 [8].

The UAS NF/NEF also provides a Notification Endpoint to the USS, so that USS can include this Notification Endpoint together with UUAA updated parameters, as shown in clause 5.2.4. By providing the Notification Endpoint, the UAS NF/NEF is implicitly subscribed to be notified of re-authentication, update authorization data or revocation of UAV from USS, if the UUAA result is successful in step 5.

4.	[Conditional] Multiple round-trip messages as required by the authentication method used by USS. Naf_Authentication_AuthenticateAuthorize Response messages from USS shall include GPSI and shall include a authentication message based on authentication method used that is forwarded transparently to UE over NAS MM transport messages. The authentication message in step4d may contain UUAA Aviation Payload required by the USS if it was not provided by the UE before.

5.	USS to UAS NF/NEF: (final) Naf_Authentication_authenticate Response message, shall include: GPSI, a UUAA result (success/failure) for the UAV and the UAS NF, may include an authorized/new CAA-Level UAV ID for the UAV and a UUAA Authorization Payload to the UAV (e.g. security info to be used to secure communications with USS), and a final authentication message (e.g. indicating success or failure, and if the UUAA is for re-authentication, indicating whether the UAS service related network resource can be released in case of UUAA failure) based on authentication method used that is forwarded transparently to UE over NAS MM transport messages.

The Naf_Authentication_AuthenticateAuthorize response also provides an Authentication Session Correlation ID to the UAS NF/NEF, so that the USS can use it to further communicate with the UAS NF/NEF (e.g. to revoke authorization), as shown in clause 5.2.4.

6.	UAS NF/NEF to AMF: (final) Nnef_Authentication_authenticate Response message, forwards information received from USS in step 5. If UUAA for re-authentication failed and UAS NF/NEF received indication that the UAS service related network resource can be released in step 5, the UAS NF/NEF includes an indication that the PDU sessions associated with the "DNN(s) subject to aerial services" can be released.

The Nnef_Authentication_AuthenticateAuthorize response also provides an Authentication Session Correlation ID to the AMF, so that UAS NF/NEF can use it to further communicate with the AMF (e.g. to revoke authorization), as shown in clause 5.2.4.

7a.	[Conditional] UAS NF/NEF to AMF: If UUAA-MM succeeded and UAS NF/NEF has not subscribed to AMF for the Mobility Event Exposure before, UAS NF/NEF subscribes to AMF for the mobility event notification by sending Namf_EventExposure_Subscribe request with the mobility events as described in TS 23.502 [3], Table 5.2.2.3.1-1 with Event ID = Reachability Filter.

7b.	[Conditional] UAS NF/NEF to AMF: If UUAA-MM failed and UAS NF/NEF has subscribed to AMF for the Mobility Event Exposure earlier, UAS NF/NEF unsubscribes to AMF for the mobility event notification by sending Namf_EventExposure_Unsubscribe request with Subscription Correlation ID.

8a.	[Conditional] AMF to UAS NF/NEF: The AMF acknowledges the subscription request from 7a by sending Namf_EventExposure_Subscribe response with Subscription Correlation ID.

8b.	[Conditional] AMF to UAS NF/NEF: The AMF acknowledges the un-subscription request from 7b by sending Namf_EventExposure_Unsubscribe response.

9.	AMF to UE: (final) NAS MM transport message forwarding authentication message from USS including authentication/authorization result (success/failure).

10.	[Conditional] if UUAA-MM succeeded, AMF triggers a UE Configuration Update procedure to deliver to the UAV authorization information from USS, as described in clause 5.2.2.1.

11.	[Conditional] If UUAA-MM fails during a Re-authentication and Re-authorization and there are PDU session(s) established using UAS services, and the USS has indicated that the network resources can be released, AMF may trigger these PDU Sessions release. AMF identifies the relevant PDU session(s) for UAS services based on the DNN/S-NSSAI value of the PDU session.

NOTE 2:	When the UUAA-MM fails during a Re-authentication, and the USS has not indicated that the network resources can be released, the USS can initiate UUAA revocation as described in clause 5.2.7.

[Conditional] if UUAA-MM fails, based on network policy the AMF may trigger Network-initiated Deregistration procedure described (as specified in clause 4.2.2.3.3 of TS 23.502 [3]) and it shall include in the explicit De-Registration Request the appropriate rejection cause value.

If there is an AMF relocation for the UAV, the new serving AMF shall notify the UAS NF about the new AMF ID and the related CAA-level UAV ID using the existing AMF event notification service.

At any time after the initial registration, the USS (via UAS NF/NEF) or the AMF may initiate Re-authentication procedure for the UAV. For AMF initiated case the Re-authentication procedure shall start from step 2. USS initiated re-authentication procedure is described in clause 5.2.4.

* * * * Start of 2nd Change * * * *

5.2.4.3	USS initiated UAV Re-authorization procedure in 5GS

Figure 5.2.4.3-1: UAV Re-authorization procedure in 5GS

UAS NF stores the UE UUAA context after successful UUAA procedure as explained in clause 5.2.2.2 for UUAA-MM and in clause 5.2.3 for UUAA-SM procedure. The UUAA context may be stored in the UDSF or may be stored locally in the UAS NF depending on deployments.

1.	The USS sends a Naf_Authentication_Notification request to UAS NF for re-authorization of the UAV. The USS includes GPSI, CAA-Level UAV ID, Notification Correlation Information, an authorization message to be transparently delivered to the UAV. The CAA-Level UAV ID may be a new CAA-Level UAV ID. The authorization message may e.g. include a UUAA Authorization Payload, a C2 Authorization Result and/or (C2) Security Information.

2.	Based on the received GPSI and Notification Correlation Information from the USS, the UAS NF/NEF determines the corresponding Notification Correlation Information for Nnef_Authentication_Notification request.

3a	For UUAA-MM re-authorization, the UAS-NF/NEF sends a Nnef_Authentication_Notification request including the CAA-Level UAV ID and the authorization message to the serving AMF.

3b	For UUAA-SM re-authorization or C2 re-authorization, the UAS-NF/NEF sends a Nnef_Authentication_Notification request to the SMF serving the UUAA or C2 for the UE which includes the corresponding PDU session identity, CAA-Level UAV ID and the authorization message.

4.	The UAS NF responds back to the USS indicating that re-authorization request has been successfully initiated.

5a.	In the case of UUAA-MM:

If the UE is in CM_Idle state, the AMF initiates the Network Triggered Service Request procedures as described in clause 4.2.3.3 of TS 23.502 [3].

The AMF delivers the CAA-Level UAV ID and the authorization message to the UE using NAS MM Transport.

5b	In the case of UUAA-SM or C2 re-authorization:

The SMF identifies, based on the received information, the PDU Session that is serving the UUAA-SM or C2 re-authorization and invokes the Network Requested PDU Session Modification procedure (figure 4.3.3.2-1 of TS 23.502 [3] triggering event SMF Requested modification) by sending Namf_Communication_N1N2MessageTransfer, including the CAA-Level UAV ID and the authorization message in the N1_SM_Container (step 3b in figure 4.3.3.2-1 of TS 23.502 [3]) .

The Network Triggered service request procedure is invoked by AMF to forward the CAA-Level UAV ID and the authorization message included in the N1_SM_container to the UE (from step 3a in figure 4.2.3.3-1 of TS 23.502 [3]).

6.	The UE receives the CAA-Level UAV ID and the authorization message (which may e.g. include a UUAA Authorization Payload, a C2 Authorization Result and/or (C2) Security Information) and act on it accordingly (outside the scope of 3GPP).

* * * * Start of 3rd Change * * * *

5.2.4.4	USS initiated UAV Re-authorization procedure in EPS

Figure 5.2.4.4-1: UAV Re-authorization procedure in EPS

UAS NF/NEF stores the UE UUAA context after successful UUAA procedure as in clause 5.2.3 for UUAA-SM procedure. The UUAA context may be stored in the UDSF or may be stored locally in the UAS NF/NEF depending on deployments.

1.	The USS sends a request to UAS NF/NEF for re-authorization of the UAV. The USS includes GPSI, Service Level Device Identity (e.g. CAA-Level UAV ID), Authentication Session Correlation Id, and authorization message to be transparently delivered to the UAV. The Service Level Device Identity (e.g. CAA-Level UAV ID) may be a new Service Level Device Identity (e.g. CAA-Level UAV ID). The authorization message may e.g. include a UUAA Authorization Payload, a C2 Authorization Result and/or (C2) Security Information.

2.	Based on the received GPSI and Notification Correlation Information from the USS, the UAS NF/NEF determines the corresponding Notification Correlation Information for Nnef_Authentication_Notification request.

NOTE:	In EPS the UUAA context is always UUAA-SM.

3.	The UAS NF/NEF sends a Nnef_Authentication_Notification request to the SMF+PGW-C serving the UUAA or C2 which includes the corresponding PDN Connection identity, Service Level Device Identity (e.g. CAA-Level UAV ID) and the authorization message.

4.	The UAS NF/NEF responds back to the USS indicating that re-authorization request has been successfully initiated.

5.	The SMF+PGW-C identifies, based on the received information, the PDN Connection that is serving the UUAA-SM and invokes the PDN GW initiated bearer modification without QoS update procedure (figure 5.4.3-1 of TS 23.401 [6]) by sending Update Bearer Request message, including the Service Level Device Identity (e.g. CAA-Level UAV ID) and the authorization message in the PCO.

The Update Bearer Request message including the Service Level Device Identity (e.g. CAA-Level UAV ID) and the authorization message is forwarded by MME as Downlink NAS Transport to the UE (steps 4 and 5 in figure 5.4.3-1 of TS 23.401 [6]).

6.	The UE receives the Service Level Device Identity (e.g. CAA-Level UAV ID) and the authorization message (which may e.g. include a UUAA Authorization Payload, a C2 Authorization Result and/or (C2) Security Information) and acts on it accordingly (outside scope of 3GPP).

* * * * Start of 4th Change * * * *

5.2.5.2.3	UE initiated PDU Session Establishment for C2 Communication

If C2 authorization is requested during PDU session establishment to a PDU session used specifically for C2 communication to UAV-C the UAV requests C2 authorization as follows.

Figure 5.2.5.2.3-1: PDU Session establishment for C2 communication (separate PDU Sessions for UAS services)

0.	The UAV has performed a successful UUAA with the USS (UUAA-SM or UUAA-MM) and the USS has for the corresponding GPSI subscribed for PDU Session Status Event from the NEF.

1.	When the UAV needs to establish C2 communication the UAV determines that a new dedicated PDU session is required for connectivity to UAV-C. The UE initiates PDU Session establishment procedure for a DNN/S-NSSAI dedicated for connectivity to UAV-C. In the PDU Session establishment request CAA-Level UAV ID and a C2 aviation payload to be used for C2 authorization shall be included and forwarded to the SMF. The pairing information includes the CAA-Level UAV IDs of the requesting UAV and identification information for the UAV-C to pair may be included in C2 aviation payload. The UAV may also include other information such as Flight Authorization information. The USS may also use its locally configured pairing information for UAV - UAV-C pairing authorization which then takes precedence over UAV provided pairing information.

2.	For a UAV with aerial subscription the SMF determines based on the DNN/S-NSSAI of the PDU session and the presence of CAA-Level UAV ID that authorization is required. The SMF then sends a Nnef_Auth_Request, which is used to request authorization to pair the UAV with UAV-C, to the UAS NF/NEF that includes the GPSI, CAA-Level UAV ID and C2 Aviation Payload and optionally the UAV location (e.g. Cell ID) if provided by the AMF and the DNN and S-NSSAI of the PDU session.

For a UAV with aerial subscription, if the SMF determines based on the requested DNN/S-NSSAI that the authorization procedure with the USS is required, but the UAV has not provided the CAA-Level UAV ID, the SMF rejects the PDU session establishment with a cause indicating that USS authorization is required.

The SMF also provides a Notification Endpoint to the UAS NF/NEF. By providing the Notification Endpoint, the SMF is implicitly subscribed to be notified of re-authorization, update authorization data or revocation of C2 connectivity from UAS NF/NEF, if the C2 authorization result is successful in step 5.

3.	The UAS NF/NEF checks that a valid UUAA is stored for the GPSI and forwards the received authorization request as a Naf_Auth_Request to the USS.

The UAS NF/NEF also provides a Notification Endpoint to the USS. By providing the Notification Endpoint, the UAS NF/NEF is implicitly subscribed to be notified of re-authorization, update authorization data or revocation of C2 connectivity from USS, if the UUAA result is successful in step 5.

NOTE:	The USS may trigger a UAV re-authentication/re-authorization in response to the query from the UAS NF/NEF.

4.	The USS performs C2 authorization based on the received information and includes in the Naf_Auth_Response sent to the UAS NF/NEF CAA-Level UAV-ID (potentially new) and included in the authorization message the C2 authorization result, i.e. whether the UAV is allowed to be paired with the UAV-C, and security information.

5.	The UAS-NF/NEF forwards the information received from the USS the Nnef_Auth_Response sent to the SMF.

6.	To informs the UE about the C2 authorization result the SMF includes the authorization result and, optionally, a new CAA-Level UAV ID if received from the USS, in the PDU Session Accept sent to the UE and let the PDU session establishment procedure continue until finalized.

If a failed C2 authorization result is received from the USS, the SMF instead rejects the PDU establishment and include a reason code indicating not authorized.

7.	[Conditional] If the C2 authorisation is successful the USS subscribes via the UAS-NF to a PDU Session Status event for the PDU session used for C2 including in the request the GPSI of the UAV. The UAS NF determines DNN, S-NSSAI corresponding to the PDU session used for C2 communication and uses this DNN, S-NSSAI to subscribe to SMF for PDU Session Status event. The SMF detects, as described in step 6-7 of figure 4.15.3.2.3-1 in TS 23.502 [3], when the PDU Session is established and send the PDU Session Status event report to the UAS NF/NEF by means of Nsmf_EventExposure_Notify message, including GPSI and UE IP Address. The UAS NF/NEF then forwards the event message to the USS.

8.	[Conditional] The USS stores the received UE IP address and invokes, with the received PDU Session IP address and the IP-address of the authorized paired UAV-C as input, the USS initiated pairing policy configuration procedure (see figure 5.2.5.2.4-1) to request corresponding traffic to be allowed on the PDU session by the UPF.

Unless a dedicated QoS is requested for the C2 flows, this procedure does not invoke any interaction with the UE, AMF or RAN.

* * * * End of Changes * * * *