SA WG2 Meeting #S2-149E	S2-2200567r01

February 14th – 25th, 2022, Electronic meeting

Title:	[DRAFT] Reply LS on Use, if any, of network provided "Indication of country of UE location"

Response to:	S2-2200043 / C1-217150

Release:	Release 17

Work Item:	5GSAT_ARCH

Source:	OPPO[to be changed to SA2]

To:	CT1

Cc:	SA1

Contact Person:

#### Name:	Jingran Chen

E-mail Address:	chenjingran@oppo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

SA2 would like to thank CT1 for their LS on Use, if any, of network provided "Indication of country of UE location".

SA2 has been aware that SA1 has indicated that there is no stage 1 service requirement on appliying the "Indication of country of UE location" and SA2 would like to provide the responses below:

The indication of country of UE location is useful to assist UE to determine its location more accurately if the country previously determined by the UE is different from the indication by the AMF. UE that is getting rejected with cause value #78 has not respected the country boundaries for PLMN selection, which implies the UE might not be aware of its own location. Without country indication from the core network, such UE would need to find PLMNs that are allowed in the UE location by trial and error attempting to register to each of them in order, only to get rejected by each available PLMN (in priority order) that is available but not allowed in the present UE location.

The country indication from the CN is intended to assist the UE by telling it in which country it is attempting to select PLMN. This indication is non-exclusive and it only indicates the country, without excluding any MCC assigned for that country or any MCC that would be allowed in the country (such as those assigned for international satellite access).

# 2	Actions

To CT1

ACTION:	SA2 kindly asks CT1 to take the above answers into account and to align their specifications with them.

# 3	Dates of next TSG SA WG2 meetings

SA2#150E	6th-12th April 2022	electronic meeting