3GPP TSG-WG SA2 Meeting #149E e-meeting 	S2-2200582

Elbonia, February 14th – 25th, 2022

Source:	vivo

Title:	Key Issue: 5GC supports efficient mobility

Document for:	Approval

Agenda Item:	9.9

Work Item / Release:	FS_VMR / Rel-18

Abstract: this paper proposes a key issue on 5GC supports efficient mobility and service continuity.

1. Introduction

According to the SP-211636, the study on Vehicle Mounted Relays, the following work tasks needs to be studied:

- WT#1: efficient mobility and service continuity for UE or a group of UEs to efficiently deliver data during different mobility scenarios (including mobility of the mobile base station relays);

The mobility of a UE or group of UEs includes following scenarios:

-	UE moves from macro base station to mobile base station, or vice versa;

-	Group of UEs move from mobile base station to macro base station;

-	UE or group of UEs move from one mobile base station to another mobile base station;

-	UE or group of UEs move together with the mobile base station.

Following aspects need to be studied for handover:

-	Handover for a UE or group of UEs between mobile base stations or between mobile base station and macro base station;

-	Handover for the mobile base station;

-	Handover combination for group of UEs and mobile base station.

Following aspects need to be studied for idle mobility:

-	Efficient mobile registration during mobility with mobile base station-

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-05.

# * * * * First change * * * *

5	Key Issues

5.X	Key Issue #X: Efficient mobility due to VMR change

5.X.1	Description

During the mobility of the mobile base station relay, a UE or a group of UEs served by the mobile base station relay will move together with the mobile base station, and when the mobile base station is, e.g., out of service area, or out of service time, the group of UEs will be served by macro base station or another mobile base station relay nearby.

Following scenarios need to be considered for the mobility due to VMR change:

-	Group of UEs move from mobile base station relay to macro base station;

-	Group of UEs move from one mobile base station relay to another mobile base station relay;

Hence, following aspects need to be studied for UEs moving to/from mobile base station relay, with the same donor-CU or a different donor-CU:

-	Efficient mobility management for a group of UEs between mobile base station relays or between mobile base station relay and macro base stations.

# * * * * End of changes * * * *