3GPP TSG-WG SA2 Meeting #149E e-meeting 	S2-2200587r02

Elbonia, February 14th – 25th, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon, Tencent, Tencent Cloud

Title:	New KI: QoS Sustainability analytics enhancement on finer granularity area and additional input data

Document for:	Approval

Agenda Item:	9.23

Work Item / Release:	FS_eNA_Ph3 / Rel-18

Abstract: It proposes a new key issue of QoS Sustainability analytics enhancement on finer granularity area and additional input data.

# 1. Introduction/Discussion

This contribution is related to WT#3.8:

WT#3.8.	enhancements on QoS Sustainability analytics.

- Whether and how to enhance the QoS sustainability analytics with finer granularity and additional input data

In SP-210619, 5GAA indicates that further enhancements of QoS predictions (e.g., below Cell level, more KPIs reporting, richer information) would benefit plenty of V2X use cases, such as, Tele-operated Driving, Tele Operated Support, Automated intersection crossing, UC Awareness Confirmation, Cooperative Lateral Parking, HD maps, and etc.

However, currently in eNA R16 and R17, NWDAF can only provide Cell level QoS Sustainability analytics with limited QoS parameters, especially for V2X use cases i.e., Cell Level UE Throughput and QoS flow retainability are currently supported. More KPIs  reported by QOS sustainability analytics would benefit for V2X use cases. For instance, for Tele-operated Driving case the DL latency and DL reliability are important QoS parameters so that prediction notifications from NWDAF would be helpful. This requirement has been also indicated by the 5GAA LS (S2-2108993).

Therefore, the following aspects are needed to be studied for QoS Sustainability analytics enhancement:

-	Whether and how to define a finer granularity area below Cell level for QoS sustainability analytics in order to derive the QoS sustainability analytics on the finer granularity area?

- 	Whether and how to enhance the QoS sustainability analytics with additional input data?

- 	Whether and how to enhance the QoS sustainability analytics with additional output?

-	Whether and how PCF to be involved in QoS sustainability analytics?

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-81.

# * * * * First change (All new text) * * * *

## 5.X	Key Issue #X: Enhancements on QoS Sustainability analytics

### 5.X.1	General description

In SP-210619, 5GAA indicates that further enhancements of QoS predictions and sustainability analytics (e.g., below Cell level, more KPIs reporting, richer information) would benefit plenty of V2X use cases, such as, Tele-operated Driving, Tele Operated Support, Automated intersection crossing, UC Awareness Confirmation, Cooperative Lateral Parking, HD maps, and etc.

Currently in R16 and R17, NWDAF can provide Cell level QoS Sustainability analytics with limited QoS parameters, i.e., RAN UE Throughput and QoS flow retainability. More KPIs reported by QoS sustainability analytics would benefit, e.g. for V2X use cases. For instance, for Tele-operated Driving case the DL latency and DL reliability are important QoS parameters so that prediction notifications from NWDAF would be helpful. This requirement has been also indicated by the 5GAA LS (S2-2108993).

Therefore, the following aspects are needed to be studied for QoS Sustainability analytics enhancement:

-	Whether and how to define the use cases which are applicable to the QoS sustainability analytics with finer granularity, and

how to enhance the QoS sustainability analytics with a finer granularity area below Cell level for such cases? or alternatively a new Analytics ID is required?

- 	Whether and how to enhance the QoS sustainability analytics (or a new Analytics ID) with additional input data?

- 	Whether and how to enhance the QoS sustainability analytics (or a new Analytics ID) with additional output (e.g. more KPIs reporting, richer information)?

-	Whether there are use cases for 5GC NFs other than AF to be service consumer of the enhanced QoS sustainability analytics (or a new Analytics ID)? If so what are the triggers to subscribe /request or actions to be taken by the 5GC NFs?

# * * * * End of changes * * * *