3GPP TSG-WG SA2 Meeting #149E e-meeting 	S2-2200600

Elbonia, February 14th – 25th, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	Key Issue: MBS data reception in RRC Inactive mode

Document for:	Approval

Agenda Item:	9.18

Work Item / Release:	FS_5MBS_Ph2 / Rel-18

Abstract: this paper proposes a key issue on MBS data reception in RRC Inactive mode.

1. Introduction

This document proposes key issues on MBS data reception in RRC Inactive mode.

According to the SP-211647, the study on 5G multicast-broadcast services Phase 2 targeted at R18 allows 5GC to support more efficient MBS content delivery and expanding the usage of MBS.

One of the Working Task that this study attempts to address is:

WT#1. How to enable support for and enhancement related to end-to-end MBS traffic delivery for, including large number of UEs: WT#1.1 Enabling UE's receiving Multicast MBS Session data in RRC states in RRC Inactive state; NOTE 1: RRC Inactive state support requires collaboration with RAN WGs.

WT#1.1 explores the potential enhancement on top of Rel-17 mechanism for the scenario with requiring the efficient resource usage. Rather than rejecting connection requests, NG RAN could temporarily move a few UEs (or all) at the time to RRC Inactive, enable those UEs to receive the downlink data in a non-feedback behavior.

Such scenario was also discussed and well-accepted by the RAN WGs. As per the approved WID NR_MBS_enh for Rel-18 (RP-213568),

-	"Specify support of multicast reception by UEs in RRC_INACTIVE state [RAN2, RAN3]

-	PTM configuration for UEs receiving multicast in RRC_INACTIVE state [RAN2]

-	Study the impact of mobility and state transition for UEs receiving multicast in RRC_INACTIVE.  (Seamless/lossless mobility is not required) [RAN2, RAN3]".

The following Figure 1 demonstrates the example scenario and aspects to be considered.

Figure 1: Example scenario.

Based on the above-mentioned Figure 1, the following aspects to be considered:

-	[A] How to assist NG-RAN node to turn UEs into RRC Inactive state. The parameters provided by 5GC;

-	[B] When and which the UEs within multicast MBS group shall be turned into RRC Inactive state;

-	Which criteria should be taken into consideration?

-	Could partial/all/none UEs in the group be switched into RRC Inactive?

-	[C] When and Which UEs shall be turn into CM-CONNECTED state;

-	How to turn UEs into CM-CONNECTED state.

-	[D] How to manage the MBS session if exist group member change into RRC-inactive state (and mobility).

.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-47.

# * * * * First change * * * *

5	Key Issues

5.A	Key Issue #A: Multicast MBS data reception in RRC Inactive state

5.A.1	Description

It is needed to explore the potential enhancement on top of Rel-17 mechanism for the scenario as mentioned in RP-213568. Rather than rejecting connection requests, NG RAN could temporarily move a few UEs (or all) at the time to RRC Inactive, enable those UEs to receive the downlink data in a non-feedback behavior.

Therefore, the following aspect will be studied for multicast:

-	Whether and how to provide MBS assistance information by 5GC for switching UEs belonging to MBS session between CM-CONNECTED state to CM-CONNECTED with RRC Inactive state by the RAN node (and vice versa). At least following aspects are to be considered:

-	Which and how 5GC NF decides the MBS assistance information;

-	Whether and how to enhance current procedures (including mobility) for MBS session with group members are in RRC Inactive state. At least following scenarios are to be considered:

-	Mobility for NG-RAN deployment with homogeneous and non-homogeneous support of MBS;

-	Local MBS and location dependent MBS;

NOTE 1:	During the study of this key issue, RAN WGs will be involved.

NOTE 2:	RAN WG will determine how the switching for the UEs belongs to MBS session between CM-CONNECTED state to CM-CONNECTED with RRC Inactive state is performed by the RAN node (and vice versa).

NOTE 3:	Seamless/lossless mobility is not required.

# * * * * End of changes * * * *