3GPP TSG-WG SA2 Meeting #149E e-meeting 	S2-2200600

Elbonia, February 14th – 25th, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon, Nokia, Nokia Shanghai Bell

Title:	Key Issue: MBS data reception in RRC Inactive mode

Document for:	Approval

Agenda Item:	9.18

Work Item / Release:	FS_5MBS_Ph2 / Rel-18

Abstract: this paper proposes a key issue on MBS data reception in RRC Inactive mode.

# 1. Introduction

This document proposes key issues on MBS data reception in RRC Inactive mode.

According to the SP-211647, the study on 5G multicast-broadcast services Phase 2 targeted at R18 allows 5GC to support more efficient MBS content delivery and expanding the usage of MBS.

One of the Working Task that this study attempts to address is:

WT#1. How to enable support for and enhancement related to end-to-end MBS traffic delivery for, including large number of UEs: WT#1.1 Enabling UE's receiving Multicast MBS Session data in RRC states in RRC Inactive state; NOTE 1: RRC Inactive state support requires collaboration with RAN WGs.

WT#1.1 explores the potential enhancement on top of Rel-17 mechanism for the scenario with requiring the efficient resource usage. Rather than rejecting connection requests, NG RAN could temporarily move a few UEs (or all) at the time to RRC Inactive, enable those UEs to receive the downlink data in a non-feedback behavior.

Such scenario was also discussed and well-accepted by the RAN WGs. As per the approved WID NR_MBS_enh for Rel-18 (RP-213568),

-	"Specify support of multicast reception by UEs in RRC_INACTIVE state [RAN2, RAN3]

-	PTM configuration for UEs receiving multicast in RRC_INACTIVE state [RAN2]

-	Study the impact of mobility and state transition for UEs receiving multicast in RRC_INACTIVE.  (Seamless/lossless mobility is not required) [RAN2, RAN3]".

The following Figure 1 demonstrates the example scenario and aspects to be considered.

Figure 1: Example scenario.

Based on the above-mentioned Figure 1, the following aspects to be considered:

-	[A] How to assist NG-RAN node to turn UEs into RRC Inactive state. The parameters provided by 5GC;

-	[B] When and which the UEs within multicast MBS group shall be turned into RRC Inactive state;

-	Which criteria should be taken into consideration?

-	Could partial/all/none UEs in the group be switched into RRC Inactive?

-	[C] When and Which UEs shall be turn into CM-CONNECTED state;

-	How to turn UEs into CM-CONNECTED state.

-	[D] How to manage the MBS session if exist group member change into RRC-inactive state (and mobility).

.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-47.

# * * * * First change * * * *

# 5	Key Issues

## 5.A	Key Issue #A: Multicast MBS data reception in RRC Inactive state

### 5.A.1	Description

Potential enhancement on top of Rel-17 for the scenario mentioned in RAN WID NR_MBS_enh (RP-213568) need to be studied: In order to provide an MBS session to more UEs in a cell, NG RAN could enable UEs within an MBS multicast session to receive MBS data while in CM-CONNECTED with RRC Inactive state.

The following aspect will be studied for multicast:

-	Whether, how and which MBS assistance information to provide from 5GC to RAN for an MBS session allowing UEs in CM-CONNECTED with RRC Inactive state to receive MBS content, including the aspect

which 5GC NF(s) determine the MBS assistance information and how they do so;

-	Whether and how to enhance the current procedures (including mobility related procedures) for MBS session with members in RRC Inactive state.

NOTE 1:	During the study of this key issue, RAN WGs will be involved.

NOTE 2:	RAN WG will determine how the switching for the UEs belongs to MBS session between CM-CONNECTED state to CM-CONNECTED with RRC Inactive state is performed by the RAN node (and vice versa).

# * * * * End of changes * * * *