3GPP TSG-WG SA2 Meeting #149E e-meeting 	S2-2200649

Elbonia, February 14th – 25th, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	FS_GMEC TR 23.700-74 Definitions of terms and abbreviations

Document for:	Approval

Agenda Item:	9.2

Work Item / Release:	FS_GMEC / Rel-18

Abstract: This paper introduces the definitions of terms and abbreviations for TR 23.700-74

# 1. Introduction/Discussion

The definitions of terms and abbreviations for FS_GMEC TR 23.700-74 “Study on generic group management, exposure and communication enhancements”

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-74 V0.0.0.

# * * * * First change * * * *

# 3	Definitions of terms and abbreviations

## 3.1	Terms

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1], in 23.501 [2] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Example: text used to clarify abstract rules by applying them literally.

The following definitions of terms are copied from TS 23.501 [2]:

5G LAN-type Service: A service over the 5G system offering private communication using IP and/or non-IP type communications.

5G LAN-Virtual Network: A virtual network over the 5G system capable of supporting 5G LAN-type service.

5G VN Group: A set of UEs using private communication for 5G LAN-type service.

Private communication: A communication between two or more UEs belonging to a restricted set of UEs.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], in 23.501 [2] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

<ABBREVIATION>	<Expansion>

5G LAN	5G Local Area Network

5G VN	5G LAN-Virtual Network

# * * * * End of changes * * * *