SA WG2 Meeting #S2-149E	S2-2200690r08

14 February – 25 February 2022, Electronic, Elbonia	(revision of S2-190xxxx)

Source:	CATT, China Mobile, Tencent

Title:	Architecture assumptions and principles

Document for:	Approval

Agenda Item:	9.11

Work Item / Release:	FS_EDGE_Ph2 / Rel-18

Abstract of the contribution: this contribution proposes the architectural assumption of FS_EDGE_Ph2.

Introduction

The list of assumptions and principles below are drafted considering WTs in SP-211601 in general.

Proposal

It is proposed to adopt the following changes into TR 23.700-48.

***** BEGIN 1st CHANGE *****

4	Architectural assumptions and requirements

## 4.1	Architectural Assumptions

The architecture for support of Edge Computing in 5GC shall be based on the following architecture assumptions:

-  The architecture for Edge computing specified in Release 17 is

used as basis for further potential enhancement.

## 4.2	Architectural Requirements

The architecture for support of Edge Computing in 5GC shall be based on the following architecture requirements:

-   The solutions should be applied to the Edge Hosting Environment (EHE) which is under control of either the operator or 3rd parties.

-   The solutions should be possible to support an Edge Hosting Environment (EHE) which is under control of a group of operators.

-   The solutions should minimize the impact on the application layer.

***** End of CHANGE *****