SA WG2 Meeting #149e	S2-2200723r02

# February 14th – 25th, 2022; Elbonia               	       (revision of S2-220xxxx)

Source: 	vivo, InterDigital

Title: 	FS_eNPN_Ph2 KI proposal of Returning to Home Network

Document for: 	Approval

Agenda Item: 	9.4

Work Item / Release:	FS_eNPN_Ph2 / Rel-18

Abstract of the contribution: This contribution proposed to add a KI of mechanism to stagger the return of UEs to their home network.

# 1	Discussion

TS 22.261 clause 6.41.2.7 describes requirements of mitigate 5GS overload when large number of UEs are returning to the home network.

#### 6.41.2.7	Returning to Home Network

The 5G system shall provide mechanisms to mitigate user plane and control plane overload caused by a high number of UEs returning from a temporary local access of a hosting network to their home network in a very short period of time.

The 5G system shall provide mechanisms to minimize the impact on the UEs communication e.g., to prevent user plane and control plane outages when returning to a home network together with other high number of UEs in a very short period of time, after terminating their temporary local access to a hosting network.

According to SA1 specification TR 22.844, when local service is over, large number of UEs would attempt registration back to their home network. This may lead to a signaling peak in the home network and result in longer time scales for users to re-register to/re-select their home network. Therefore, it is desirable that user registrations are managed by spreading out the registration attempts over time and limiting the number of users attempting to register simultaneously, to avoid signaling overload and unnecessary waiting times for the returning users.

Therefore, it is proposed to study the mechanism to stagger the return of UEs to their home network, so as to spread out registration attempts over time and keep the number of UEs attempting to register simultaneously within a manageable limit.

# 2 Proposal

It is proposed to update TR 23.XXX on FS_eNPN_Ph2 as follows.

# 5	Key Issues

## 5.X	Key Issue #X: Support for returning to home network

### 5.X.1	Description

According to SA1 specification TR 22.844, when local service is over, large number of UEs would attempt registration back to their home network. This may lead to a signaling peak in the home network and result in longer waiting for users to re-register to/re-select their home network. Therefore, it is desirable that user registrations are managed by spreading out the registration attempts over time and limiting the number of users attempting to register simultaneously, to avoid signaling overload and unnecessary waiting times for the returning users.

There are various load control mechanisms already defined e.g.:

-	Access control and barring as defined in clause 5.2.5 of TS 23.501

-	Control Plane Load Control, Congestion and Overload Control as defined in clause 5.19 of TS 23.501

This key issue aims to study whether the existing mechanisms for overload control in the network can support all the use cases for supporting PALS and whether additional mechanisms need to be defined including::

-  Wheher and how to mitigate user plane and control plane overload caused by a high number of UEs returning from a temporary local access of a hosting network to their home network in a very short period of time.

-  Whether and how to minimize the impact on the UE's communication e.g., to prevent user plane and control plane outages when returning to a home network together with other high number of UEs in a very short period of time, after terminating their temporary local access to a hosting network.

-  	Whether and how to stagger the return of UEs to their home network, so as to spread out registration attempts over time and keep the number of UEs attempting to register simultaneously within a manageable limit.

************************** END OF CHANGE ***********************