3GPP TSG-WG SA2 Meeting #149E e-meeting 	S2-2200774r17

Elbonia, February 14th – 25th, 2022	(revision of S2-220xxxx)

Source:	CATT, Huawei, Hisilicon, Nokia, OPPO, Futurewei?, Intel, Tencent?, Lenovo?, vivo?, Ericsson?, Samsung?, Interdigital Inc.,

Title:	New Key Issue: The differentiated QoS handling considering different importance of media units

Document for:	Approval

Agenda Item:	9.19

Work Item / Release:	FS_XRM / Rel-18

Abstract: Proposes a new key issue to study the differentiated QoS handling considering different importance of media units for WT#3.3.

1. Introduction

XR services are characterized by high data rate and low latency. XR services bring more chances or more quickly to the RAN congestion. If there is data congestion in the RAN, the XR media units can be dropped by the RAN. In this release, it is anticipated that 5GS QoS framework will be enhanced to support different QoS handling for different XR media unit. And there are different levels of media units in the same QoS Flow, e.g., slices/tiles in the same I/B/P frame, I/B/P frame, GOP, multi-layer sub-streams and media stream. The RAN can drop the least important media units to quickly recover from the congestion state. In such cases, In order to identify the different importance of the different media units in different levels, the 5GS QoS framework needs to get importance/priority information, identify and mark the packets of the media unit. The 5GS QoS framework also needs to consider different relationship for different level media units.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-60.

# * * * * First change (all texts new)* * * *

5.x	Key Issue #x – Differentiated PDU Group Handling

5.x.1	Description

XR/media services are characterized by high data rate and low latency. In this release, it is anticipated that the 5GS QoS framework will be enhanced to support different QoS handling for PDU Group. PDU Groups can carry different content, e.g., slices/tiles in an I/B/P frame, I/B/P frames, etc.

This key issue proposes to support differentiated QoS handling considering different importance of PDU Groups e.g., by treating differently packets (i.e., PDUs) belonging to less important PDU Group(s) to reduce the resource wasting. The key issue includes the following aspects:

-	Which types of PDU Group shall be supported for the differentiated handling?

-	How does the 5GS identify the PDUs of one PDU Group?

-	How does the 5GS determine the importance information for a given PDU Group and/or dependency information between different PDU Groups?

-	Which network entity needs the importance/dependency information associated with the PDU Group and how it receives it?

-	Whether and how to enhance the QoS model and policy control for the importance/dependency information associated with a given PDU Group?

-	Definition of the importance/dependency information enabling differentiated PDU Group handling

-

NOTE 1:		RAN related aspects will be coordinated with RAN WGs.

NOTE 2:	Aspects related to how PDU Groups may depend on each other will be coordinated with SA4.