3GPP TSG-WG SA2 Meeting #149E e-meeting	S2-2200801r02

Elbonia, February 14 – 25, 2022

Source:	China Telecom, vivo, Ericsson, Samsung

Title:	FS_eNA_ph3 KI for WT#3.7: NWDAF-assisted URSP

Document for:	Approval

Agenda Item:	9.23

Work Item / Release:	FS_eNA_ph3 / Rel-18

Abstract of the contribution: Proposal for a new key issue about NWDAF assisted URSP.

# Discussion

UE Route Selection Policy (URSP) is used by UE to determine how the traffic of a detected application can route, e.g. associated to an established PDU Session, establishment of a new PDU Session. The URSP may be pre-configured in the UE or may be provisioned to UE from PCF and it cannot be adjustable in time. However, the route selection descriptor of URSP can be variable according to time or location or other factors for the same application. With the analytics from NWDAF, it would help PCF to select the best route (e.g. 3GPP or non-3GPP Access Type, S-NSSAI value, DNN value…) for the application(s) to get better resource utilization for the network or better experience for specific UE.

In TR 23.700-91, Solution#28 provides an NWDAF solution related to URSP. That solution is to solve how NWDAF verifies the 5GC delivered URSP is performed by UE or not. Therefore, none of R17 and previous solution discuss about how the NWDAF assists PCF to update the component content and priority of URSP rules/RSD in real time to make them applicable to the current use of applications by UE and current network conditions.

Here are several examples that the NWDAF may help in URSP decisions:

- When UE always uses App-A at certain time and place, PCF can leverage the analytics of UE’s behaviour from NWDAF to update the Time Window and Location Criteria part in the RSD for App-A in the URSP with higher precedence.

- When multiple existing PDU Sessions matching several RSD with different network slice, PCF can leverage the prediction of the slice load from NWDAF to deprioritize of URSP rules containing the slice with large load.

- When using different route for target application, e.g. 3GPP/non-3GPP/Prose, PCF can leverage the analytics of user experience from NWDAF to make decision on URSP.

- etc.

Considering some existing Analytics ID(s) can provide some information to PCF for making decisions in R17 TS23.288, such as "Service Experience", "WLAN performance", etc., which may be also helpful in URSP decisions, therefore, existing analytics and/or new analytics (including input and output) from NWDAF to assist PCF on URSP decisions need to be discussed in this key issue.

This new key issue is to address the WT#3.7: NWDAF-assisted URSP in SP-211650.

# Proposal

Proposal: It is proposed to add a new key issue about supporting UE route selection by adjusting URSP based on assistance analytics information from NWDAF.

The following changes are proposed for TR 23.700-81.

* * * * Start of Changes * * * *

## 5.2.X	Key issue #X: < NWDAF-assisted URSP >

### 5.2.X.1	Description

This key issue is to address the WT#3.7: NWDAF-assisted URSP.

NWDAF can provide some network analytics to PCF for policy decisions described in clause 6.1.1.3 in TS 23.503 [x]. However, further investigation is required on whether existing Analytics IDs or new analytics ID(s) can be used by PCF to assist for URSP decision. Furthermore, whether such analytics-assisted URSP rules require a new (set of) interaction(s) between NWDAF and PCF. Finally, if a new analytics ID is required, what are input (and output) data to (and from) the NWDAF, respectively.

In this key issue, the following aspects need to be studied:

- Which and how existing Analytics IDs, or new Analytics ID(s) can be used to assist PCF on URSP decisions.

- What procedures trigger the subscription to these Analytics IDs by the PCF.

- Whether a new (set of) interactions(s) are required between NWDAF and PCF (incl. triggers to update UE policy).

- What information should be collected (or provided) as input (or output) by the NWDAF for the new analytics ID(s), (if a new Analytics ID is required).

* * * * End of Changes * * * *