SA WG2 Meeting #S2-149E	S2-2200844r01

# E-meeting, February 14th – 25th, 2022

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3234 | rev | 1 | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | BSF service update for session binding information update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17,5GS_Ph1 |  |  |  |  |  | Date: |  |  | 2022-01-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | PCF will update the session binding information in BSF, but now in TS23.502/TS23.503, the BSF will not update the session binding information update in AF. In SA2#148 emeeting, one comment is that :”whether SMF will report the MAC @ added to the PDU Session and the PCF will store it in the BSF as associated to this UE MAC address.” After check the R15 disucssion, we find some papers to verify our proposal. 1) One discussion paper S2-182145 which said: “For Ethernet PDU Session, the UE may operate in bridge mode with regard to a LAN it is connecting to the 5GS, thus different MAC addresses may be used as source address of different frames sent UL over a single PDU Session (and destination MAC address of different frames sent DL over the same PDU Session). Hence, the PCF needs to know the UE MAC address change (allocated or released for the UE) to bind the different MAC address to the same PDU Session.” 2) one pCR S2-182889(Agreed) corresponding with the above discussion paper and also in TS 23.503 6.1.3.5: “The UE MAC address change trigger shall trigger a SMF interaction with the PCF if a new UE MAC address is detected or a used UE MAC address is inactive for a specific period during the lifetime of the Ethernet type PDU Session. The SMF reports that the UE MAC address change trigger was met and the new or released UE MAC address.” 3) one pCR S2-183021(Agreed) and also in TS 23.503 6.1.1.2.2: “For a PDU Session, the PCF ensures that it is updated each time an IP address is allocated or de-allocated to the PDU Session or, for Ethernet PDU Sessions supporting binding of AF request based on MAC address, each time it has been detected that a MAC address is used or no more used by the UE in the PDU Session.” All the above documents and specification can verify that, for ethernet PDU session, when a UE operate in bridge mode, different MAC addresses may associated with one ethernet PDU session, and SMF will report to PCF and PCF will update BSF. Case 1: AF may request the BSF about the PCF and related session binding information, and then AF does not communicate with the PCF immediately. So it is needed that when the PDU session related session binding information update in BSF (e.g. new UE mac address added in this PDU session), the BSF should notify this update to AF for the PDU session(which is identified by UE MAC address). Case 2: there are multiple devices behind the UE, and the devices may move between UEs, so the PCF may change for the specific device address,and such update should be notified to AF through BSF. Case3: There is no need for AF finding every BSF for MAC address session binding, if the BSF can notify the session binding information update for the new MAC addresses. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding BSF notify AF with session binding information update |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | AF can not get the correct time session binding information. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.13.2.6;5.2.13.2.8; |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<<< Start of changes >>>

##### 5.2.13.2.6	Nbsf_Management_Susbcribe service operation

Service Operation name: Nbsf_Management Susbcribe

Description: NEF or AF or PCF for a UE can subscribe to be notified of newly registered or deregistered PCF for a PDU Session. NEF or AF can subscribe to be notified of UE MAC address (es) change for the PDU Session. In addition, NEF or AF can subscribe to be notified of newly registered or deregistered PCF for a UE.

NOTE 1:	If BSF has already the requested information at the time of the subscription, it will accept the subscription request, and will immediately provide the results in the Outputs parameters.

Inputs, Required: SUPI, DNN(s) [Required, if PCF subscription is for a PDU Session], S-NSSAI(s) [Required, if PCF subscription is for a PDU Session], callback URI.

Inputs, Optional: GPSI, indication of registration/deregistration per (DNN, S-NSSAI).

Indication of registration/deregistration per (DNN, S-NSSAI) indicates to the BSF to report when the first SM policy association is established and when the last SM policy association is terminated to the same (DNN, S-NSSAI) combination.

NOTE 2:	It is up to stage3 to ensure an unambiguous error proof way for the BSF to differentiate between PCF for a PDU Session and PCF for a UE. This may or may not require providing the BSF additional input parameter(s).

Outputs, Required: When the subscription is accepted: Subscription Correlation ID.

Outputs, Conditional: One or more instance per (DNN, S-NSSAI) of (UE address(es) [If available], PCF address(es) [If available], PCF instance ID [If available], PCF Set ID [If available], level of Binding [If available] (see clause 6.3.1.0 of TS 23.501 [2])) or notification of registration/deregistration per (DNN, S-NSSAI).

##### NOTE 3:	The parameter UE address(es) is not applicable in the case of PCF for a UE.

<<< Second of changes >>>

##### 5.2.13.2.8	Nbsf_Management_Notify service operation

Service Operation name: Nbsf_Management Notify

Description: BSF can notify NEF or AF or PCF for a UE of newly registered PCF for a PDU Session or of deregistered PCF for a PDU Session, or UE MAC address(es) for the PDU session.

Inputs, Required: Notification Correlation Information, One or more instance per (DNN, S-NSSAI) of (UE address(es) [Required, if PCF notification is for a PDU Session], PCF address(es), PCF instance ID [Conditional, if available] and PCF Set ID [Conditional, if available]), level of Binding [Conditional, if available] (see clause 6.3.1.0 of TS 23.501 [2]) or notification of registration/deregistration per (DNN, S-NSSAI), UE MAC address(es) for the PDU session[ Required, if notification is for UE MAC address(es) change].

# Inputs, Optional: None.

# Outputs, Required: None.

# Outputs, Optional: None.

<<<End of changes >>>