3GPP TSG-WG SA2 Meeting #149E e-meeting  	S2-2200849

# Elbonia, February 14 – 25, 2022	(revision of S2-2105938)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0425 | rev | 1 | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of Data collection from OAM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2022-01-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In LS S2-2106634 SA2 asks SA5 whether OAM can provide performance measurements including the mean number of registered UEs and number of established PDU sessions of the network slice instance according to not only S-NSSAI but also S-NSSAI and NSI ID. In the Reply LS (S2-2200075/ S5-216416), SA5 says: The association between an S-NSSAI and an NSI ID is maintained in the NRM specified in TS 28.541. A short explanation is as follows: the attribute networkSliceInfo holds the association between an S-NSSAI, and a core network part identified by the cNSIId (which holds the value of the NSI ID) both belonging to the same NetworkSlice instance. The performance measurements for a NetworkSlice instance can be calculated by aggregation of the measurements correlated with each cNSIId in the cNSIIdList. As for the latest status of network slice information, the SA5 says: The model for the NWDAFFunction in TS 28.541 holds the parameter networkSliceInfoList, as described in Answer 1 the attribute networkSliceInfo includes the associated cNSIId (which holds the value of the NSI ID). Base on the above, a new configuration named networkSliceInfoList is defined for the NWDAF which is authorized to collect management data of network slices. The networkSliceInfoList includes a list of network slice information, i.e. networkSliceInfo, which includes S-NSSAI, optional NSI ID, and the corresponding DN of the NetworkSlice instance (i.e. the identifier of the managed object for the network slice instance). Through the networkSliceInfo, the NWDAF can get the identifier of the managed object by S-NSSAI and optional NSI ID. Then the NWDAF can consume the management services and collect data related to the network slice instance from OAM. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the description of the data collection about slice from the OAM. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The description of data collection from the OAM does not align with the specification from SA5. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.1, 6.2.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.2.3.1	General

The NWDAF may collect relevant management data from the services in the OAM as configured by the PLMN operator.

‐	NG RAN or 5GC performance measurements as defined in TS 28.552 [8].

‐	5G End to end KPIs as defined in TS 28.554 [10].

NWDAF shall use the following services to have access to the information provided by OAM:

-	Generic performance assurance and fault supervision management services as defined in TS 28.532 [6].

‐	PM (Performance Management) services as defined in TS 28.550 [7].

‐	FS (Fault Supervision) services defined in TS 28.545 [9].

Editor's note:	The MDA services are to be defined by SA WG5 and NWDAF could subscribe to them. The reference to SA WG5 specification will be added when defined in SA WG5.

NWDAF can be configured to invoke the existing OAM services to retrieve the management data that are relevant for analytics generation, which may include NF resources usage information (e.g. usage of virtual resources assigned to NF) and NF resource configuration information (e.g. life cycle changes of NF resource configurations).

OAM perform the required configuration in order to provide the information requested by NWDAF subscription and perform the tasks, e.g. data collection, data processing, associated with the subscribed request from NWDAF.

Another usage of OAM services is when the target of data collection is a specific UE, via MDT based retrieval of information:

-	Measurement collection for MDT as defined in TS 37.320 [20].

In addition, NWDAF can be provisioned with Network Slice information (i.e. as defined by the NetworkSliceInfo in the TS 28.541[22]) when a slice is created or modified via OAM configuration mechanism as defined in TS 28.541 [22] and TS 28.532 [6].

# * * * * Next change * * * *

6.2.3.2	Procedure for data collection from OAM

The interactions between NWDAF and OAM for data collection are illustrated in Figure 6.2.3.2-1. The data collected depends on the use cases. This figure is an abstraction of the OAM performance data file report management service that is defined TS 28.532 [6]. The actual OAM services and reporting mechanisms that NWDAF may use are specified in TS 28.532 [6], TS 28.550 [7] or TS 28.545 [9].

The flow below assumes the NWDAF is configured on how to subscribe to the relevant OAM services.

OAM shall setup the required mechanisms to guarantee the continuous data collection requested by NWDAF.

Figure 6.2.3.2-1: Data collection from OAM performance data file report management service

1.	(Clause 11.3.1.1.3.2 of TS 28.532 [6]), Subscribe (Input): NWDAF subscribes to the notification(s) related to the services provided by the management service producer.

2.	(Clause 11.3.1.1.3.3 of TS 28.532 [6]), Subscribe (Output): management service producer responses to NWDAF if the subscription is success or not.

3.	Data processing: management service producer prepares the data.

4.	(Clause 11.3.1.1.1 of TS 28.532 [6]), Notification (notifyFileReady): management service producer notifies the data file is ready.

As the final step, NWDAF fetches data by using FTP (not specified in 3GPP, based on vendor implementation).

NOTE 1:	The call flow in Figure 6.2.3.2-1 only shows a subscribe/notify model for the simplicity, however both request-response and subscription-notification models are supported.

NOTE 2:	NWDAF is configured with the Network Slice information (i.e. NetworkSliceInfo including a DN (Distinguished Name) of the NetworkSlice managed object relating to the the network slice instance associated to the S-NSSAI and NSI ID if available as defined in TS 28.541 [22]). Based on the Network Slice information, the NWDAF uses the managed object identifier to consume the management services and access the management data relating to the S-NSSAI and NSI ID (including the NRF serving the network slice, the NFs associated to the network slice, the NG RAN or 5GC performance measurements defined in TS 28.552 [8], or the 5G end to end KPIs defined in TS 28.554 [10]) provided by OAM.

# * * * * End of changes * * * *