3GPP TSG-SA WG2 Meeting #149E e-meeting 	S2-2200886r02

Elbonia, February 14 – 25, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon, Ericsson

Title:	New Key issue for WTI#7: support of NSAC involved multi NSACF

Document for:	Approval

Agenda Item:	9.14

Work Item / Release:	FS_eNS_Ph3 / Rel-18

Abstract: This document adds the key issue for WT#7.

- Discussion

One of the ENS3 WT#7 is to support the multi NSACF, i.e.

“7. Study whether and how to enhance the support of NSAC when more than one NSACF is involved in enforcing a shared maximum allowed number of the UEs or PDU Sessions for a network slice in one PLMN or in roaming, in order to avoid fragmentation of the shared maximum allowed number.”

This paper add the related key issue description.

- Proposal

It is proposed to capture the following changes vs. TS 23.700-41.

PS, there are ongoing CR discussed in Rel-17 under the eNS2 WID. If this issue is resolved at Rel-17, the solution to this key issue is not needed at Rel-18.

FIRST CHANGE

5.X	Key Issue #X: Support of NSAC involving multi service Area

5.X.1	Description

For one S-NSSAI, there is only one configured gloabl Maximum allowed number value for NSAC. It is possible to split a PLMN into multi-service areas. This impacts the use cases below as there will be more than one NSACF handling the UE:

- Multi NSACF deployed within one PLMN: For NSACF deployment more than one service area are defined within one PLMN. For each service area one NSACF or NSACF set is selected for slice admission control. This include the control of the maximum allowed number of UE or PDU session.

- Roaming: when one user resides at the visit PLMN, the NSAC (Maximum PDU session) may be controlled by the NSACF in the VPLMN (e.g. for LBO PDU session), or the NSACF in the HPLMN (e.g. for HR PDU session).

- EPS interworking: when the user establishes a HR PDN connection at the EPS network and move to 5GS later, the NSACF(Maximum UE number) selected by the SMF+PGW-C and AMF may be different.

This key issue addresses the above cases, to ensure consistent NSAC handling based on the global count against the configured global Maximum aallowed number. The following aspects will cover by the key issue:

- UE Registration

- PDU Session establishment

- Session continuity when UE move across the service area

End of CHANGE