3GPP TSG-WG SA2 Meeting #149E e-meeting  	S2-2200924r02

# Elbonia, February 14 – 25, 2022	(revision of S2-220xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0079 | rev | - | Current version: | 17.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of RAN Sharing for L2 Relay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe |  |  |  |  |  | Date: |  |  | 2022-01-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to the RAN2 LS in S2-2201810, RAN2 will have basic support of RAN sharing for L2 relay in Rel-17. And RAN2 has achieved the following agreements that have an impact to discovery message: - cellAccessRelatedInfo from SIB1 is forwarded before PC5-RRC connection using discovery message irrespective whether there is RAN sharing or not. - Carry cellAccessRelatedInfo from SIB1 in discovery message using RRC container (will be specified in RRC specification). - For which discovery message to use to carry cellAccessRelatedInfo, rely on SA2 to decide which discovery message to use. So this paper proposes to use Announcement message (Model A) or Response message (Model B) to carry the RRC container. Since the RRC container includes the cell identity, the NCGI is not required to be included separately in the discovery message. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add RRC container into the discovery message to align with RAN2 agreement. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incompleted functionality for RAN sharing support. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.2, 5.8.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.4.2	5G ProSe Layer-2 UE-to-Network Relay

The 5G ProSe Layer-2 UE-to-Network Relay provides forwarding functionality that can relay any type of traffic over the PC5 link.

The 5G ProSe Layer-2 UE-to-Network Relay provides the functionality to support connectivity to the 5GS for 5G ProSe Layer-2 Remote UEs. A UE is considered to be a 5G ProSe Layer-2 Remote UE if it has successfully established a PC5 link to the 5G ProSe Layer-2 UE-to-Network Relay. A 5G ProSe Layer-2 Remote UE can be located within NG-RAN coverage or outside of NG-RAN coverage.

For 5G ProSe UE-to-Network Relay Discovery, the standalone discovery is used, and both Model A and Model B are supported.

For PLMN selection and relay selection in the 5G ProSe Layer-2 Remote UE:

-	The 5G ProSe Layer-2 Remote UE checks whether the PLMN(s) within the RRC container (as defined in TS 38.331 [16]) obtained from the 5G ProSe Layer-2 UE-to-Network Relay(s) during 5G ProSe UE-to-Network Relay Discovery in clause 6.3.2.3 are authorized to be connected to via a 5G ProSe Layer-2 UE-to-Network Relay(s), and only the authorized PLMN(s) are then available PLMNs for NAS PLMN selection;

-	The 5G ProSe Layer-2 Remote UE selects the 5G ProSe Layer-2 UE-to-Network Relay considering the selected PLMN by NAS layer.

# * * * * Second change * * * *

5.8.3.3	Identifiers for 5G ProSe Layer-2 UE-to-Network Relay

The following parameters may be used in Announcement message (Model A) or Response message (Model B) in addition to the parameters as specified in clause 5.8.3.1 for 5G ProSe Layer-2 UE-to-Network Relay (re)selection:

-	NCGI: indicates the NCGI of the serving cell of the 5G ProSe Layer-2 UE-to-Network Relay.

-	RRC container as defined in TS 38.331 [16].

# * * * * End of changes * * * *