3GPP TSG-WG SA2 Meeting #149E e-meeting 	S2-2200943r01

Elbonia, February 14th – 25th, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	Solution for Power Saving based on AMF awareness of coverage information

Document for:	Approval

Agenda Item:	9.24

Work Item / Release:	FS_5GSAT_Ph2 / Rel-18

Abstract: A solution for support of UE in discontinuous coverage is presented.

# 1. Introduction/Discussion

In Rel-18 FS_5GSAT_Ph2 study item, it is proposed to study the support of the UE in discontinuous coverage as shown below:

- WT#1: Discontinuous coverage

WT#1.1: Architectural enhancements to support discontinuous coverage for mobility enhancement (e.g. paging enhancement)

WT#1.2: Architectural enhancements considering prediction, awareness & notification of UE wake-up time, power saving optimizations.

The CN needs to be aware of the coverage information for UE in discontinuous coverage, e.g. within network coverage in time (t0, t1), outside the network coverage in time (t1, t2). Based on the coverage information of the UE, the AMF/MME can optimize the power saving mechanisms accordingly.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-28.

# * * * * First change (All new text) * * * *

## 6.X	Solution X: Power Saving based on AMF awareness of coverage information

### 6.X.1	Description

This solution resolves Key Issue #X about the power saving enhancements for UE in discontinuous coverage.

In order to make sure the UE is kept in power saving mode without any uplink MO signalling request, this solution enhances the AMF to derive power saving parameters, e.g. eDRX parameters, periodic registration timer and the active time for MICO mode, based on the coverage information. Then only when UE is within the network coverage, the periodic registration update request would be initiated or the UE would wake up to monitor the paging occasion.

As the discontinuous coverage information it taken into account when determining the parameters for eDRX, MICO mode the existing procedures for reachability (including exposure), MT data buffering, etc can be reused.

### 6.X.2	Procedures

Before moving the UE to CM-IDLE state, the NG-RAN or AMF may decide to trigger the AN release procedure based on the coverage information.

Editor’s Note: Whether it is RAN, the AMF or either that triggers AN/S1 release based on coverage information is FFS.

UE

RAN

AMF

SMF

1. trigger AN/S1 release based on coverage info

2. N2 UE Context Release Request

5. (power saving parameters)

4. determine power saving parameters

9. Nsmf_PDUSession_UpdateSMContext

3. trigger AN/S1 release based on coverage info

UCU Procedure

6. N2 UE Context Release Command

7. AN Connection Release

8. N2 UE Context Release Complete

Figure 6.X.2-1: high-level procedure for power saving enhancement

- If the RAN detects that the UE is about to be out of network coverage based on the coverage information, the (R)AN may trigger the AN release procedure to move UE into CM-IDLE state before entering the non-coverage area.

- The RAN may send an N2 UE Context Release Request message to the AMF.

- If the AMF detects that the UE is about to leave the current network coverage based on the coverage information, the AMF may trigger the AN release procedure to move UE into CM-IDLE state when the UE is still within the network coverage.

- AMF may determine the power saving parameters for the UE based on the discontinuous coverage information. The power saving parameters can be periodic registration timer, active time for MICO mode and the eDRX parameters. This is to make sure the UE is kept in power saving mode without waking up or any uplink signalling request when there is no network coverage.

- AMF may trigger the UE Configuration Update procedure to update the power saving parameters.

- The AMF sends an N2 UE Context Release Command to the (R)AN.

- The RAN requests the UE to release the (R)AN connection. Upon receiving (R)AN connection release confirmation from the UE, the (R)AN deletes the UE's context.

- The (R)AN confirms the N2 Release by returning an N2 UE Context Release Complete (List of PDU Session ID(s) with active N3 user plane, User Location Information, Age of Location Information) message to the AMF.

- [Conditional] AMF to SMF: For each of the PDU Sessions in the N2 UE Context Release Complete, the AMF invokes Nsmf_PDUSession_UpdateSMContext Request in order to release N3 resources as defined in Clause 4.2.6 of TS 23.502 [x].

Editor’s Note: The impact of the UE waking up to send MO traffic or signalling in the MICO/PSM sleep cycles and how to mitigate that is FFS.

### 6.X.3	Impacts on services, entities and interfaces

RAN:

-	trigger AN release based on the coverage information for the UE.

AMF:

-	trigger AN release based on the coverage information for the UE.

-	configure the power saving parameters based on the coverage information for the UE.

# * * * * End of changes * * * *