3GPP TSG-SA2 Meeting #149-e	S2-2200958r09

# Online, , 14th Feb 2022 - 25th Feb 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0092 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MBS Frequency Selection Area Identifier |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai-Bell, Ericsson, Huawei, HiSilicon, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2022-01-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | At the last meeting a temporary name "MBS_XXX:ID" and only initial procedures were agreed. It was also contentious if the MBS_XXX_ID would be used to encode the service area. RAN 3 agreed that the MBX-XXX-ID is configured via OAM, and that the MBS service area is containing cell list and TAI list and is signalled during broadcast session establishment: Agreed R3-221166 #### 9.3.1.w MBS Service Area This IE contains the MBS service area. IE/Group Name; Presence; Range; IE type and reference; Semantics description. MBS Area Cell List; 0..<maxnoofCellsforMBS>. >>NR CGI; M; 9.3.1.7. MBS Area TAI List; 0..<maxnoofTAIforMBS>. >>TAI; M; 9.3.3.11. From RAN3 chair´s notes: Broadcast Session Management: For BC, establish the shared NG-U during Broadcast Session Setup Request/Response. Include Session ID, Area Session ID (optional), Service Area information, MB-SMF container: MBS Session Information Request Transfer in the BROADCAST SESSION SETUP message and MBS Session Information Modify Request Transfer in the MODIFICATION REQUEST message. MBS xxx ID/SAI is used to identify a preconfigured MBS area and it could be provided to RAN node via OAM. Name the MBS xxx ID/SAI as SAI in the interim before SA2 gives final name. As to the length of SAI, it depends on the decision of SA2.RAN3 send LS to SA2 on the conclusion in RAN3 Exchange the list of SAI that neighbor cells supported via Xn interface. In case of CU/DU split, SAIs of each cell should be configured in DU by OAM and be provided from DU to CU. It is thus clear that the MBS XXX ID is not directly related to the MBS service area, and to avoid confusion a name avoiding "MBS service area" seems preferable. In addition, the MBS XXX ID is not yet shown in many parts of the specifation (e.g. call flows and service operations). |
| Summary of change: |  | The MBS XXX ID is renamed to "MBS Frequency Selection Area (FSA) ID" and the documentation of its usage is completed by adding the FSA ID in call flows and service operations. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unresolved issues for the MBS XXX_ID remain and prevent the usage of the MBS broadcast service. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 6.5.4, 6.9.1, 6.11, 7.1.1.2, 7.3.1, 9.1.3.6, 9.1.3.7, 9.3.2.2, 9.3.2.3, 9.4.3.2, 9.4.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], TS 23.501 [5] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

FEC	Forward Error Correction

CDN	Content Delivery Network

FEC	Forward Error Correction

FSA	Frequency Selection Area

LL SSM	Lower Layer SSM

MBMS	Multimedia Broadcast/Multicast Service

MBS	Multicast/Broadcast Service.

MB-SMF	Multicast/Broadcast Session Management Function.

MBSF	Multicast/Broadcast Service Function.

MBSTF	Multicast/Broadcast Service Transport Function.

MB-SMF	Multicast/Broadcast Session Management Function.

MB-UPF	Multicast/Broadcast User Plane Function

PTM	Point To Multipoint

PTP	Point To Point

SSM	Source Specific IP Multicast address.

LL SSM	Lower Layer SSM

TMGI	Temporary Mobile Group Identity

2nd change

### 6.5.4	MBS  Frequency Selection Area ID

The MBS Frequency Selection Area (FSA) ID is used for broadcast MBS session to guide the frequency selection of the UE.

MBS FSA ID identifies a preconfigured area within, and in proximity to, which the cell(s) announces the MBS FSA ID and the associating frequency (details see TS 38.300 [9]). MBS FSA ID and their mapping to frequencies are provided to RAN nodes via OAM.

Based on this configuration, RAN nodes announce in SIBs over the radio interface information about the MBSFSA IDs and frequencies.

When a broadcast MBS session is created, the AF may provide MBS FSA ID(s) based on the business agreement. If the AF does not provide MBS FSA ID(s), the MB-SMF determines MBS FSA ID(s) based on configured mapping from MBS service area and/or broadcast MBS session information (e.g., application ID) to MBS FSA ID(s) and sends the determined MBS FSA ID(s), to the AF (optionally via NEF).

NOTE 1:	The same MBS FSA ID(s) can be assigned to multiple broadcast sessions.

The MBS FSA ID(s) of a broadcast MBS session are communicated in the service announcement towards the UE. The UE compares those MBS FSA IDs(s) with the MBS FSA ID(s) in SIBs for frequency selection.

During MBS Session Start for Broadcast in clause 7.3.1 and MBS Session Update for Broadcast in clause 7.3.3, the MB-SMF may include the MBS FSA ID(s) for the MBS session and send them to the NG-RAN nodes via the AMF. The NG-RAN nodes may then use those MBS FSA ID(s) to determine cells/frequencies within the MBS serice area to broadcast MBS session data. Details see TS 38.300 [9] and TS 38.413 [15].

3rd change

### 6.9.1	MBS Session Context

The MBS Session Context contains all information describing a particular MBS session in the 5GS and is created in each node involved in the delivery of the MBS data.

The content of the Multicast MBS Session Context is described in Table 6.9.1-1.

Table 6.9.1-1: Multicast MBS Session context

| Parameter | Description | NG-RAN | AMF | SMF | MB-SMF |
| --- | --- | --- | --- | --- | --- |
| State | State of MBS session ('Active' or 'Inactive' or 'Configured') | X (note 2) |  | X (note 2) | X |
| SSM (source specific IP multicast address) | IP multicast address identifying the MBS session. |  |  | X (note 1) | X (note 1) |
| TMGI | Temporary Mobile Group Identity allocated to the MBS Session. | X | X | x | X |
| Area Session Identifier | Used for MBS session with location dependent content. When present, the Area Session Identifier together with the TMGI uniquely identify the MBS Session in a specific MBS service area. | X (note 1) | X | X (note 1) | X (note 1) |
| MB-SMF | The MB-SMF that handles the MBS session. | X | X | X |  |
| QoS information | QoS information of the MBS session. | X |  | X | X |
| MBS Service Area | Area over which the MBS session data is distributed (i.e. Cell ID list or TAI list). | X (note 1) |  | X (note 1) | X (note 1) |
| NG-RAN Node ID(s) | NG-RAN nodes which are involved in the multicast session |  | X |  |  |
| AMF | The AMF(s) which are selected for the MBS session | X |  |  | X |
| IP multicast and source address for data distribution | IP addresses identifying the SSM user plane transport for shared delivery between MB-UPF and NG-RAN and for individual delivery between MB-UPF and UPF when the IP multicast transport is used. | X (note 1) |  | X (note 1) | X (note 1) |
| SMF | The SMF(s) that manages the associated PDU session. |  |  |  | X |
| UE ID | ID identifying the UE that successfully join the Multicast MBS Session. For NG-RAN it is NGAP UE ID and for SMF it is SUPI. | X (note 3) |  | X (note 3) |  |
| IP address for distribution | The IP addresses and TEID of NG-RAN used for the user plane between NG-RAN and MB-UPF and between MB-UPF and UPF when Point to Point tunnel is used. | X (note 1) |  | X (note 1) | X (note 1) |
| TEID for data distribution | The tunnel ID used for receiving the multicast data for shared delivery by NG-RAN and for individual delivery by UPF | X |  | X | X |
| PCF | The MB-PCF that provides policy control for the MBS session. |  |  |  | X (note 1) |
| NOTE 1: It is an optional parameter. NOTE 2: The value 'Configured' is not applicable for NG-RAN and SMF. NOTE 3: the UE ID is available within the UE Context which contains the MBS information. |  |  |  |  |  |

In Broadcast MBMS mode, an MBS Session Context is created in the NG-RAN, AMF, MB-SMF and MBSF as a result of the MBS Session Start procedure.

The content of the Broadcast MBS Session Context is described in Table 6.9.1-2.

Table 6.9.1-2: Broadcast MBS Session context

| Parameter | Description | NG-RAN | AMF | MB-SMF |
| --- | --- | --- | --- | --- |
| TMGI | Temporary Mobile Group Identity allocated to the MBS Session. | X | X | X |
| Area Session Identifier | Used for MBS session with location dependent content. When present, the Area Session Identifier together with the TMGI uniquely identify the MBS Session in a specific MBS service area. | X (note 1) | X (note 1) | X (note 1) |
| AMF | The AMF(s) which are selected for the MBS session | X |  | X |
| QoS information | QoS information for the MBS Session, including the QoS parameters of QoS flows. | X |  | X |
| MBS Service Area | Area over which the MBS session data is distributed (i.e. Cell ID list or TAI list). | X | X | X |
| NG-RAN Node ID(s) | NG-RAN nodes which are selected for the broadcast session |  | X |  |
| IP multicast address for data distribution | IP addresses identifying the user plane transport used for shared delivery between MB-UPF and NG-RAN when the IP multicast transport is used. | X (note 1) |  | X (note 1) |
| NG-RAN IP Address for data distribution | The IP address of NG-RAN used for the user plane between NG-RAN and MB-UPF when Point to Point tunnel is used. | X (note 1) |  | X (note 1) |
| TEID for data distribution | The tunnel ID used for receiving the broadcast data for shared delivery by NG-RAN | X |  | X |
| PCF | The PCF that provides policy control for the MBS session. |  |  | X (note 1) |
| MBS FSA ID | MBS Frequency Selection Area (FSA) ID is used for broadcast MBS sessions to guide the frequency selection of the UE | X |  | X |
| NOTE 1: It is an optional parameter. |  |  |  |  |

4th change

## 6.11	Service Announcement

Service Announcement provides the UE with descriptions specifying the multicast or broadcast services to be delivered as part of MBS Session.

The Service Announcement includes the MBS Session ID(s), which is represented by TMGI or a Source Specific IP Multicast Address, for the service. When the MBS Session ID is Source Specific IP Multicast Address, the Service Announcement may include the PLMN ID of the PLMN and NID for an SNPN in which the service is delivered.

The Service Announcement includes an MBS Service Type, which indicates whether the MBS Session for the service is multicast or broadcast.

NOTE 1:	A Source Specific IP Multicast Address as MBS Session ID indicates a multicast session.

For local MBS service, the Service Announcement may include the MBS service area. The MBS service area used by AF can be Cell ID list, TAI list, geographical area information or civic address information. Amongst them, Cell ID list and TAI list shall only be used by AFs who reside in trust domain, and when the AFs are aware of such information.

If the MBS Session is multicast, the Service Announcement may include the DNN and S-NSSAI of the PDU Session to indicate which PDU Session is associated with the MBS Session.

NOTE 2:	For multicast, AF or MBSF provides Service Announcement only after the MBS information is available to 5GC or the start time need be included, to avoid potential rejection sent by SMF of the MBS session join request.

NOTE 3:	The MBS Service related information, e.g. default PLMN ID, DNN and S-NSSAI can be pre-configured in the UE.

NOTE 4:	If DNN and S-NSSAI information is not provided in the service announcement or pre-configured, how UE determines the PDU session to join the MBS Session is implementation specific.

If the MBS Session is broadcast, the Service Announcement may include the MBS FSA ID(s) and/or frequency information associated with the broadcast MBS session.

The Service Announcement may be provided to a UE by AF or MBSF, or may be retrieved by the UE from those entities.

NOTE 5:	How the UE can get the Service Announcement from other entities is not specified.

5th change

#### 7.1.1.2	MBS Session Creation without PCC

This procedure is used by the AF to start the MBS Session towards 5GC and consist of TMGI allocation, and MBS session creation, and they apply to both multicast and broadcast communications unless otherwise stated.

For multicast, MBS session establishment procedure triggered by UE join requests may follow the MBS session creation procedure to reserve resources towards NG-RAN. For broadcast, the MBS session start procedure to reserve resources towards NG-RAN is triggered by the MBS session creation procedure.

For both broadcast and multicast communication, the TMGI allocation may be separated from the MBS Session creation request.

For multicast communication, TMGI allocation procedure is applicable if TMGI is used as MBS Session ID.

Figure 7.1.1.2-1: MBS Session Creation without PCC

Steps 1 to 6 are optional and only applicable if TMGI is used as MBS Session ID and required to be pre-allocated.

1.	AF sends Nnef_TMGI_Allocate Request (TMGI number) message to NEF/MBSF to request allocation of a TMGI(s) to identify new MBS session(s).

NOTE 1:	Depending on the network deployment and use case, MB-SMF may receive requests from AF directly, or via NEF, or via MBSF, or via NEF and MBSF.

2.	NEF checks authorization of AF.

NOTE 2:	NEF is not required if AF is in trusted domain.

3.	NEF/MBSF discovers and selects an MB-SMF using NRF or based on local configuration,

4.	NEF/MBSF sends an Nmbsmf_TMGI_Allocate Request (TMGI number) message to the MB-SMF.

5.	MB-SMF allocates TMGI(s) and returns the TMGI(s) to the NEF/MBSF via the Nmbsmf_TMGI_Allocate response (TMGI(s), expiration time).

6.	The NEF or MBSF responds to the AF by sending an Nnef_TMGI_Allocate Response (TMGI(s), expiration time).

7.	The AF may perform a Service Announcement towards UEs. The AF informs UEs about MBS Session information with MBS Session ID, e.g., TMGI, SSM, and possibly other information e.g., MBS service area, session description information, etc.

The MBS service area information can be Cell ID list, TAI list, geographical area information or civic address information. Amongst them, Cell ID list and TAI list shall only be used by AFs who reside in trust domain, and when the AFs are aware of such information.

The UE needs to be aware if the service is broadcast or multicast to decide if JOIN is to be performed.

Editor's note:	How to do service announcements requires SA WG4 /WG6 coordination.

8.	AF of content provider may provide description for an MBS session (possibly providing information for a previously allocated TMGI to NEF via a Nnef_MBSSession_Create request (([MBS Session ID], service type, MBS information, [TMGI allocation indication]). If step 1-6 has not been executed before, the AF may provide a SSM or it may request that the network allocates an identifier for the MBS session (i.e., TMGI). The AF provides the service type (i.e. either multicast service or broadcast service). MBS session information may further include QoS requirements and Any UE indication (indicating whether a multicast MBS session is "open to any UEs"), MBS service area, start and end time of the MBS session and MBS session state (active/inactive). In addition, MBS information may also indicate whether the allocation of an ingress transport address is requested.

If geographical area information or civic address information was provided by the AF as MBS service area, NEF/MBSF translates the MBS service area to Cell ID list or TAI list.

For broadcast communication, the AF may determine MBS FSA ID(s) for the broadcast session based on business agreements and include them in the description of the MBS session.

Editor's note:	What other information is to be sent by AF is FFS.

9	NEF/MBSF checks authorization of content provider.

10.	NEF/MBSF discovers MB-SMF candidates and selects MB-SMF as ingress control node, possibly based on MBS service area. If a TMGI is included in step 8, NEF/MBSF finds MB-SMF based on that TMGI.

11.	NEF/MBSF sends Nmbsmf_MBSSession_Create Request (MBS Session ID, service type, TMGI allocation indication, MBS service area information, ingress transport address request indication) to MB-SMF, to request MB-SMF to reserve ingress resources for a MBS distribution session, The NEF/MBSF provides MBS Session ID or request allocation of a TMGI, and indicate the requested service type (either multicast service or broadcast service) and MBS session state (active/inactive). It also indicates that the allocation of an ingress transport address is requested if this was requested in step 8, or if the MBSF decides to insert an MBSTF into the user plane for the MBS session. The request also includes the Any UE indication if provided in step 8.

The MBS service area is provided by NEF/MBSF to the MB-SMF if provided by the AF in step 8.

MBS FSA ID(s) are provided by NEF/MBSF to the MB-SMF if provided by the AF in step 8.

12.	If requested to do so, or if a source specific multicast is provided as MBS Session ID in step 11, the MB-SMF allocates a TMGI.

For broadcast communication, if no MBS FSA ID(s) have been received, the MB-SMF selects MBS FSA ID(s) for the broadcast session based on local configuration.

12.	If a source specific multicast is provided as MBS Session ID in step 11, the MB-SMF updates its NF profile at the NRF with the serving MBS Session ID. If an MBS service area information was received in step 11, the MB-SMF updates its NF profile at the NRF with that information.

NOTE 3:	If TMGI is used to represent an MBS Session, MB-SMF does not need to update NRF if the TMGI range(s) supported by an MB-SMF is already included in the MB-SMF profile when MB-SMF register itself into NRF.

13.	The MB-SMF derives the required QoS parameters locally.

14	MB-SMF selects the MB-UPF. If the allocation of an ingress transport address was requested in step 11, the MB-SMF requests the MB-UPF to reserve user plane ingress resources. If multicast transport of the MBS data towards RAN nodes is to be used, the MB-SMF also request the MB-UPF to reserve for the outgoing data a tunnel endpoint and the related identifiers (source IP address, SSM and GTP Tunnel ID) and to forward data received at the user plane ingress resource using that tunnel endpoint.

If the allocation of an ingress transport address was not requested in step 11, the MB-SMF provides the SSM received as MBS session ID to the MB-UPF and requests the MB-UPF to join the corresponding multicast tree from the content provider. The MB-SMF may also defer the configuration to join the corresponding multicast tree e.g. based on information that the session is inactive, QoS requirements and MBS start/end time until receiving the first query for the MBS session as part of the establishment procedure in clause 7.2.1.3, or until receiving a request to activate the MBS session via the MBS Session Update procedure in clause 7.1.1.6 or 7.1.1.7.

15.	If requested, MB-UPF selects an ingress address (IP address and port) and a tunnel endpoint for the outgoing data and provides it to MB-SMF.

16.	For broadcast communication, the MB-SMF continues the procedure towards the AMF and NG-RAN as specified in clause 7.3.1.

17.	MB-SMF indicates the possibly allocated ingress address to the NEF/MBSF. MB-SMF may include TMGI if it is allocated in step 11. For broadcast communication, the MB-SMF includes any MBS FSA ID(s) selected in step 11. It also indicates the success or failure of reserving transmission resources.

18.	[Optional] If the MBSF decides to use an MBSTF, the NEF/MBSF provides the ingress address received in step 14 towards the MBSTF as DL destination. If the allocation of an ingress transport address was requested in step 8, the MBSF requests the MBSTF to allocate the user plane ingress resources. If the allocation of an ingress transport address was not requested in step 8, the MBSF provides the SSM received as Multicast session ID in step 8 and requests the MBSTF to join the corresponding multicast tree from the content provider.

19.	[Conditional on step 19]	If requested, the MBSTF selects an ingress address (IP address and port) and provides it to NEF/MBSF.

20.	The NEF/MBSF-C indicates the possibly allocated ingress address and other parameters (e.g. TMGI) to the AF via an Nnef_MBSSession_Create response ([TMGI], [Allocated ingress address])). If MBS Session ID is not provided in step 8, or the MBS Session ID is SSM, the NEF/MBSF provides the allocated TMGI. If AF requested the allocation of an ingress transport address, the message also includes the allocated ingress address. For broadcast communication, the message also includes any MBS FSA ID(s) received in step 17.

21.	Same as step 7. The AF may also perform a service announcement at this stage.

22.	For multicast communication, depending on configuration UEs can join the MBS Session as specified in clause 7.2.1.

6th change

### 7.3.1	MBS Session Start for Broadcast

The Broadcast Session Start follows the common procedure specified in clause 7.1.1.2 or clause 7.1.1.3, which consist of TMGI Allocation and MBS Session Create. It is possible for AF to allocate TMGI once but create the MBS Session for multiple times. A combined procedure to perform both TMGI allocation and MBS Session Create is available.

The TMGI Allocation is used by AF to obtain the TMGI as MBS Session ID (i.e. TMGI) and perform service announcement towards UEs.

The MBS Session Create (with MBS service type set to broadcast service) is used by the AF to indicate the impending start of the transmission of MBS data, and to provide the session attributes, so that resources for the MBS Session are set up in the MB-UPF and in the NG-RAN for 5GC Shared MBS traffic delivery. The MBS Session Create can be used if TMGI has not been allocated. In this case, MB-SMF will allocate a unique TMGI for the AF and then start the MBS Session.

NOTE 1:	When the multicast transport between NG-RAN and MB-UPF is described below, source specific multicasting is assumed.

To receive the data of broadcast communication service, the UE is either preconfigured with needed configuration (e.g. USD as defined in TS 26.346 [13]) for the UE to receive MBS service, or provisioned with the configuration of broadcast session on application level (service announcement; the configuration may for instance be performed using SIP signalling, or methods described in TS 26.346 [13]). If the needed configuration is pre-configured, the UE does not need to interact with network.

Figure 7.3.1-1: MBS Session Establishment for Broadcast

0	Based on OAM configuration, RAN nodes announce in SIBs over the radio interface information about the MBS FSA IDs and frequencies of neigbouring cells.

1.	To establish broadcast MBS session, the AF performs TMGI allocation and MBS session creation as specified in clause 7.1.1.2 or 7.1.1.3. The MBS service type indicates to be broadcast service. The MBS FSA ID(s) of a broadcast MBS session are communicated in the service announcement towards the UE. The UE compares those MBS FSA IDs(s) with the MBS FSA ID(s) in SIBs for frequency selection.

2.	The MB-SMF may use NRF to discover the AMF(s) supporting MBS based on the MBS service area and select the appropriate one(s). Then the MB-SMF sends the Namf_MBSBroadcast_ContextCreate (TMGI, LL SSM, 5G QoS Profile, MBS service area, [MBS FSA ID(s)]) messages to the selected AMF(s) in parallel if the service type is broadcast service. The MB-SMF may include a maximum response time in the request.

3.	The AMF transfers the N2 message in the received Namf_MBSBroadcast_ContextCreate Request (TMGI, LL SSM, N2 SM information (5G QoS Profile), [MBS FSA ID(s)]) message to all NG-RANs which support MBS in the MBS service area. The AMF may include the MBS service area.

4.	NG-RAN creates a Broadcast MBS Session Context, stores the TMGI, the QoS Profile in the MBS Session Context. The LL SSM are optional parameters and only provided by MB-SMF to NG-RAN if N3mb multicast transport is configured to be used in the 5GC. If MBS FSA ID(s) were received, the NG-RAN may use those MBS FSA ID(s)s to determine cells/frequencies within the MBS serice area to broadcast MBS session data based on OAM configuration about the MBS FSA IDs and related frequencies.

5.	If NG-RAN prefers to use N3mb multicast transport (and if LL SSM is available in NG-RAN), the NG-RAN joins the multicast group (i.e. LL SSM).

If NG-RAN prefers to use N3mb point-to-point transport (or if the LL SSM is not available in NG-RAN) between the NG-RAN and MB-UPF, NG-RAN provides its N3mb DL Tunnel Info.

6.	The NG-RAN reports successful establishment of the MBS Session resources (which may include multiple MBS QoS Flows) by sending MBS Session Resource Setup Response (TMGI, N2 SM information (N3mb DL Tunnel Info)) message(s) to the AMF. N3mb DL Tunnel Info is only available when point-to-point transport applies between MB-UPF and NG-RAN. The NG-RAN reports in N2 SM container partially successful result, if not all MBS Session resources (i.e. all MBS QoS flows admitted) are established successfully in all requested cells.

7.	The AMF transfers the Namf_MBSBroadcast_ContextCreate Response () to the MB-SMF. The AMF should respond success when it receives the first success response from the NG-RAN(s). And if all NG-RAN(s) report failure, the AMF should respond failure. The MB-SMF store the AMF(s) which responds success in the MBS Session Context as the downstream nodes. If the AMF receives the NG-RAN response from all involved NG-RAN(s), the AMF should include an indication of completion of the operation in all NG-RANs.

8.	If N3mb point-to-point transport is to be used (i.e. N3mb DL Tunnel Info is present in the Namf_MBSBroadcast_ContextCreate Response message from AMF), the MB-SMF sends an N4mb Session Modification Request to the MB-UPF to allocate the N3mb point-to-point transport tunnel for a replicated MBS stream for the MBS Session. Otherwise, step 8 can be skipped.

9.	NG-RAN advertises the TMGI representing the MBS service over radio interface. Step 9 can take place in parallel with step 6.

10.	Another NG-RAN may report successful establishment of the MBS Session resources (which may include multiple MBS QoS Flows) by sending MBS Session Resource Setup Response (TMGI, N2 SM information (N3mb DL Tunnel Info)) message after the AMF transferred the Namf_MBSBroadcast_ContextCreate Response () to the MB-SMF.

11.	The AMF transfers the Namf_MBSBroadcast_ContextStatusNotify request () to the MB-SMF. When the AMF receives the response from all NG-RAN nodes, the AMF includes an indication of the completion of the operation. If the AMF does not receive responses from all NG-RAN nodes before the maximum response time elapses since the reception of the Namf_MBSBroadcast_ContextCreate Request, then the AMF should transfer the Namf_MBSBroadcast_ContextStatusNotify request () which indicates partial success or failure.

12.	If N3mb point-to-point transport is to be used (i.e. N3mb DL Tunnel Info is present in the MBS Session Start Response message from AMF), the MB-SMF sends an N4mb Session Modification Request to the MB-UPF to allocate the N3mb point-to-point transport tunnel for a replicated MBS stream for the MBS Session. Otherwise, step 12 can be skipped.

13.	The AF starts transmitting the DL media stream to MB-UPF using the N6mb Tunnel, or optionally un-tunnelled i.e. as an IP multicast stream using the HL MC address.

14.	The MB-UPF transmits the media stream to NG-RAN via N3mb multicast transport or point-to-point transport.

15.	The NG-RAN transmits the received DL media stream using DL PTM resources.

NOTE 2:	Step 6-8 and 2-4 are comparable to step 2-5 and 6-7 in clause 7.2.1.4, respectively.

7th change

#### 9.3.2.2	Namf_MBSBroadcast_ContextCreate service operation

Service operation name: Namf_MBSBroadcast_ContextCreate

Description: This service operation is used to create the broadcast session context towards the AMF.

Inputs, Required: MBS Session ID, Broadcast service area, N2 container (e.g. MBS Session ID, MBS QoS profile, Broadcast service area).

Inputs, Optional: Area Session ID, Optional parameters in the N2 container(MBS IP Multicast Tunnel Info, Area Session ID, MBS FSA ID(s)).

Outputs, Required: Result Indication.

Outputs, Optional: N2 container (NG-RAN MBS Tunnel Info).

8th change

#### 9.3.2.3	Namf_MBSBroadcast_ContextUpdate service operation

Service operation name: Namf_MBSBroadcast_ContextUpdate

Description: This service operation is used to update the broadcast session context towards the AMF.

Inputs, Required: MBS Session ID.

Inputs, Optional: Broadcast service area, N2 container (MBS Session ID, Possible MBS QoS profile, Possible Broadcast service area, Possible Area Session ID, Possible MBS IP Multicast Tunnel Info, MBS FSA ID(s)).

Outputs, Required: Result Indication.

Outputs, Optional: N2 container (MBS Session ID, NG-RAN MBS Tunnel Info).

9th change

#### 9.1.3.6	Nmbsmf_MBSSession_Create service operation

Service operation name: Nmbsmf_MBSSession_Create

Description: Create a new multicast session or broadcast session during MBS session creation. Optionally subscribe to notifications for this MBS session.

Input, Required: MBS Session ID (SSM or TMGI) or TMGI request, MBS Service Type (multicast or broadcast).

Input, Optional: DNN, S-NSSAI, MBS service area, MBS activation time, MBS termination time, service description, QoS flow information, Input Transport Address Request, session activity status (active/inactive). For a multicast session, indication that any UE may join. For a broadcast session, MBS FSA ID(s). For subscription to notifications event ID(s), Notification Target Address.

Output, Required: Result Indication.

Output, Optional: TMGI, Expiry Time of the TMGI, Cause, MB-UPF tunnel info, MBS FSA ID(s).

10th change

#### 9.1.3.7	Nmbsmf_MBSSession_Update service operation

Service operation name: Nmbsmf_MBSSession_Update

Description: Update the established multicast session or broadcast session, e.g. QoS update.

Input, Required: MBS Session ID.

Input, Optional: QoS flow information, MBS service area, session activity status (active/inactive), For a broadcast session, MBS FSA ID(s).

Output, Required: Result Indication.

Output, Optional: Cause, MBS FSA ID(s).

11th change

#### 9.4.3.2	Nnef_MBSSession_Create service operation

Service operation name: Nnef_MBSSession_Create

Description: Create a new multicast session or broadcast session. Optionally implicitly subscribe to notifications for this MBS session.

Input, Required: MBS Session ID (SSM or TMGI) or TMGI request, Service Type (broadcast or multicast).

Input, Optional: MBS service area (mandatory for broadcast. Can be Cell ID list and/or TAI list, or geographical area information, or civic address information), MBS start time, MBS stop time, service requirement (e.g. Flow Description, QoS), Input Transport Address Request, session activity status (active/inactive), for subscription to notifications notification target address. For a multicast session, indication that any UE may join the multicast session. An Application ID. For a broadcast session, MBS FSA ID(s).

Editor's note:	How multiple service requirements with multiple application IDs are handled is FFS.

Output, Required: Result Indication.

Output, Optional: TMGI, Cause, MB-UPF tunnel info, MBS FSA ID(s).

12th change

#### 9.4.3.3	Nnef_MBSSession_Update service operation

Service operation name: Nnef_MBSSession_Update

Description: This service is used by the NF service consumer to update the established multicast session or broadcast session, e.g. QoS update. For multicast MBS Session, this service is also used to activate or deactivate the MBS Session.

Input, Required: MBS Session ID.

Input, Optional: Service requirement, MBS service area, session activity status (active/inactive). For a broadcast session, MBS FSA ID(s).

Output, Required: Result Indication.

Output, Optional: Cause, MBS FSA ID(s).

End of changes