3GPP TSG-WG SA2 Meeting #149E e-meeting 	S2-2200965r05

Elbonia, February 14th – 25th, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon, Xiaomi, vivo

Title:	Key Issue: Support of authorisation, policy and parameter provisioning for ranging services

Document for:	Approval

Agenda Item:	9.20

Work Item / Release:	FS_Ranging_SL / Rel-18

Abstract: The SL ranging is an added capability on the existing SL capability. Just like the authorisation and provision for ProSe and V2X communication, in order to perform SL ranging, we need to study what parameters should be provided and how to provide them to UE.

1. Introduction

In SP-211647 for SA2 FS_Ranging_SL, 5GC will make enhancements to support Ranging-based services and sidelink positioning for commercial, V2X and public safety use cases in-coverage, in partial coverage, and out-of-coverage of the 5G network.

Figure 6.37.1-1 in TS 22.261: illustration of ranging between UEs with or without 5G coverage

When a SL ranging capability UE wants to participate in SL ranging, it needs to get the necessary configuration parameters. If the parameters are preconfigured in the ME during manufacture, the UE can work without provision of configuration parameters from the network; otherwise, the UE should access to the network first to get the configuration parameters. It should be studied whether a UE is authorised for SL ranging service, what parameters should be provided, and how to provide them to UE. This is corresponding to the Work Task 1.1:

- Work Task 1: Architecture enhancements to enable Ranging Service and sidelink positioning, which include:

- Work Task 1.1: authorization and policy/parameter provisioning for a UE or a group of UEs.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-86.

# * * * * First change (all new) * * * *

5.x	Key Issue #X: Support of authorisation and policy/parameter provisioning to UE

5.x.1	Description

A Ranging/SL positioning capable UE needs necessary configuration data to perform ranging operations, which contains the policies and/or parameters related to the service. Additionally, whether the service is permitted shall also be authorised.

RAN

UE

5GC

1. UE accesses to NW

2. Ranging service configuration parameters

RAN

UE

5GC

1. UE accesses to NW

2. Ranging service configuration parameters

In order to enable Ranging based service and Sidelink positioning over PC5, the following aspects related to authorisation and policy/parameter provisioningneed to be studied:

-	Study the principle, and identify the necessary and additional information for service authorisation and (pre-)configuration/provisioning to the UE based on what is specified in TS 23.287 [y] clause 5.1.2.1, and in TS 23.304 [x] clauses 5.1.1, 5.1.2.1, and 5.1.3.1.

-	Study the procedures related to service authorisation and policy/parameter (pre-)configuration/provisioning to a UE based on what is specified in TS 23.287 clause 6.2 and TS 23.304 clause 6.2.

NOTE X: Architectural refrence model as documented in clause 4.2 of TS 23.304 [x] and clause 4.2 of TS 23.287 [y] will be used as baseline for this key issue.

# * * * * End of changes * * * *