SA WG2 Meeting #149e	S2-2200969r04

# February 14th – 25th, 2022; Elbonia               		(revision of S2-220)

Source: 	China Mobile, China Telecom (?), vivo (?), Qualcomm Incorporated (?), Intel (?), Samsung (?), OPPO (?), Huawei?, HiSilicon?, Lenovo?, Motorola Mobility?

Title: 	FS_AIMLsys Key Issue: 5GC information exposure to the UE

Document for: 	Approval

Agenda Item: 	9.21

Work Item / Release:	FS_AIMLsys / Rel-18

Key Issue description is proposed for FS_AIMLsys in TR23.700-80.

# 1	Discussion

This contribution is related to part of the WT#1.1b (highlighted in cyan) in the FS_AIMLsys:

Objective 1 (WT#1.1): Study the possible architectural and functional extensions to support the Application layer AI/ML operations defined in TS 22.261, more specifically:

- Support monitoring of network resource utilization in the 5G system relevant to the UE in order to support Application AI/ML operation with AI/ML model provider.

- 5GS information exposure extensions for 5GC NF(s) to expose UE and/or network conditions and performance prediction (e.g. location, QoS, load, congestion, etc.) and whether and how to expose such information to the UE and/or to the authorized 3rd party to assist the Application AI/ML operation.

From the UE side, it may benefit from network analytics information which are exposed by 5GS, for example,

-	According to the prediction of network status, the UE can split the AI/ML model into several parts to upload in different network status. In the worst network status, the UE decides to buffer the AI/ML model and not to upload. Also, the UE can choose to upload the model in good air interface environment at mid night at home or, via other RAT for example, the WiFi access.

-	The UE can request the DN performance analytics to decide which application server to connect. Also, the UE can decide the power saving strategy according to the UE behaviour predictions and observed service experience analytics. The NWDAF can also provide the intelligent network selection strategy to assist the UE to select a proper access type, 3GPP or non-3GPP network.

A number of Analytics ID have already been defined in TS 23.288[x]. however, which existing or new analytics ID(s) is benefit to the UE should be identified and how does the UE get the analytics from 5GC needs to be studied.

.

# 2 Proposal

It is proposed to be adopted in TR23.700-80 for FS_AIMLsys topic.

# 5	Key Issues

## 5.X	Key Issue #X: 5GC information exposure to UE

### 5.x.1 	General Description

This contribution is related to WT#1.1b, which aims at studying the 5GC information exposure to UE.

As specified in SA1 TR 22.874 and TS 22.261, the application layer AI/ML model can be downloaded from AF/AS to UE to facilitate local intelligence, i.e., the UE can do inference and make decisions locally, for the use cases such as image recognition, speech recognition, media quality enhancement, etc. The UE may need the network analytics outputs from the network to assist its local decision.

This Key Issue is to study whether and how to enhance the 5GC to expose information to the UE as follows:

- Whether existing NF services are enough to serve the request of the UE, and how the services need to be enhanced.

-

- .

- What are the required information to be provided to UE to support the Application AI/ML operation

- Study how the 5GC exposes and how the UE consumes the information

-