SA WG2 Meeting #149E	S2-2201001r02

# February 14th – 25th, 2022 ; Elbonia                   		 	 				(revision of S2-220xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0081 | rev |  | Current version: | 17.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Security procedures for L3 relaying |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Interdigital Inc., LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe |  |  |  |  |  | Date: |  |  | 2022-01-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Security procedures for authentication of the 5G ProSe Remote UE and for authorisation of both 5G ProSe UE-to-Network Relay and the 5G ProSe Remote UE has been postponed to wait for SA3 progress. It was proposed earlier to specify detailed call flows for CP security solution in TS 23.304 but now that SA3 has already decided to specify CP and UP solution and the related call flows are already available in (draft) TS 33.503, it is more appropriate for SA2 TS to refer to the existing text in TS 33.503 that has already been adopted in TS 23.304 vocabulary. TS 33.503 clause 6.3.3.3 already specifies the call flow procedure details for Remote UE and Relay UE security procedures for Control Plane solution. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the main principles of CP security procedures for authentication and authorisation of 5G ProSe Layer-3 UE-to-Network relaying to so far empty clause 5.1.4.3. Being aware of SA3 work in this area, this clause is left a high level description that refers to TS 33.503 for further details. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 5G ProSe Layer-3 UE-to-Network relaying does not work as the security procedures are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 5.1.4.3	Principles for Network controlled authorisation for ProSe UE-to-Network Relay

##### 5.1.4.3.1	General

5G ProSe Remote UE data is transported unmodified over the 5G ProSe UE-to-Network Relay. If required, the 5G ProSe Remote UE data can be ciphered. The PLMN serving the 5G ProSe Remote UE can be the same PLMN or different from the one serving the 5G ProSe UE-to-Network Relay.

Prior to data relaying, both 5G ProSe Layer-3 Remote UE and 5G ProSe Layer-3 UE-to-Network Relay are authenticated and authorised for relay service as specified in clause 6.5.1. Both Control Plane and User Plane procedures are specified for 5G ProSe UE-to-Network relaying in TS 33.503 [29]. 5G ProSe Remote UE's authentication is done via 5G ProSe UE-to-Network Relay.

##### 5.1.4.3.2	Principles for control plane based authorisation for Layer-3 UE-to-Network Relay

Control plane-based authorisation for Layer-3 UE-to-Network Relay is characterised by the following principles:

-	Relay authorisation is pre-requisite to PDU Session establishment specified in clause 6.5.1.1.

-	Relay network is the PLMN serving the 5G ProSe UE-to-Network Relay and the Remote network is the 5G ProSe Remote UE's HPLMN. These networks can be the same PLMN or two different PLMNs.

-	NAS signalling relayed via the 5G ProSe UE-to-Network Relay is used for the Remote UE authentication and authorisation.

-	The AMF serving the 5G ProSe UE-to-Network Relay interacts with the Remote UE's AUSF.

-	During the 5G ProSe UE-to-Network Relay registration, the 5G ProSe UE-to-Network Relay receives information on whether it is authorised to be a 5G ProSe UE-to-Network Relay.

Editor's note: It is to be verified together with SA3 whether it is feasible that during the same procedure, the AMF gets information on which 5G Remote UEs may use this UE if the Remote UE belongs to the same PLMN or PLMNs whose UEs determined by the MCC and MNC may use this UE as a 5G ProSe UE-to-Network Relay if the Remote UE belongs to different PLMN.

-	During PC5 connection establishment, the 5G ProSe Remote UE provides its UE ID (SUCI) to the 5G ProSe UE-to-Network Relay which sends that info to the AMF to get authenticated and authorisation:

-	the 5G ProSe UE-to-Network Relay's AMF selects AUSF as specified in TS 23.501 [4] clause 6.3.4 and initiates authentication for the 5G ProSe Remote UE towards the 5G ProSe Remote UE's AUSF;

- 	the 5G Remote UE and the 5G ProSe UE-to-Network Relay perform PC5 connection establishment based on outcome of 5G ProSe Remote UE authentication and authorization via 5G ProSe UE-to-Network Relay's AMF.

-	After authentication and authorisation procedures have been completed, the 5G ProSe Remote UE may keep the PC5 link for further signalling.

For call flow details, see TS 33.503 [29] clause 6.3.3.3.