SA WG2 Meeting #149E	S2-2201001r06

# February 14th – 25th, 2022 ; Elbonia                   		 	 				(revision of S2-220xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0081 | rev |  | Current version: | 17.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Security procedures for L3 relaying |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Interdigital Inc., LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe |  |  |  |  |  | Date: |  |  | 2022-01-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Security procedures for authentication of the 5G ProSe Remote UE and for authorisation of both 5G ProSe UE-to-Network Relay and the 5G ProSe Remote UE has been postponed to wait for SA3 progress. It was proposed earlier to specify detailed call flows for CP security solution in TS 23.304 but now that SA3 has already decided to specify CP and UP solution and the related call flows are already available in (draft) TS 33.503, it is more appropriate for SA2 TS to refer to the existing text in TS 33.503 that has already been adopted in TS 23.304 vocabulary. TS 33.503 clause 6.3.3.2 and clause 6.3.3.3 already specifies the call flow procedure details for Remote UE and Relay UE security procedures for User Plane solution and Control Plane solution respectively. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the main principles of UP and CP security procedures for authentication and authorisation of 5G ProSe Layer-3 UE-to-Network relaying to so far empty clause 5.1.4.3. Being aware of SA3 work in this area, this clause is left a high level description that refers to TS 33.503 for further details. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 5G ProSe Layer-3 UE-to-Network relaying does not work as the security procedures are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 5.1.4.3	Principles for Network controlled authentication and authorisation for 5G ProSe Layer-3 UE-to-Network Relay

##### 5.1.4.3.1	General

Security procedures over Control Plane and User Plane are specified for 5G ProSe Layer-3 UE-to-Network relaying in TS 33.503 [29].

##### 5.1.4.3.2	Principles for Control Plane based authentication and authorisation for 5G ProSe Layer-3 UE-to-Network Relay

Control Plane-based authentication and authorisation for 5G ProSe Layer-3 UE-to-Network Relay call flow and procedure is defined in TS 33.503 [29]. It is characterised by the following principles:

-	5G ProSe Layer-3 UE-to-Network Relay's NAS signalling is used for the 5G ProSe Layer-3 Remote UE's authentication and authorisation.

-	The AMF serving the 5G ProSe UE-to-Network Relay selects AUSF as specified in TS 23.501 [4] clause 6.3.4 considering also the 5G ProSe Layer-3 Remote UE information and interacts with the 5G ProSe Layer-3 Remote UE's AUSF.

- 	The 5G ProSe Layer-3 Remote UE and the 5G ProSe Layer-3 UE-to-Network Relay perform PC5 connection establishment based on successful outcome of 5G ProSe Layer-3 Remote UE authentication and authorization via 5G ProSe UE-to-Network Relay's AMF.