3GPP TSG-SA WG2 Meeting #149E (e-meeting)	S2-2201023r01

# 14 – 25 February, 2022, Electronic meeting		                  (revision of S2-220xxxx)

Title:	[Draft] Response LS on Clarification on Scheduled Location Time

Response to:	S2-2200197 (C4-220306) LS on Clarification on Scheduled Location Time

Release:	Rel-17

Work Item:	5G_eLCS_Ph2

Source:	SA2

To:	CT4

Cc:	CT1

Contact Person:

Name:                   Yunjing Hou

E-mail Address:   houyunjing (at) catt (dot) cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	S2-2201022 (TS 23.273 CR 0225)

1. Overall Description:

SA2 thanks CT4 for their LS on Clarification on Scheduled Location Time. SA2 discussed the questions and agreed the following answers.

Q1: Is scheduled location time applicable for bulk operation?

A1: Bulk operation applies to 5GC-MT-LR and Deferred 5GC-MT-LR request targeting to a group of UE identified by an group ID. Because the scheduled location time is applicable for the 5GC-MT-LR and Deferred 5GC-MT-LR request, therefore SA2 believes that the scheduled location time can also be applicable for bulk operation.

Q2: When scheduled location time is provided in MT-LR and MO-LR, which network function(s) will handle the preparation phase and execution phase logic?

A2: LMF is responsible to handle the preparation phase and execution phase logic.

Q3: Has SA2 already considered the risks of HTTP timeout when defining the synchronized service operation invocation for MT-LR and MO-LR with scheduled location time?

A3: SA2 discussed the question and agreed the CR in the attachment to solve the risks of HTTP timeout.

2. Actions:

To CT4 group.

ACTION: 	SA2 kindly asks CT4 to take the above information into account.

3. Date of Next TSG-SA2 Meetings:

TSG-SA2 Meeting #150-e		06 - 12 April 2022			Electronic Meeting