3GPP TSG-WG SA2 Meeting #149E e-meeting	S2-2201028

e-Meeting, 14th-25th Feb, 2022

# Title:	[Draft] Reply LS on lists of 5GS forbidden tracking area for non-3GPP access

# Response to:	LS S2-2200083/ C1-220810 on lists of 5GS forbidden tracking area for non-3GPP access

# Release:	Rel-17

# Work Item:	TEI17_N3SLICE

Source:	SA2

To:	CT1

Cc:	None

Contact Person:

Name:	Zhu Jinguo

E-mail Address:	zhu dot Jinguo at ZTE dot com dot cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	TS23.501 CR 3556

1. Overall Description:

SA2 would like to thank CT1 for their LS on lists of 5GS forbidden tracking area for non-3GPP access.

SA2 has discussed and agreed the following:

In case of Untrusted or Trusted Non 3GPP access the UE cannot determine the current TAI for non-3GPP access in order to add this TAI into the list of 5GS forbidden tracking areas.

Thus, in case of Untrusted or Trusted Non 3GPP access, the network should not indicate to the UE that the current tracking area is not allowed or that roaming is not allowed in this tracking area.

The network may reject the UE registration request due to other reasons e.g.:

- the UE not being allowed to access to the PLMN or to any of the slices it is trying to register for

- the RAT is forbidden

SA2 has agreed with the attached CR to clarify this.

Regarding the questions asked by CT1:

- how does the UE know the current TAI for non-3GPP access in order to add this specific TAI into the list of 5GS forbidden tracking areas? and

SA2 Answer: In case of Untrusted or Trusted N3GPP access the UE is not expected to know the current TAI for non-3GPP access in order to add this specific TAI into the list of 5GS forbidden tracking areas.

- does the UE need to keep track of multiple non-3GPP TAIs within the list of 5GS forbidden tracking areas for the non-3GPP access?

SA2 Answer: NO

2. Actions:

To: CT1

ACTION:

SA2 asks kindly CT1 to take the above information into consideration.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #150E                               April 4 – 8, 2022