SA WG2 Meeting #149E (e-meeting)	S2-2201035r02

February 14 – 25, 2022, Elbonia	(revision of S2-220xxxx)

Source:	ZTE, LG Electronics, Nokia, Nokia Shanghai Bell, NTT DOCOMO, NEC.

Title:	New key issue: KI for objective #1 – Support of network slice service continuity

Document for:	Approval

Agenda Item:	9.14 Study on Network Slicing Phase 3

Work Item / Release:	FS_eNS_Ph3 / Rel-18

Abstract of the contribution: This paper proposes KI for objective #1 – network slice Service continuity

Discussion

This paper proposes Key Issue for FS_eNS_Ph3 objective #1 (Service continuity).

1.	Study whether and how to address the following scenario in order to provide service continuity: if an existing network slice or network slice instance cannot serve the PDU session,  or if the existing network slice instance cannot meet the performance requirements of the applications. The study should investigate whether deployment optimization is sufficient. Minimized system optimisations can be considered if valuable.

Proposal

It is proposed to agree the following changes into TR 23.700-41 on FS_eNS_Ph3.

* * * * Start of 1st Change (All new text) * * * *

5.X	Key Issue #X: Support of Network Slice Service continuity

5.X.1	Description

The following scenarios are to be considered in this key issue:

1) No mobility scenario:

Scenario 1a): network slice is overloaded in NG-RAN

Scenario 1b): network slice or network slice instance is overloaded in CN

Scenario 1c): network performance of the network slice cannot meet the requirement of applications.

2) Inter RA Mobility scenario:

Scenario 2a): network slice is not supported in the target RAN node

Scenario 2b): network slice in target RAN node is overloaded

Scenario 2c): network slice is not supported in the target CN.

Scenario 2d): network slice or network slice instance is overloaded in the target CN

This key issue will study whether and how to address the scenarios above in order to provide service continuity for existing PDU session.

NOTE 1:	For scenario 1a) and 2b), TR38.832[xx] already has conclusion. However it doesn’t preclude that SA2 can revisit the conclusions in TR38.832[xx].

NOTE 2:	For scenario 2a), TR38.832[xx] does not make any recommendations since it has system impact and require study in SA2.

NOTE 3:	Coordination with RAN working groups may be needed to conclude the key issue.

* * * * Next Change  * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[xx]	3GPP TR 38.832: "Study on enhancement of Radio Access Network (RAN) slicing".

* * * * End of Changes * * * *