3GPP TSG-SA2 Meeting #149E	S2-2201048

# 14 – 25 February 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 0704 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Fixes for AF session with required QoS update procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Ericsson, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | IIoT |  |  |  |  |  | Date: |  |  | 2022-02-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Provide updates to 23.503 for TSC QoS Changes are required to align TS23.503 AF Session with Required QoS with TS 23.502 clause 4.15.6.6 and 4.15.6.6a (Setting Up and Updating AF Session with required QoS). TS 23.502 specifies individual QoS parameters (Alternative QoS Related parameters) may be specifed instead of a QoS Reference in Alt-QoS Requirements while corresponding text is missing from TS23.503. Note the Alternative QoS Related parameters are defined as the parameters required in 23.501 clause 5.7.1.2a for the Alternative QoS Profile. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 23.503 clause 6.1.3.22 is updated for consistency with 23.502, which allows an AF to specify individual Alternative QoS Related Parameters instead of a QoS Reference for Alternative QoS Requirements. Also text is corrected so that Alternative Service Requirements can be provided in addition to the QoS Reference or individual QoS parameters |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TS23.503 is inconsistent with TS23.502 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.22 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "Technical Specification Group Services and System Aspects; System Architecture for the 5G System".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 23.203: "Policies and Charging control architecture; Stage 2".

[5]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[6]	3GPP TS 23.179: "Functional architecture and information flows to support mission-critical communication service; Stage 2".

[7]	Void.

[8]	3GPP TS 32.240: "Charging management; Charging architecture and principles".

[9]	3GPP TS 23.402: "Architecture enhancements for non-3GPP accesses".

[10]	3GPP TS 23.161: "Network-Based IP Flow Mobility (NBIFOM); Stage 2".

[11]	3GPP TS 23.261: "IP flow mobility and seamless Wireless Local Area Network (WLAN) offload; Stage 2".

[12]	3GPP TS 23.167: "3rd Generation Partnership Project; Technical Specification Group Services and Systems Aspects; IP Multimedia Subsystem (IMS) emergency sessions".

[13]	3GPP TS 29.507: "Access and Mobility Policy Control Service; Stage 3".

[14]	Void.

[15]	3GPP TS 22.011: "Service Accessibility".

[16]	3GPP TS 23.221: "Architectural requirements".

[17]	3GPP TS 29.551: "5G System; Packet Flow Description Management Service; Stage 3".

[18]	3GPP TS 32.421: "Telecommunication management; Subscriber and equipment trace; Trace concepts and requirements".

[19]	3GPP TS 24.526: "UE Equipment (UE) policies for 5G System (5GS); Stage 3".

[20]	3GPP TS 32.291: "Charging management; 5G system, Charging service; stage 3".

[21]	3GPP TS 32.255: "Telecommunication management; Charging management; 5G Data connectivity domain charging; Stage 2".

[22]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[23]	3GPP TS 23.280: "Common functional architecture to support mission critical services; Stage 2".

[24]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[25]	3GPP TS 23.216: "Single Radio Voice Call Continuity (SRVCC); Stage 2".

[26]	3GPP TS 23.272: "Circuit Switched (CS) fallback in Evolved Packet System (EPS); Stage 2".

[27]	3GPP TS 23.316: "Wireless and wireline convergence access support for the 5G System (5GS)".

[28]	3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services".

[29]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[30]	3GPP TS 24.237: "IP Multimedia (IM) Core Network (CN) subsystem IP Multimedia Subsystem (IMS) Service Continuity; Stage 3".

[31]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia telephony; Media handling and interaction".

[32]	3GPP TS 29.510: "5G System; Network Function Repository Services; Stage 3".

[33]	3GPP TS 23.548: "5G System Enhancements for Edge Computing; Stage 2".

[34]	3GPP TS 23.304: "Proximity based Services (ProSe) in the 5G System (5GS)".

[35]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

#### [xx]	3GPP TS 29.514: "Policy Authorization Service; Stage 3".

SECOND CHANGE

#### 6.1.3.22	AF session with required QoS

The AF may request that a data session to a UE is set up with a specific QoS (e.g. low latency or jitter) and priority handling. The AF can request the network to provide QoS for the AF session based on the service requirements with the help of a QoS Reference parameter that refers to pre-defined QoS information. Instead of the QoS Reference, the AF may provide individual QoS parameters associated to the Flow Description.

a)	When the AF provides only a QoS Reference to determine the QoS parameters but no individual QoS parameters:

-	When the PCF authorizes the service information from the AF, it derives the QoS parameters of the PCC rule based on the service information and the indicated QoS Reference.

NOTE 1:	A SLA has to be in place between the operator and the ASP defining the possible QoS levels and their charging rates. For each of the possible pre-defined QoS information sets, the PCF needs to be configured with the corresponding QoS parameters and their values as well as the appropriate Charging key (or receive this information from the UDR).

-	The AF may change the QoS by providing a different QoS Reference while the AF session is ongoing. If this happens, the PCF shall update the related QoS parameter sets in the PCC rule accordingly.

b)	When the AF provides individual QoS parameters instead of a QoS Reference:

-	The AF provides one or more of the following individual QoS parameters, i.e. Requested Priority, Maximum Burst Size, Requested 5GS Delay, Requested Maximum  Bitrate and Requested Guaranteed Bitrate.

NOTE 2:	Different combinations of individual QoS parameters with specific parameter names exist and they are described in TS 23.501 [2] (for Time Sensitive Communication), in clause 6.1.3.23 (for integration with Time Sensitive Networking) and in TS 29.514 [xx].

-	If the AF request for QoS is sent via the TSCTSF and the request contains a Requested 5GS Delay, the TSCTSF determines a Requested PDB considering the UE-DS-TT Residence Time (either provided by the PCF or pre-configured).

-	When the PCF authorizes the service information from the AF, it derives the QoS parameters of the PCC rule based on the service information and the individual QoS information received from the AF and TSCTSF. The PCF should select a standardized, pre-configured or existing dynamically assigned 5QI that matches the individual QoS parameters. If no 5QI exists that matches the individual QoS parameters, the PCF generates a new dynamically assigned 5QI based on the individual QoS parameters.

NOTE 3:	In this release of the specification, the information about the Packet Error Rate that is required for the AF session has to be configured in the PCF so that it can be taken into account for the selection or generation of the most appropriate 5QI.

-	The AF may change the QoS by providing different values for the individual QoS parameters while the AF session is ongoing. If this happens, the PCF shall update the related QoS parameter sets in the PCC rule accordingly.

-	The PCF may reject the individual QoS parameters received from the AF based on operator policy or impossibility to support the requested values of the individual QoS parameters. If this happens, the PCF may provide one or more combinations of individual QoS parameters that can be supported in the response to the AF.

In addition to the QoS reference or the individual QoS parameters described above, the AF may provide the following QoS Parameters associated with the Flow Description. One of the following:

a)	In the case that the AF takes the role of TSN AF which is a 5GC NF: TSC Assistance Container: describing the traffic characteristics and needed for TSCAI determination (as described in clause 5.27.2.4 of TS 23.501 [2]); or

b)	In the case that the AF is not the TSN AF: parameters that describe traffic characteristics and subsequently used for the determination of TSC Assistance Container as described in clause 5.27.2.3 of TS 23.501 [2].

The PCF generates a PCC Rule with service data flow filter (including IP Packet Filter set as in clause 5.7.6.2 of TS 23.501 [2]) or Ethernet Packet Filter set as in clause 5.7.6.3 of TS 23.501 [2]) derived from the Flow Descriptions provided by the AF, the derived PCC rule QoS parameters such as 5QI, ARP, GBR and MBR (see clause 6.3.1 for all possible PCC rule QoS parameters) and the associated TSC Assistance Container as received from the TSN AF or TSCTSF.

For TSC QoS, the PCF derives the 5QI value as defined in clause 5.27.3 of TS 23.501 [2], the PCF derives the MBR using the Requested Maximum Bitrate provided by the AF and sets the GBR equal to the MBR unless the AF provides a Requested Guaranteed Bitrate, in which case the MBR and GBR are set separately.

If the PCF gets informed about Policy Control Request Triggers relevant for the AF session, the PCF shall inform the AF about it as defined in clause 6.1.3.18.

If an AF session can adjust to different QoS parameter combinations, the AF may provide Alternative Service Requirements in a prioritized order (indicating the preference of the QoS requirements with which the service can operate) in addition to the QoS Reference or individual QoS parameters. Alternative Service Requirements contain:

- When the AF requests the network to provide QoS with a QoS reference, one or more QoS Reference parameters in a prioritized order.

- When the AF requests the network to provide QoS with individual parameters, one or more Alternative QoS Related Parameter set(s) in a prioritized order. Each Alternative QoS Related Parameter set is comprised of the following individual parameters: Requested 5GS Delay and Requested Guaranteed Flow Bitrate.

NOTE 4:	In this release of the specification, the information about the Packet Error Rate that is required for the each of the Alternative QoS Related Parameter sets has to be configured in the PCF.

If the AF request is sent via the TSCTSF, the TSCTSF determines a Requested PDB considering the Requested 5GS Delay and UE-DS-TT Residence Time.

An AF that provides Alternative Service Requirements shall also subscribe to receive notifications from the PCF for successful resource allocation and when the QoS targets can no longer (or can again) be fulfilled as described in clause 6.1.3.18.

When the PCF authorizes the service information from the AF and generates a PCC rule, it shall also derive Alternative QoS parameter sets for this PCC rule based on the QoS Reference parameters or the Alternative QoS Related parameter sets in the Alternative Service Requirements. If the AF provided Alternative QoS Related parameter sets in the request, the PCF may reject any of the Alternative QoS Related parameter sets received based on operator policy or impossibility to support the requested values of the individual parameters. If this happens, the PCF may provide one or more Alternative QoS Related parameter sets that can be supported in the response to the AF.

The PCF shall enable QoS Notification Control and include the derived Alternative QoS parameter sets (in the same prioritized order indicated by the AF) in the PCC rule sent to the SMF. When the PCF notifies the AF that QoS targets can no longer be fulfilled, the PCF shall include the QoS Reference parameter or the set of Alternative QoS Related parameters corresponding to the Alternative QoS parameter set referenced by the SMF, or an indication that the lowest priority QoS Reference or the lowest priority set of Alternative QoS Related parameters of the Alternative Service Requirements cannot be fulfilled (as described in clause 6.1.3.18).

NOTE 2:	The AF behaviour is out of the scope of this TS but can include adaptation to the change of QoS (e.g. rate adaptation) as well as application layer signalling with the UE.

The AF may change the Alternative Service Requirements while the AF session is ongoing. If this happens, the PCF shall update the Alternative QoS parameter sets in the PCC rule accordingly.

The AF may indicate to the PCF that the UE does not need to be informed about changes related to Alternative QoS Profiles. With this indication received from the AF, the PCF decides whether to disable the notifications to the UE when changes related to the Alternative QoS Profiles occur and sets the Disable UE notifications at changes related to Alternative QoS Profiles parameter in the PCC rule accordingly.

THIRD CHANGE

#### 6.1.3.23a	Support of Time Sensitive Communication and Time Synchronization

Enablers for Time Sensitive Communication and Time Synchronization are defined in TS 23.501 [2] clause 5.27.

In the case of integration with IEEE TSN network, the TSN AF interacts with the PCF as described in clause 6.1.3.23.

When the PCF determines that the PDU Session is potentially impacted by (g)PTP based time synchronization service, if integration with IEEE TSN does not apply, the PCF provides the following parameters to the TSCTSF:

-	5GS user-plane Node information:

-	5GS Bridge ID;

-	UE-DS-TT Residence time;

-	port number of the DS-TT;

-	MAC address of the Ethernet port of DS-TT (i.e. DS-TT port MAC address) (for Ethernet type PDU Session), or IP address of the UE (for IP type PDU Session);

-	Port Management Information Container and the related port number.

-	User plane node Management Information Container.

Upon reception of the above information, if the TSCTSF does not have a corresponding AF session, the TSCTSF shall create an AF-session with the PCF. The TSCTSF may request that an AF session to a UE is set up with a specific QoS as described in clause 6.1.3.22.

The TSCTSF may use the PTP Port state of NW-TT and DS-TT in the Port/User plane node Management Information Container to determine the Port Pairs that will be used for (g)PTP delivery. Based on this the TSCTSF may request appropriate QoS treatment for the (g)PTP flows from PCF.

End of CHANGEs