3GPP TSG-SA2 Meeting #149E	S2-2201050r01

# 14-25 February 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3557 | rev | - | Current version: | 16.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | LLDP neighbor discovery information fixes |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | IIoT |  |  |  |  |  | Date: |  |  | 2021-10-2 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Now the 5GS with support of LLDP neighbor discovery will transfer to TSN AF via PMIC/BMIC about the discovered LLDP neighbor discovery information for each discovered neighbor of NW-TT and/or DS-TT. However, the collected LLDP neighboring discovery information by TSN AF is missing in the Bridge information indicated to the CNC. Without this neighboring discovery information for each discovered neighbor, the CNC cannot perform stream scheduling and path calculation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Provide updates to 23.501 for including LLDP neighboring discovery information in the Bridge information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete TS 23.501 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.28.1, 5.28.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

5.28	Support of integration with TSN

5.28.1	5GS TSN bridge management

5GS functions acts as one or more TSN Bridges of the TSN network. The 5GS Bridge is composed of the ports on a single UPF (i.e. PSA) side, the user plane tunnel between the UE and UPF, and the ports on the DS-TT side. For each 5GS Bridge of a TSN network, the port on NW-TT support the connectivity to the TSN network, the ports on DS-TT side are associated to the PDU Session providing connectivity to the TSN network.

The granularity of the 5GS TSN bridge is per UPF for each network instance. The bridge ID of the 5GS TSN bridge is bound to the UPF ID of the UPF as identified in TS 23.502 [3]. The TSN AF stores the binding relationship between a port on UE/DS-TT side and a PDU Session during reporting of 5GS TSN bridge information. The TSN AF also stores the information about ports on the UPF/NW-TT side. The UPF/NW-TT forwards traffic to the appropriate egress port based on the traffic forwarding information. From the TSN AF point of view, a 5GS TSN bridge has a single NW-TT entity within UPF and the NW-TT may have multiple ports that are used for traffic forwarding.

NOTE 1:	How to realize single NW-TT entity within UPF is up to implementation.

NOTE 2:	Ethernet PDU Session type in this release of the specification may be subject to the constraint that it supports a single N6 interface in a UPF associated with the N6 Network Instance.

There is only one PDU Session per DS-TT port for a given UPF. All PDU Sessions which connect to the same TSN network via a specific UPF are grouped into a single 5GS bridge. The capabilities of each port on UE/DS-TT side and UPF/NW-TT side are integrated as part of the configuration of the 5GS Bridge and are notified to TSN AF and delivered to CNC for TSN bridge registration and modification.

NOTE 3:	It is assumed that all PDU sessions which connect to the same TSN network via a specific UPF are handled by the same TSN AF.

Figure 5.28.1-1: Per UPF based 5GS bridge

NOTE 4:	If a UE establishes multiple PDU Sessions terminating in different UPFs, then the UE is represented by multiple 5GS TSN bridges.

In order to support TSN scheduled traffic (clause 8.6.8.4 in IEEE Std 802.1Q-2018 [98]) over 5GS Bridge, the 5GS supports the following functions:

-	Configure the bridge information in 5GS.

-	Report the bridge information of 5GS Bridge to TSN network after PDU session establishment.

-	Receiving the configuration from TSN network as defined in clause 5.28.2.

-	Map the configuration information obtained from TSN network into 5GS QoS information (e.g. 5QI, TSC Assistance Information) of a QoS Flow in corresponding PDU Session for efficient time-aware scheduling, as defined at clause 5.28.2.

The bridge information of 5GS Bridge is used by the TSN network to make appropriate management configuration for the 5GS Bridge. The bridge information of 5GS Bridge includes at least the following:

-	Information for 5GS Bridge:

-	Bridge ID

Bridge ID is to distinguish between bridge instances within 5GS. The Bridge ID can be derived from the unique bridge MAC address as described in IEEE Std 802.1Q [98], or set by implementation specific means ensuring that unique values are used within 5GS;

-	Number of Ports;

-	list of port numbers.

-	Capabilities of 5GS Bridge as defined in IEEE Std 802.1Qcc [95]:

-	5GS Bridge delay per port pair per traffic class, including 5GS Bridge delay (dependent and independent of frame size, and their maximum and minimum values: independentDelayMax, independentDelayMin, dependentDelayMax, dependentDelayMin), ingress port number, egress port number and traffic class.

-	Propagation delay per port (txPropagationDelay), including transmission propagation delay, egress port number.

-	VLAN Configuration Information.

NOTE 5:	This Release of the specification does not support the modification of VLAN Configuration Information at the TSN AF.

-	Topology of 5GS Bridge as defined in IEEE Std 802.1AB [97]:

-	LLDP Configuration Information.

-	LLDP Discovery Information for each discovered neighbor of NW-TT port and DS-TT port.

-	Traffic classes and their priorities per port as defined in IEEE Std 802.1Q [98].

-	Stream Parameters as defined in clause 12.31.1 in IEEE Std 802.1Q [98], in order to support PSFP:

-	MaxStreamFilterInstances: The maximum number of Stream Filter instances supported by the bridge;

-	MaxStreamGateInstances: The maximum number of Stream Gate instances supported by the bridge;

-	MaxFlowMeterInstances: The maximum number of Flow Meter instances supported by the bridge (optional);

-	SupportedListMax: The maximum value supported by the bridge of the AdminControlListLength and OperControlListLength parameters.

The following parameters: independentDelayMax and independentDelayMin, how to calculate them is left to implementation and not defined in this specification.

DS-TT and NW-TT report txPropagationDelay to the TSN AF relative to the time base of the TSN GM clock (identified by the TSN time domain number received in PMIC). If the TSN AF has subscribed for notifications on txPropagationDelay and if the difference to the previously reported txPropagationDelay is larger than the txPropagationDelayDeltaThreshold received in PMIC, the corresponding DS-TT or NW-TT informs the TSN AF about the updated txPropagationDelay using PMIC signalling.

NOTE 6:	Configuration of TSN time domain number and txPropagationDelayDeltaThreshold via PMIC is optional for NW-TT. NW-TT can instead be pre-configured with the threshold and the single time domain that is used by the CNC for bridge configuration and reporting.

Bridge ID of the 5GS Bridge, port number(s) of the Ethernet port(s) in NW-TT could be preconfigured on the UPF. The UPF is selected for a PDU Session serving TSC as described in clause 6.3.3.3.

This release of the specification requires that each DS-TT port is assigned with a globally unique MAC address.

NOTE 7:	The MAC address of the DS-TT port must not be used in user data traffic; it is used for identification of the PDU Session and the associated bridge port within the 3GPP system.

Port number of Ethernet port on the DS-TT for the PDU Session is assigned by the UPF during PDU session establishment. The port number of the DS-TT Ethernet port for a PDU Session shall be reported to the SMF from the UPF and further stored at the SMF. SMF provides the port number and MAC address of the Ethernet port in DS-TT of the related PDU session and port number(s) and MAC address(es) of the Ethernet port(s) in NW-TT to the TSN AF via PCF. If a PDU session for which SMF has reported port numbers to TSN AF is released, then SMF informs TSN AF accordingly. When there are multiple network instances within a UPF, each network instance is considered logically separate. The network instance for the N6 interface (clause 5.6.12) may be indicated by the SMF to the UPF for a given PDU session during PDU session establishment.

The TSN AF is responsible to receive the bridge information of 5GS Bridge from 5GS, as well as register or update this information to the CNC.

SECOND CHANGE

5.28.3	Port and bridge management information exchange in 5GS

5.28.3.1	General

Port and bridge management information is exchanged between CNC and TSN AF. The port management information, is related to Ethernet ports located in DS-TT or NW-TT.

5GS shall support transfer of standardized and deployment-specific port management information transparently between TSN AF and DS-TT or NW-TT, respectively inside a Port Management Information Container. NW-TT may support one or more ports. In this case, each port uses separate Port Management Information Container. 5GS shall also support transfer of standardized and deployment-specific bridge management information transparently between TSN AF and NW-TT, respectively inside a Bridge Management Information Container. Table 5.28.3.1-1 and Table 5.28.3.1-2 list standardized port management information and bridge management information, respectively.

Table 5.28.3.1-1: Standardized port management information

| Port management information | Applicability (see NOTE 6) |  | Supported operations by TSN AF | Reference |
| --- | --- | --- | --- | --- |
|  | DS-TT | NW-TT | (see NOTE 1) |  |
| General |  |  |  |  |
| Port management capabilities (see NOTE 2) | X | X | R |  |
| Bridge delay related information |  |  |  |  |
| txPropagationDelay | X | X | R | IEEE Std 802.1Qcc [95] clause 12.32.2.1 |
| txPropagationDelayDeltaThreshold (see NOTE 13) | X | X | RW |  |
| Traffic class related information |  |  |  |  |
| Traffic class table | X | X | RW | IEEE Std 802.1Q [98] clause 12.6.3 and clause 8.6.6. |
| Gate control information |  |  |  |  |
| GateEnabled | X | X | RW | IEEE Std 802.1Q [98] Table 12-29 |
| AdminBaseTime | X | X | RW | IEEE Std 802.1Q [98] Table 12-29 |
| AdminControlList | X | X | RW | IEEE Std 802.1Q [98] Table 12-29 |
| AdminCycleTime (see NOTE 3) | X | X | RW | IEEE Std 802.1Q [98] Table 12-29 |
| AdminControlListLength (see NOTE 3) | X | X | RW | IEEE Std 802.1Q [98] Table 12-29 |
| AdminCycleTimeExtension | X | X | RW | IEEE Std 802.1Q [98] Table 12-29 |
| Tick granularity | X | X | R | IEEE Std 802.1Q [98] Table 12-29 |
| General Neighbour discovery configuration (NOTE 4) |  |  |  |  |
| adminStatus | D | X | RW | IEEE Std 802.1AB [97] clause 9.2.5.1 |
| lldpV2LocChassisIdSubtype | D | X | RW | IEEE Std 802.1AB [97] Table 11-2 |
| lldpV2LocChassisId | D | X | RW | IEEE Std 802.1AB [97] Table 11-2 |
| lldpV2MessageTxInterval | D | X | RW | IEEE Std 802.1AB [97] Table 11-2 |
| lldpV2MessageTxHoldMultiplier | D | X | RW | IEEE Std 802.1AB [97] Table 11-2 |
| NW-TT port neighbour discovery configuration |  |  |  |  |
| lldpV2LocPortIdSubtype |  | X | RW | IEEE Std 802.1AB [97] Table 11-2 |
| lldpV2LocPortId |  | X | RW | IEEE Std 802.1AB [97] Table 11-2 |
| DS-TT port neighbour discovery configuration |  |  |  |  |
| lldpV2LocPortIdSubtype | D |  | RW | IEEE Std 802.1AB [97] Table 11-2 |
| lldpV2LocPortId | D |  | RW | IEEE Std 802.1AB [97] Table 11-2 |
| Neighbour discovery information for each discovered neighbour of NW-TT (NOTE XX) |  |  |  |  |
| lldpV2RemChassisIdSubtype |  | X | R | IEEE Std 802.1AB [97] Table 11-2 |
| lldpV2RemChassisId |  | X | R | IEEE Std 802.1AB [97] Table 11-2 |
| lldpV2RemPortIdSubtype |  | X | R | IEEE Std 802.1AB [97] Table 11-2 |
| lldpV2RemPortId |  | X | R | IEEE Std 802.1AB [97] Table 11-2 |
| TTL |  | X | R | IEEE Std 802.1AB [97] clause 8.5.4 |
| Neighbour discovery information for each discovered neighbour of DS-TT (NOTE 5) |  |  |  |  |
| lldpV2RemChassisIdSubtype | D |  | R | IEEE Std 802.1AB [97] Table 11-2 |
| lldpV2RemChassisId | D |  | R | IEEE Std 802.1AB [97] Table 11-2 |
| lldpV2RemPortIdSubtype | D |  | R | IEEE Std 802.1AB [97] Table 11-2 |
| lldpV2RemPortId | D |  | R | IEEE Std 802.1AB [97] Table 11-2 |
| TTL | D |  | R | IEEE Std 802.1AB [97] clause 8.5.4.1 |
| Stream Parameters (NOTE 11) |  |  |  |  |
| MaxStreamFilterInstances | X |  | R | IEEE Std 802.1Q [98] clause 12.31.1.1 |
| MaxStreamGateInstances | X |  | R | IEEE Std 802.1Q [98] clause 12.31.1.2 |
| MaxFlowMeterInstances | X |  | R | IEEE Std 802.1Q [98] clause 12.31.1.3 |
| SupportedListMax | X |  | R | IEEE Std 802.1Q [98] clause 12.31.1.4 |
| Per-Stream Filtering and Policing information (NOTE 10) |  |  |  |  |
| Stream Filter Instance Table (NOTE 8) |  |  |  | IEEE Std 802.1Q [98] Table 12-32 |
| > StreamFilterInstanceIndex | X | X | RW | IEEE Std 802.1Q [98] Table 12-32 |
| > Stream Identification type | X | X | RW | IEEE 802.1CB [83] clause 9.1.1.6 |
| > Stream Identification Controlling Parameters | X | X | RW | IEEE 802.1CB [83] clauses 9.1.2, 9.1.3, 9.1.4 (NOTE 12) |
| > PrioritySpec | X | X | RW | IEEE Std 802.1Q [98] Table 12-32 |
| > StreamGateInstanceID | X | X | RW | IEEE Std 802.1Q [98] Table 12-32 |
| Stream Gate Instance Table (NOTE 9) |  |  |  | IEEE Std 802.1Q [98] Table 12-33 |
| StreamGateInstanceIndex | X | X | RW | IEEE Std 802.1Q [98] Table 12-33 |
| PSFPAdminBaseTime | X | X | RW | IEEE Std 802.1Q [98] Table 12-33 |
| PSFPAdminControlList | X | X | RW | IEEE Std 802.1Q [98] Table 12-33 |
| PSFPAdminCycleTime | X | X | RW | IEEE Std 802.1Q [98] Table 12-33 |
| PSFPAdminCycleTimeExtension | X | X | RW | IEEE Std 802.1Q [98] Table 12-33 |
| PSFPTickGranularity | X | X | R | IEEE Std 802.1Q [98] Table 12-33 |
| Time Synchronization Information |  |  |  |  |
| TSN Time domain number (NOTE 14) | X | X | RW |  |
| NOTE 1: R = Read only access; RW = Read/Write access. NOTE 2: Indicates which standardized and deployment-specific port management information is supported by DS-TT or NW-TT. NOTE 3: AdminCycleTime and AdminControlListLength are optional for gate control information. NOTE 4: If DS-TT supports neighbour discovery, then TSN AF sends the general neighbour discovery configuration for DS-TT Ethernet ports to DS-TT. If DS-TT does not support neighbour discovery, then TSN AF sends the general neighbour discovery configuration for DS-TT Ethernet ports to NW-TT using the Bridge Management Information Container (refer to Table 5.28.3.1-2) and NW-TT performs neighbour discovery on behalf on DS-TT. When a parameter in this group is changed, it is necessary to provide the change to every DS-TT and the NW-TT that belongs to the 5GS TSN bridge. It is mandatory that the general neighbour discovery configuration is identical for all DS-TTs and the NW-TTs that belongs to the bridge. NOTE 5: If DS-TT supports neighbour discovery, then TSN AF retrieves neighbour discovery information for DS-TT Ethernet ports from DS-TT. TSN AF indicates the neighbor discovery information for each discovered neighbor of DS-TT port to CNC. If DS-TT does not support neighbour discovery, then TSN AF retrieves neighbour discovery information for DS-TT Ethernet ports from NW-TT, using the Bridge Management Information Container (refer to Table 5.28.3.1-2), the NW-TT performing neighbour discovery on behalf on DS-TT. NOTE 6: X = applicable; D = applicable when validation and generation of LLDP frames is processed at the DS-TT. NOTE 7: Void. NOTE 8: There is a Stream Filter Instance Table per Stream. NOTE 9: There is a Stream Gate Instance Table per Gate. NOTE 10: TSN AF indicates the support for PSFP to the CNC only if each DS-TT and NW-TT of the 5GS bridge has indicated support of PSFP. DS-TT indicates support of PSFP using port management capabilities, i.e. by indicating support for the Per-Stream Filtering and Policing information and by setting higher than zero values for MaxStreamFilterInstances, MaxStreamGateInstances, MaxFlowMeterInstances, SupportedListMax parameters. When available, TSN AF uses the PSFP information for determination of the traffic pattern information as described in Annex I. The PSFP information can be used at the DS-TT (if supported) and at the NW-TT (if supported) for the purpose of per-stream filtering and policing as defined in IEEE Std 802.1Q [98] clause 8.6.5.1. NOTE 11: TSN AF composes a Stream Parameter Table towards the CNC. It is up to TSN AF how it composes the Stream Parameter Table based on the numerical values as received from DS-TT and NW-TT port(s) and for the bridge for each individual parameter. NOTE 12: The set of Stream Identification Controlling Parameters depends on the Stream Identification type value as defined in IEEE Std 802.1CB [83] Table 9-1 and clauses 9.1.2, 9.1.3, 9.1.4. NOTE 13: Indicates how much the txPropagationDelay needs to change so that DS-TT/NW-TT report a change in txPropagationDelay to TSN AF. This is optional for NW-TT. NOTE 14: Indicates the (g)PTP domain (identified by a domain number) that is assumed by the CNC as the reference clock for time information in the scheduled traffic (gate control) information, PSFP information and bridge delay related information. This is optional for NW-TT. NOTE XX: TSN AF indicates the neighbor discovery information for each discovered neighbor of NW-TT port to CNC. |  |  |  |  |

Table 5.28.3.1-2: Standardized bridge management information

| Bridge management information | Supported operations by TSN AF (see NOTE 1) | Reference |
| --- | --- | --- |
| Information for 5GS Bridge |  |  |
| Bridge Address | R |  |
| Bridge ID | R |  |
| NW-TT port numbers | R |  |
| Traffic forwarding information |  |  |
| Static Filtering Entry (NOTE 3) | RW | IEEE Std 802.1Q [98] clause 8.8.1 |
| General Neighbour discovery configuration (NOTE 2) |  |  |
| adminStatus | RW | IEEE Std 802.1AB [97] clause 9.2.5.1 |
| lldpV2LocChassisIdSubtype | RW | IEEE Std 802.1AB [97] Table 11-2 |
| lldpV2LocChassisId | RW | IEEE Std 802.1AB [97] Table 11-2 |
| lldpV2MessageTxInterval | RW | IEEE Std 802.1AB [97] Table 11-2 |
| lldpV2MessageTxHoldMultiplier | RW | IEEE Std 802.1AB [97] Table 11-2 |
| DS-TT port neighbour discovery configuration for DS-TT ports (NOTE 4) |  |  |
| >DS-TT port neighbour discovery configuration for each DS-TT port |  |  |
| >> DS-TT port number | RW |  |
| >> lldpV2LocPortIdSubtype | RW | IEEE Std 802.1AB [97] Table 11-2 |
| >> lldpV2LocPortId | RW | IEEE Std 802.1AB [97] Table 11-2 |
| Discovered neighbour information for DS-TT ports (NOTE 4) |  |  |
| >Discovered neighbour information for each DS-TT port (NOTE 4) |  |  |
| >> DS-TT port number | R |  |
| >> lldpV2RemChassisIdSubtype | R | IEEE Std 802.1AB [97] Table 11-2 |
| >> lldpV2RemChassisId | R | IEEE Std 802.1AB [97] Table 11-2 |
| >> lldpV2RemPortIdSubtype | R | IEEE Std 802.1AB [97] Table 11-2 |
| >> lldpV2RemPortId | R | IEEE Std 802.1AB [97] Table 11-2 |
| >> TTL | R | IEEE Std 802.1AB [97] clause 8.5.4.1 |
| Stream Parameters (NOTE 5) |  |  |
| MaxStreamFilterInstances | R | IEEE Std 802.1Q [98] |
| MaxStreamGateInstances | R | IEEE Std 802.1Q [98] |
| MaxFlowMeterInstances | R | IEEE Std 802.1Q [98] |
| SupportedListMax | R | IEEE Std 802.1Q [98] |
| NOTE 1: R = Read only access; RW = Read/Write access. NOTE 2: General neighbour discovery information is included only when NW-TT performs neighbour discovery on behalf of DS-TT. When a parameter in this group is changed, it is necessary to provide the change to every DS-TT and the NW-TT that belongs to the 5GS TSN bridge. NOTE 3: If the Static Filtering Entry information is present, NW-TT uses Static Filtering Entry information to determine the NW-TT egress port for forwarding UL TSC traffic. If the Static Filtering Entry information is not present, then the forwarding information as in clause 5.8.2.5.3 applies. This release of the specification does not support Static Filtering Entries in the downlink direction. NOTE 4: DS-TT discovery configuration and DS-TT discovery information are used only when DS-TT does not support LLDP and NW-TT performs neighbour discovery on behalf of DS-TT. TSN AF indicates the discovered neighbor information for each DS-TT port to CNC. NOTE 5: TSN AF indicates the support for PSFP to the CNC only if each DS-TT and NW-TT of the 5GS bridge have indicated support of PSFP. The support of PSFP at the NW-TT ports is expressed by setting higher than zero values for MaxStreamFilterInstances, MaxStreamGateInstances, MaxFlowMeterInstances, SupportedListMax parameters. |  |  |

Exchange of port and bridge management information between TSN AF and NW-TT or DS-TT allows TSN AF to:

1)	retrieve port management information for a DS-TT or NW-TT Ethernet port or bridge management information for a 5GS TSN bridge;

2)	send port management information for a DS-TT or NW-TT Ethernet port or bridge management information for a 5GS TSN bridge;

3)	subscribe to and receive notifications if specific port management information for a DS-TT or NW-TT Ethernet port changes or bridge management information changes;

4)	delete selected entries in the following data structures:

-	"DS-TT port neighbour discovery configuration for DS-TT port" in BMIC using the DS-TT port number to reference the selected entry.

-	"Stream Filter Instance Table" in PMIC using the Stream Filter Instance ID to reference the selected entry.

-	"Stream Gate Instance Table" in PMIC using the Stream Gate Instance ID to reference the selected entry.

-	"Static Filtering Entries table" in BMIC using the (MAC address, VLAN ID) pair to reference the selected entry.

Exchange of port management information between TSN AF and NW-TT or DS-TT is initiated by DS-TT or NW-TT to:

-	notify TSN AF if port management information has changed that TSN AF has subscribed for.

Exchange of bridge management information between TSN AF and NW-TT is initiated by NW-TT to:

-	notify TSN AF if bridge management information has changed that TSN AF has subscribed for.

Exchange of port management information is initiated by DS-TT to:

-	provide port management capabilities, i.e. provide information indicating which standardized and deployment-specific port management information is supported by DS-TT.

TSN AF indicates inside the Port Management Information Container or Bridge Management Information Container whether it wants to retrieve or send port or bridge management information or intends to (un-)subscribe for notifications.

End of CHANGEs