SA WG2 Meeting #149e	S2-2201064r01

# February 14th – 25th, 2022; Elbonia               		(revision of S2-220)

Source: 	Nokia, Nokia Shanghai Bell, Xiaomi

Title: 	Enhancements to power savings for XR services

Document for: 	Approval

Agenda Item: 	9.19

Work Item / Release:	FS_XRM / Rel-18

Abstract of the contribution: This paper proposes a KI to address Work Task #4.2

- Discussion

The FS_XRM SID contains the following Work Task:

WT#4: Study potential enhancements of power management considering traffic pattern of media services:

- WT 4.1: Void.

- WT 4.2: Power saving enhancement e.g. support trade-off of throughput/latency/reliability considering device battery life, whether and how to enhance CDRX, considering XR/media traffic pattern.

WT#4.2 addresses the scenario where information on the XR media traffic like pattern, periodicity and other key statistics as well as device battery status are exposed to the network to enhance power saving schemes operating in the RAN like CDRX and PDCCH monitoring mechanisms.

2 Proposal

It is proposed to update TR 23.700-60 as follows

# * * * * First change (all new text)* * * *

x.1	Key Issue #X: Enhancements to power savings for XR services

x.1.1 	Description

The objective of this Key Issue is to study whether and how to enhance power management schemes in 5GS by exploiting information regarding the traffic generated by XR media services and device status. In particular, this Key Issue evaluates the possibility of using device status information like battery lifetime, battery status, and information regarding the traffic generated by XR media services to improve power management schemes like CDRX to achieve the best trade-off among key performance indicators like throughput, latency, reliability and power saving.

About the power consumption of multi-modality service applications, except the wireless connection which depends on several factors including bitrates, distance from radio access network, channel conditions, frequency range etc, the following factors should be considered: tracking and Sensing (3DoF tracking may be done with low power consumption than 6DoF), Compute and Media Processing (CPU), and Render (GPU) which depends on frame rates, resolution, display technology, and the display etc.(Identified by TR 26.928[Z]).

Solutions for this Key Issue rely on the enhancements to power saving schemes to support XR services studied during RAN1 SI on XR evaluation in Release 17 and under investigation in Release 18 and builds on top of those features.

For this Key Issue the following areas should be studied:

- Whether and how information related to enhancement of power management (e.g. the device battery capacity and status) needs to be identified and collected, represented and used in the 5GS?

- Whether and how information related to the traffic generated by XR media services is collected, represented and used in the 5GS?

  - Where and how to obtain and represent XR/media traffic characteristics and related information within the 5GS?

  - Which type of XR/media traffic characteristics and information like traffic pattern and statistics is needed for power management of the 5GS?

  - Where and how to obtain and represent XR/media traffic characteristics and related information within the 5GS?

  - Which NF(s) of 5GS needs to know about these information? And how to adjust the parameters for the better power management?