SA WG2 Meeting #149E (e-meeting)	S2-2201069r03

February 14 – 25, 2022, Elbonia	(revision of S2-220xxxx)

Source:	Nokia, Nokia Shanghai Bell, NEC, Ericsson, Huawei

Title:	New key issue: KI for objective #5 – Network Slice Area of Service not mapping to existing TAs boundaries, and Temporary network slices.

Document for:	Approval

Agenda Item:	9.14 Study on Network Slicing Phase 3

Work Item / Release:	FS_eNS_Ph3 / Rel-18

Abstract of the contribution: This paper proposes KI for objective #5

# Discussion

This paper proposes Key Issue for FS_eNS_Ph3 objective #5.

5.	Study deployment considerations when a service provided has an area of service that does not overlap with the already deployed Tracking Areas and/or have a limited lifetime, and how existing mechanisms including network slicing can help support such scenarios. If existing mechanisms are concluded to be not sufficient to achieve the scenarios, study whether and how additional mechanisms can resolve the analysed gap.

# Proposal

It is proposed to agree the following changes into TR 23.700-41 on FS_eNS_Ph3.

* * * * Start of 1st Change (All new text) * * * *

## 5.X	Key Issue #X: Network Slice Area of Service for services not mapping to existing TAs boundaries, and Temporary network slices.

### 5.X.1	Description

Network Slices are deployed for services over an Area of Service which may match the existing TAs or for which the Area of Service can be different. Currently, the network slice availability (i.e. where the network slices are defined to be supported) is designed to match deployed TA boundaries. In addition, the UEs and network configuration can be impacted when network slices are deployed and decommissioned over certain time interval (e.g. the Configured NSSAI can change when a network slice is no longer available or becomes available, this can affect the Allowed NSSAI and other parameters, and in turn the RA may need to change, etc).

This Key Issue will study how to address the issues described above and whether system level improvements are needed to mitigate e.g. the deployment and control plane issues that arise due to the currently defined system behaviour:

-	The support of services over network slices when the services have Area of Service not matching the existing deployed TA boundaries

-	The support of network slices which have a limited lifetime (including how to gracefully terminate a network slice which can apply also to network slices which have a longer lifespan).

NOTE 1:	Coordination with SA5 can be needed for Network Slice Life Cycle Management aspects.

* * * * End of Changes * * * *