SA WG2 Meeting #149e	S2-2201080r03

# February 14th – 25th, 2022; Elbonia               	    		 	 				(revision of S2-220)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0048 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | On NAT between PSA UPF and EASDF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC |  |  |  |  |  | Date: |  |  | 2022-02-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The C4-220332 / S2-2200202 LS on NAT between the PSA UPF and the EASDF states the following DNS contexts created in the EASDF contain the UE IP address. The EASDF identifies the DNS context corresponding to an incoming DNS Query message by matching the source IP address of the DNS Query message with the UE IP address provisioned in DNS contexts (see clause 5.2.3 of TS 29.556). CT4 kindly asks SA2 to clarify whether NAT may be deployed between the PSA UPF and the EASDF, and if so, how the SMF obtains the NATed IP address that it then should provision as the "UE IP address" in DNS contexts. In this release of the specification, neither SA2 not CT4 have defined N4 extensions for the SMF to get information on NATing enforced by the UPF and on the UPF NAT mapping between addresses and port used to carry user ttaffic. Furthermore it is also possible that the UPF does not enforce NAT for traffic exchanged with the EASDF while it enforces NAT for traffic sent to other N6 destinations (e.g. to be sent to actual applications on the Internet) |  |  |  |  |  |  |  |  |
| Summary of change: |  | This release of the specifications does not support the deployment of a NAT between the UPF and the EASDF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is unclear whether this release of the specifications defines mechanisms to support NAT between the UPF and the EASDF as highlighted by the CT4 LS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

### 5.1.1	Functional Description

The Edge Application Server Discovery Function (EASDF) includes one or more of the following functionalities:

-	Registering to NRF for EASDF discovery and selection.

-	Handling the DNS messages according to the instruction from the SMF, including:

-	Receiving DNS message handling rules and/or BaselineDNSPattern from the SMF.

-	Exchanging DNS messages from the UE.

-	Forwarding DNS messages to C-DNS or L-DNS for DNS Query.

-	Adding EDNS Client Subnet (ECS) option into DNS Query for an FQDN.

-	Reporting to the SMF the information related to the received DNS messages.

-	Buffering/Discarding DNS response messages from the UE or DNS Server.

-	Terminates the DNS security, if used.

The EASDF has direct user plane connectivity (i.e., without any NAT) with the PSA UPF over N6 for the transmission of DNS signalling exchanged with the UE. The deployment of a NAT between EASDF and PSA UPF is not supported.

Multiple EASDF instances may be deployed within a PLMN.

The interactions between 5GC NF(s) and the EASDF take place within a PLMN.

END OF CHANGES