SA WG2 Meeting #149e	S2-2201089r04

# February 14th – 25th, 2022; Elbonia               		(revision of S2-220)

Source: 	Intel

Title: 	FS_SFC architectural assumptions and Principles

Document for: 	Approval

Agenda Item: 	9.1

Work Item / Release:	FS_SFC / Rel-18

Abstract of the contribution: This PCR provides architectural assumptions and principles for FS_SFC study in Release 18.

1	Discussion

void

2 Proposal

It is proposed to update TR 23.700-18 on FS_SFC as follows:

*** BEGIN CHANGES ***

4	Architectural Assumptions and Principles

The concept of the SFC is as shown in Figure 4-1 and the following assumptions are applied in this study:

- Each service function (SF) is an opaque processing element in user plane. The SF forwarder, and SF behavior/control are out of scope of this study that reuses the features defined by IETF for these purposes.

- The initial classification for the service chain to be applied to traffic flow is performed by 5GC.

- The management aspects  of SFs, service function paths (SPF), and service function chains are performed by 3GPP OAM, which is out of scope of the present TR.

- Several service function chains can be simultaneously applied within 5GC of an operator network to meet various business requirements with third parties based on service level agreements. A traffic flow of a PDU Session is subject to a single SFC at a given time.

- The definition of terms in RFC 7665 [X] may be re-used when applicable. The traffic steering concept, e.g. defined by 3GPP (FMSS) and SFC mechanisms, especially for user plane mechanisms (e.g. VXLAN, NSH, GENEVE, GRE, VLAN, etc.), defined in IETF may be re-used when applicable.

Figure 4-1: Illustration of the service function chaining concepts

For all objectives, the architectural assumptions and principles include:

- Solutions shall build on the 5G System architectural principles as in TS 23.501 [3], including flexibility and modularity for newly introduced functionalities.

- Service path (i.e. for traffic handled by the service functions) is traversed over N6 after PSA UPF(s) in 5G network.

- The Home Routed roaming PDU Sessions do not have an offloading point in a VPLMN.

- Existing (per 3GPP R17) deployments flexibility should be preserved in deployments of SFC in 5G network.

*** END CHANGES ***