SA WG2 Meeting #149e	S2-2201089r09

# February 14th – 25th, 2022; Elbonia               		(revision of S2-220)

Source: 	Intel, Sandvine

Title: 	FS_SFC architectural assumptions and Principles

Document for: 	Approval

Agenda Item: 	9.1

Work Item / Release:	FS_SFC / Rel-18

Abstract of the contribution: This PCR provides architectural assumptions and principles for FS_SFC study in Release 18.

1	Discussion

void

2 Proposal

It is proposed to update TR 23.700-18 on FS_SFC as follows:

*** BEGIN CHANGES ***

4	Architectural Assumptions and Principles

The concept of the SFC is as shown in Figure 4-1 and the following assumptions are applied in this study:

- Each service function (SF) is an opaque processing element in user plane. The SF forwarder, and SF behavior/control are out of scope of this study that reuses the features defined by IETF for these purposes.

- The initial classification for the service chain to be applied to traffic flow is performed by 5GC.

- The management aspects of SFs, service function paths (SFP), and service function chains are out of scope of the present TR.

- Several service function chains can be simultaneously applied within 5GC of an operator network to meet various business requirements with third parties based on service level agreements. A traffic flow of a PDU Session is subject to a single SFP at a given time.

- The definition of terms and mechanisms in RFC 7665 [X] may be re-used when applicable.

- The traffic steering concept, e.g. defined by 3GPP (FMSS) and SFC mechanisms, especially for user plane mechanisms (e.g. VXLAN, NSH, GENEVE, GRE, VLAN, etc.), defined in IETF should be re-used when applicable.

Figure 4-1: Illustration of the service function chaining concepts

For all objectives, the architectural assumptions and principles include:

- Solutions shall build on the 5G System architectural principles as in TS 23.501 [3], including flexibility and modularity for newly introduced functionalities.

- Service path (i.e. for traffic handled by the service functions) is traversed over N6 after PSA UPF(s) in 5G network.

- This study focuses on PDU Sessions with a single PSA UPF, located in the HPLMN.

- Existing (per 3GPP R17) deployments flexibility should be preserved in deployments of SFC in 5G network.

*** END CHANGES ***