SA WG2 Meeting #149e	S2-2201091r03

# February 14th – 25th, 2022; Elbonia               		(revision of S2-220)

Source: 	Intel, Nokia, Nokia Shanghai Bell

Title: 	FS_SFC KI related with WT2

Document for: 	Approval

Agenda Item: 	9.1

Work Item / Release:	FS_SFC / Rel-18

Abstract of the contribution: Address FS_SFC KI related with WT2.

# 1	Discussion

This key issue is proposed based on WT#2:

WT#2: Investigate whether traffic steering policy is sufficient to support SA1 requirements, study whether and how to define SFC policies, and investigate solutions and procedures for the 5G network with SFC capabilities to identify/detect/classify user plane traffic and steer the traffic to a chain of ordered service functions for SFC processing in non-roaming and home routed roaming scenarios.

# 2 Proposal

It is proposed to update TR 23.700-18 on FS_SFC as follows:

*** BEGIN CHANGES ***

# 5	Key Issues

## 5.X	Key Issue #X:  Traffic Steering Policy and SFC Policy

### 5.X.1	Description

Currently, the SMF may be configured with the traffic steering policy related to the mechanism enabling traffic steering to the N6-LAN, DN and/or DNAIs associated with N6 traffic routing requirements provided by the AF. Also, UPF with SFC capabilities need to be enabled to support flexible SFC configuration for a PDU session that requires different SFC processing for different applications. Based on architectural assumptions and principles in clause 4 and service requirement in TS 22.261 clause 6.35, this key issue is for WT2 to study:

- whether traffic steering policy need to be enhanced for SFC and whether SFC policy need to be defined to work with traffic steering policy.

- based on the SFC policy definition, what are the required architecture enhancement for the network functions, interfaces, and the procedures to enforce SFC policies and/or traffic steering policies to identify/detect/classify user plane traffic and steer the traffic flows of the UEs requiring SFC processing for their applications in non-roaming scenarios.

- based on the SFC policy definition, whether and how the HPLMN can apply traffic steering policies and/or SFC polices for home routed traffic for cases that result in the change of routing paths for user plane traffics requiring SFC processing in non-roaming and home routed roaming scenarios.

- what are the relationship between traffic steering and SFC processing in the user plane.

- what (if any) are the new features that the UPF may support in order to allow improved R18 SFC capabilities.

- how to enhance interfaces to support UPF with SFC capability including how such capabilities may be discovered and controlled by the SMF, and what the procedures are to provision available instance and information of UPF with SFC capabilities to the SMF.

*** END CHANGES ***