SA WG2 Meeting #149e	S2-2201118r01

# February 14th – 25th, 2022; Elbonia               		            (revision of S2-220xxxx)

Source: 	NTT DOCOMO, Interdigital, Samsung, OPPO, Lenovo (?), Motorola Mobility (?) China Telecom (?), China Mobile (?)

Title: 	KI (WT#1.2): QoS and Policy enhancements

Document for: 	Approval

Agenda Item: 	9.21

Work Item / Release:	5G_AIMLsys/ Rel-18

Abstract of the contribution: This contribution proposes a KI on possible QoS, Policy enhancements to support Application AI/ML operational traffic.

# 1	Discussion

Based on the approved WT#1.2, it is proposing a KI on to study on possible QoS, Policy enhancements to support Application AI/ML operational traffic.

# 2 Proposal

It is proposed to update TR 23.XXX on 5G_AIMLsys as follows

# 5	Key Issues

## 5.X	Key Issue #X: QoS and Policy enhancements

### 5.X.1	Description

This key issue is applicable to Objective 2 (i.e. WT#1.2) for the QoS and Policy enhancements to support Application AI/ML operation traffic and to address SA1 requirement as specified in TS 22.261[x] for aggregated QoS support.

As defined by requirements in TS 22.261 [x], the AI/ML Services & Transmissions with 5GS assistance to support AI/ML model distribution, transfer, training for various applications requires a dedicated treatment in the network. It is a challenge how the 5GS can be leveraged by AI/ML service providers as a platform to provide intelligent transmission support for application layer AI/ML operation. In addition, in mobile communications systems, mobile devices (e.g. smartphones, automotive, robots) are increasingly replacing conventional algorithms (e.g. speech recognition, image recognition, video processing) with AI/ML models to enable applications.

In order to enable the 5G system to assure the QoS performance requirements as specified in clause 7.10 of TS 22.261 [x], it is important for the 5G system to be able to differentiate the specific Application AI/ML traffic while supporting regular 5GS user traffic. Hence, this study needs to determine any additional requirements to the 5G system in terms of the policy enhancement and the traffic handling (e.g. content aware AI/ML traffic classification). For example, in order to reach ultra high accuracy and ultra low latency for AI/ML, a UE may request for obviously too many transmission resources, which may bring influences to other UEs.

Generally, the awareness of predicted UL/DL data rate is beneficial for AI/ML application layer to make the decision on AI/ML operation. However, if the Application AI/ML operation traffic share the same PDU session with non-Application AI/ML traffic, it is difficult for 5G network to predict the data rate. Meanwhile, the determined AI/ML operation has the corresponding QoS requirement for UL/DL data rate respectively.

Therefore, this key issue will study solutions on below aspect(s) for the possible QoS and Policy enhancement to support Application AI/ML operation:

- What and how to specify the individual (e.g. for AI inference, model downloading, etc.) and aggregated QoS (e.g. for Federated learning group) parameters based on the requirements provided by the authorized 3rd party to 5G system to assure the QoS performance for one or a group of UEs which participate in the given Application AI/ML operation as specified in TS 22.261 [x]?

  - Whether the current QoS model for 5GS needs to be enhanced to account for the following types of application AI/ML traffic: i) data collected for training and inference and shared by the application server or client over the 5GS, and ii) AI/ML models trained by the application server or client shared over the 5GS.

- Whether any special handling is needed in congested situation for transporting Application AI/ML traffic? What are the enhancements, if any, to the 5G system to recognize the specific AI/ML traffic in order to apply the appropriate QoS policy for the traffic handling and control as well as keeping track of the traffic volume to comply with the contracted QoS agreement between the authorized 3rd party and the 5G operator? What is the priority mechanism between these traffics e.g., which traffic to be prioritized? . Whether an exclusive PDU session for Application AI-ML traffic is necessary and what is the policy enhancement if the answer is yes?

- Study how to assist the AI/ML service to react to the changing network condition timely, e.g., radio channel status.

# References

[X]	3GPP TS 22.261: "Service requirements for the 5G system".