SA WG2 Meeting #149E	S2-2201136r12

# 14 – 25 February 2022, Elbonia

Source:	Nokia, Nokia Shanghai Bell, Oppo?, Vivo?, China Telecom?, China Mobile, Huawei, HiSilicon?, InterDigital, Ericsson, ETRI, Sandvine?

Title:	KI: Key Issue for WT#1.2

Document for:	Discussion/Approval

Agenda Item:	9.23

Work Item / Release:	FS_eNA_Ph3 / Rel-18

Abstract of the contribution: This contribution proposes a KI for WT#1.2

# 1		Discussion

The agreed SID SP-211650 contains the following Working Task:

Objective#1: The following key issues deprioritized from R17 WT#1.2: Study possible mechanisms for improved correctness of NWDAF analytics

A few aspects need to be considered to improve correctness of NWDAF analytics:

-	Correctness of predictions is usually associated to accuracy, a metric which compares the number of correct predictions out of all the predictions, and that represents maybe the most prominent KPI to rate ML models. However, the accuracy can be corrupted by a wrong calculation and/or unfair feedback collection. Thus, it should be studied how to compute the accuracy metric correctly and in a reliable way, and to collect performance feedback from trusted sources.

-	The accuracy of an ML model during inference may be lower than the accuracy of the same ML model during training. This is likely to happen if the training data set differs significantly in terms of distribution and features from the input data that the ML model is fed with during inference. Knowing how the ML model is meant to be used would help to train the ML model efficiently and effectively. It should be studied how to enhance ML model provisioning to take into account how the model is used.

This contribution proposes a related key issue.

# 2		Proposal

The following change is proposed for TR 23.700-81.

# * * * * First change (all new text)* * * *

## 5.x	Key Issue #X: How to improve correctness of NWDAF analytics

Correctness of predictions is usually associated to accuracy, which represents the most prominent KPI to rate ML models. However, the accuracy can be corrupted by a wrong calculation and/or unfair feedback collection. It is thus of utmost importance to ensure that the accuracy is meaningful, consistent, and comparable.

Incorrect predictions can be due to the fact that the accuracy of an ML model during inference may be lower than the accuracy of the same ML model during training. This is likely to happen if the training data set differs significantly in terms of distribution and features from the input data that the ML model is fed with during inference. Knowing how the ML model is meant to be used would help to train the ML model efficiently and effectively.

The following aspects need to be studied:

-	How to detect that improving the correctness of an Analytics ID is needed?

-	How and what information is required to compute and represent the analytics correctness in a reliable way?

NOTE: 	Whether the information is used to represent analytics correctness may depend on the use case.

-	Whether and what action(s) should be taken when the NWDAF Analytics are considered incorrect,  whether and how a NF can continue consuming an analytics ID for which the need for improvement has been detected?

-	Whether and how the NWDAF can improve correctness of NWDAF analytics (e.g. what additional information may be used by the NWDAF to improve the correctness of existing Rel-17 NWDAF analytics)?

-	Whether there is a need for architectural and functional enhancements for improving correctness of NWDAF analytics, if

there are additional signalling load caused by any new functional enhancement and how to mitigate the signalling?

-	How and what information is needed to enhance the ML model provisioning to improve the correctness of NWDAF analytics.

-	Study mechanisms to detect that degradation on an ML model has happened and whether and what actions should be triggered.

# * * * * End of changes * * * *