SA WG2 Meeting #149e	S2-2201173r0245

# February 14th – 25th, 2022; Elbonia

Source: 	Nokia, Nokia Shanghai Bell

Title: 	FS_UPEAS new KI: study direct subscription to the UPF for UPF event exposure services

Document for: 	Approval

Agenda Item: 	9.25

Work Item / Release:	FS_UPEAS / Rel-18

Abstract of the contribution: FS_UPEAS new KI: study direct subscription to the UPF for UPF event exposure services

# 1	Discussion

FS-UPEAS objectives contain:

WT#1: Study UPF event exposure service(s) registration/deregistration, and discovery via the NRF.

WT#2: Study UPF event exposure service(s) that would support, e.g.

-	Consumption of UPF exposure services by the PCF, NWDAF, CHF, NEF, Trusted AF and other NFs (if needed).

-	(To support the UPF exposure service, if needed) Use of SMF services, PCF services, NWDAF services, CHF services, NEF services, Trusted AF services by the UPF.

NOTE 1: This will not define solutions where UPF exposes information that it is not originator of, i.e. not re-expose information owned and exposed by other NFs.

-	Relevant Event IDs.

WT#3: Evaluate usage of UPF event exposure service(s) as defined in WT#2 also considering the architectural impacts

# 2 Proposal

It is proposed to update 23.700-62 as follows

CHANGE (1) ALL text is new

# 5	Key Issues

## 5.X	Key Issue #X:  Study UPF event exposure service registration and discovery .

### 5.X.1	Description

This KI maps to the WT1 of the SID.

In R17 consumers (NEF, AF) of UPF event exposure subscribe to UPF event exposure via the SMF (as e.g. anyhow SMF actions towards the RAN are needed in order for R17 QoS monitoring to take place also in RAN). If the UPF events that the consumers of UPF event exposure services desire to subscribe to don’t require involvement of a SMF, a possible architecture may be to support a direct subscription from the consumers of UPF event exposure services to the UPF itself.

The KI will study the registration and discovery of UPF event exposure services. The following aspect should be studied:

- How to support UPF Event exposure service(s) registration/deregistration on NRF.and what parameters to be registered in the NF profile of UPF.

- How to support UPF service discovery via the NRF, and what parameters that can be used for discovery.

- How to in an efficient way discover a suitable UPF based on certain parameters e.g. a certain UE.

This key issue is not meant to define for which UPF event / exposed information a direct consumer subscription to the UPF would take place (if any) but targets solutions defining how this direct subscription would take place.