3GPP TSG-WG SA2 Meeting #149E e-meeting 	S2-2201185r08

Elbonia, February 14th – 25th, 2022	(merge of S2-2201185, S2-2100577, S2-2201124, S2-2201192)

Source:	Xiaomi, vivo, Interdigital Inc, Intel

Title:	Key Issue: Authorization and policy/parameters provisioning for PIN

Document for:	Approval

Agenda Item:	9.16

Work Item / Release:	FS_PIN / Rel-18

Abstract: this paper proposes a key issue on Authorization policy and provisioning for PIN.

# 1. Introduction

According to the SP-211643, the provisioning and configuration aspects are not explicitly listed in the objectives for the communication of PIN, PIN/PINE/PEGC/PEMC discovery, authorization/authentication for the PIN/PINE, but they are important and indispensable for the related procedures. It serves part of following work tasks:

- (WT#1.1) To study the potential architectural enhancements for supporting management of PIN, access of PIN via PIN Element with Gateway Capability (PEGC), and communication of PIN (e.g. PIN Element communicates with other PIN Elements directly or via 5GS or via PEGC and 5GS).

- (WT#2.1) To study how to identify PIN and the PIN Elements in the PIN at 5GC level to serve for authentication/authorization.

- (WT#3.1) To study the management of a PIN, e.g., create PIN, authorizing/de-authorizing PIN Elements, authorizing/de-authorizing PIN Elements with Management Capability (PEMC), authorizing/de-authorizing PIN Elements with Gateway Capability (PEGC), establishing duration of the PIN, etc.

- (WT#3.2) To study the procedures for network discovery, PIN Element discovery, capability of PIN Element discovery, as well as availability and reachability discovery, e.g., assisted by a PIN Element with Management Capability (PEMC).

There can be interaction over application layer among PIN Elements, PEGC, PEMC, and application server, that will be studied in SA6, most of the PIN configuration can be done over application layer, but considering that PEGC needs to have additional functionality for supporting, e.g., differentiating PIN Element for 5GC, access control, etc., the 5GC needs to support PEGC configuration for discovery, access control, etc.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-88.

# * * * * First change * * * *

# 5	Key Issues

## 5.X 	Key Issue #X: Supports policy and parameters provisioning for PIN

### 5.X.1	Description

In order to support the necessary procedures regarding to PIN, e.g., communication between PINEs, PINE/PEGC/PEMC discovery, authorization for PINE/PEGC/PEMC, etc., necessary policy/parameters configuration are needed.

The following aspects will be studied:

–

-	Whether and How the PIN related policy and parameter(s) identified in the other KI for PIN discovery, PINE discovery, authentication/authorization for PINE and PIN communication are configured to the PEMC, PEGC and PINE.

-	Whether and how 5GC supports provisioning of configuration information to PEGC for access control.

# * * * * End of changes * * * *